3GPP TSG-RAN WG3 Meeting #127                                                                                 R3-250361

Athens, GR, 17th – 21st, Feb, 2025

Source:	CATT

Title:	(TP for BL CR to TS 38.473) On support on-demand SSB SCell operation

Agenda Item:	17.2

Document for:	Discussion and Approval

1 Introduction

For NES on-demand SSB SCell operation topic, the WID[1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In last RAN1 meeting[3], some agreements are achieved on various aspects related to on-demand SSB SCell, the agreements are as follows:

Agreement at RAN1-118 meeting For a cell supporting on-demand SSB SCell operation, at least the following is supported. - On-demand SSB on CD-SSB or NCD-SSB the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB. FFS: Additional support of OD-SSB for CD-SSB located on sync-raster. Agreement Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst. Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1: - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

In this contribution, we will continue to discuss the on-demand SSB SCell operation topic, our views will be given as well.

2 Discussion

2.1 Interaction over F1 interface

In last RAN3 meeting [2], the agreements are as follows:

For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

RAN3 has already reached a consensus that NES gNB-DU initiates on-demand SSB. And in split architecture, the on-demand SSB configuration for UE’s SCell is also be carried out by gNB-DU.

Per previous RAN1 meetings, the RRC and MAC CE based signaling to indicate on-demand SSB transmission is supported. And the last RAN3 meeting reached the above conclusion on RRC based signaling. However, for MAC CE based signaling, whether NES gNB-CU needs to know the activation/deactivation of on-demand SSB remains undiscussed. MAC CE is used for faster configuration of UE operation and the on-demand SSB operation could be performed quickly. In our opinion, the activation/deactivation state of on-demand SSB transmission could be quite dynamic. DU can decide the SSB configuration per SCell and notify the CU. But it is not necessary for DU to inform the CU the activation/deactivation state of on-demand SSB. For MAC CE based signaling, NES gNB-CU does not need to know the activation/deactivation state of on-demand SSB.

Observation 1: NES gNB-CU does not need to know the activation/deactivation of on-demand SSB due to MAC CE signaling.

There are two cases for a cell supporting on-demand SSB SCell operation:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

For Case#1, if no always-on SSB on the NES cell, the cell is not suitable to be used as PCell/PSCell nor the cell is measured as neighbour cell (as on-demand SSB is only used for SCell operation).

For Case#2, if always-on SSB is periodically transmitted on the NES Cell, the cell can be measured as a neighbour cell. RAN1 has already supported the case where always-on SSB is NCD-SSB, but whether to support the case where always-on SSB is CD-SSB is FFS. If always-on SSB is limited to NCD-SSB, the cell is not suitable to act as PCell/PSCell as the cell coverage is not available and UE cannot camp on the cell. Otherwise, if always-on SSB can be a CD-SSB, the cell can still act as PCell/PScell.

Since the gNB-CU takes charge of the cell coverage management as well as the measurement configuration, the gNB-CU should be able to decide the cell(s) allowed to operate on-demand SSB and whether the cell(s) should transmit the always-on SSB. Furthermore, if the case where always-on SSB is CD-SSB is supported, the gNB-CU also decides whether always-on SSB should act as CD-SSB when the always-on SSB should be transmitted in the cell(s). The gNB-CU can send that information to the gNB-DU. For example, gNB-CU can send a list of allowed cells to operate on-demand SSB to gNB-DU for energy saving. For each cell in the list, gNB-CU needs to notify whether the cell should transmit always-on SSB or not. And if always-on SSB acting as CD-SSB is supported by RAN1, gNB-CU also needs to notify whether the always-on SSB should include CD-SSB. The gNB-DU receiving the list of allowed cells can apply on-demand SSB to NES cell(s) based on the information informed by the list.

Proposal 1: The gNB-CU can send a list of allowed cells for on-demand SSB to gNB-DU. For each cell in the list,

- Indicates whether the cell should transmit always-on SSB;

- If the case where always-on SSB is CD-SSB is supported by RAN1, indicates whether the always-on SSB should include CD-SSB when always-on SSB need be transmitted.

Similar as the list of cells allowed to be deactivated, the list of cells allowed for on-demand SSB can be introduced to the GNB-CU CONFIGURATION UPDATE message.

Proposal 2: A list of allowed cells for on-demand SSB can be introduced to the GNB-CU CONFIGURATION UPDATE message.

RAN1 has the conclusion that on-demand SSB can be used for L1 and/or L3 measurement. So, gNB could configure on-demand SSB based SCell L3 measurement to the UE.

Observation 2: NES gNB could configure on-demand SSB based SCell L3 measurement.

In previous RAN2 meeting [5], RAN2 has discussed how to support L3 measurement based on on-demand SSB, and the candidate solutions are as follows:

3. L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted Need more time to think of.

For option 1, UE measures on-demand SSB based on specific measurement configuration, e.g., the measurement objective for on-demand SSB or the SMTC for on-demand SSB. If RAN2 agree to choose option 1, it would be necessary for gNB-DU to indicate gNB-CU the on-demand SSB frequency or SMTC configuration. Then gNB-CU can configure on-demand SSB as the objective of SCell measurement. Note that the inter-node RRC message MeasurementTimingConfiguration defined in TS 38.331[4] is used for gNB-DU to convey the configurations of SSB/CSI-RS based measurement to gNB-CU over F1 interface. The legacy MeasurementTimingConfiguration message can be used for gNB-DU indicating the frequency or SMTC for on-demand SSB.

For option 2, UE measures on-demand SSB based on the RRC configuration including the frequency and pattern of the on-demand SSB. There is no need to introduce on-demand SSB specific measurement objective or SMTC. If RAN2 agree on option 2, there is no impact to F1AP message or inter-node RRC message.

It can be observed that inter-node RRC message MeasurementTimingConfiguration message would need enhancement to support on-demand SSB based SCell L3 measurement.

Observation 3: Inter-node RRC message would need enhancement to support the on-demand SSB based SCell L3 measurement.

2.2 Interaction over Xn interface

After a cell enters on-demand SSB mode, if there is no always-on SSB or the always-on SSB is NCD-SSB transmitted in the cell, the cell coverage is not valid and it cannot act as PCell/PSCell of UE. In that case, the cell should be considered as deactivated by the neighbour gNB. So, the gNB can indicate the neighbour gNB deactivation of the cell due to network energy saving via the NG-RAN Node Configuration Update procedure. Afterwards, if the neighbour gNB needs the cell in on-demand SSB mode to act as PCell/PSCell, i.e., the cell to be “activated”, the neighbour gNB can initiate the Cell Activation procedure to trigger the gNB getting the cell back to normal status.

Observation 4: After a cell enters on-demand SSB mode, the gNB can indicate the neighbour gNB deactivation of the cell for energy saving via the NG-RAN Node Configuration Update procedure. And the neighbour gNB can initiate the Cell Activation procedure to the gNB to trigger the cell getting out of on-demand SSB mode.

However, RAN1 have not concluded on whether to support the case where always-on SSB is CD-SSB. If the always-on SSB is CD-SSB after the cell enters on-demand SSB mode, the cell can still act as PCell/PSCell and be considered as a activated cell from the neighbour node’s point of view. In that case, the neighbour node would need to know the cell entering on-demand SSB mode that it would be necessary to enhance the Xn siganling, which can be further studied.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: NES gNB-CU does not need to know the activation/deactivation of on-demand SSB due to MAC CE signaling.

Proposal 1: The gNB-CU can send a list of allowed cells for on-demand SSB to gNB-DU. For each cell in the list,

- Indicates whether the cell should transmit always-on SSB;

- If the case where always-on SSB is CD-SSB is supported by RAN1, indicates whether the always-on SSB should include CD-SSB when always-on SSB need be transmitted.

Proposal 2: A list of allowed cells for on-demand SSB can be introduced to the GNB-CU CONFIGURATION UPDATE message.

Observation 2: NES gNB could configure on-demand SSB based SCell L3 measurement.

Observation 3: Inter-node RRC message would need enhancement to support the on-demand SSB based SCell L3 measurement.

Observation 4: After a cell enters on-demand SSB mode, the gNB can indicate the neighbour gNB deactivation of the cell for energy saving via the NG-RAN Node Configuration Update procedure. And the neighbour gNB can initiate the Cell Activation procedure to the gNB to trigger the cell getting out of on-demand SSB mode.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- draft_MeetingReport_RAN3#126_vDec05

- Draft_Minutes_report_RAN1#119_v030

- 3GPP TS 38.331 V18.3.0

- R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis

Annex: TP for BLCR to TS 38.473

Start of TP

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Cells Allowed for On-demand SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to operate on-demand SSB within the indicated cells for network energy saving purpose and take the information indicated by this IE into account.

Next Change

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| >>Mobile IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the mobileIAB-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the mobileIAB-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the mobileIAB-Support is sent in SIB1. | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed for On-demand SSB List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed for On-demand SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Transmit Always-on SSB | O |  | ENUMERATED (true, …) | If this IE is included, indicates the cell should transmit always-on SSB. | - |  |
| >>Always-on CD-SSB (FFS) | O |  | ENUMERATED (true, …) | This IE is only included when Transmit Always-on SSB is present. Indicates whether the always-on SSB should include CD-SSB. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

End of TP