3GPP TSG-RAN WG3 Meeting #127                                                                   R3-250362

Athens, Greece, Feb 15th – 21st, 2025

Source:	CATT, Fujitsu

Title:	(TP to BLCR for XnAP) On support on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Approval

1 Introduction

During the last RAN3 meetings[4], following agreements were achieved for on-demand SIB1:

RAN3#126 The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

2.1 UL WUS configuration provision

2.1.1 XnAP signalling

In the previous meetings[2][3][4], it was agreed by RAN3 that the new XnAP procedure is for UL WUS configuration provision from the NES NG-RAN node to Cell A NG-RAN node.

Following 3 options were discussed during on-line meeting:

Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB. (XnAP: Cell A gNB “confirm” means that it will broadcast received UL WUS configuration via its SIB right away ( i.e. not wait for 2 days) ; Cell A gNB “reject” means it will not perform the request.) Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.(The reason is, it may take up to 41 seconds to broadcast UL WUS even Cell A broadcast immediately, NES cell can stay as legacy until it knows that Cell A has broadcast its UL WUS.) Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. Then, the second message is sent from NES Cell gNB to Cell A gNB to indicate it is time to broadcast UL WUS configuration. And NES cell goes to OD-SIB Mode.

Option 1 is a baseline method requiring only the WUS configuration provision procedure, i.e., a request from NES gNB to Cell A gNB and a response from Cell A gNB to NES gNB.

A UL WUS Config IE should be introduced in item of the list of NES cells in the request message. For each NES cell, Cell A should evaluate whether to accept the UL WUS configuration provision. For those NES cells whose UL WUS config is accepted, Cell A shall store the UL WUS config and start to broadcast the UL WUS config.

For reporting whether the UL WUS config is accepted in Cell A, a list of cells whose UL WUS config is accepted and another list of cells whose UL WUS config is not accepted can be added to the response message.

If the Cell A gNB cannot accept UL WUS config for any NES cell it can respond with a failure message with appropriate cause.

Proposal 1: UL WUS Config IE is included in each item of the list of NES cells in the request message.

Proposal 2: For NES cell whose UL WUS config is accepted, Cell A shall store the UL WUS config and start to broadcast the UL WUS config.

Proposal 3: Include a list of cells whose UL WUS config is accepted and another list of cells whose UL WUS config is not accepted in the response message.

Proposal 4: Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS config for any NES cell.

To distinguish different UL WUS configuration provision procedures between NES gNB and Cell A gNB, a Provision ID IE can be added to the request message, response message and failure message.

Proposal 5: Provision ID is included in the request message, response message and failure message.

Option 2 and 3 requires the enhancement on aligning the time Cell A broadcasting UL WUS and the time NES cell going to OD-SIB1 operation:

- Option 2: Propose an additional signal from Cell A, who accepted the WUS config provision, at Cell A broadcasting the UL WUS configuration indicating NES cell that it is time to go to OD-SIB1 operation. Because the NES cell is not expected to go to OD-SIB1 operation until the Cell A broadcasting UL WUS config;

- Option 3: Proposes an additional signal from NES cell, who has been accepted, at the NES cell getting in OD-SIB1 operation indicating Cell A that it is the time to broadcast UL WUS. Because Cell A does not need to broadcast UL WUS until the NES cell enters NES status.

In our view, it makes some sense to introduce above enhancements. To consider both requirements of Option 2 and Option 3, the cell A broadcasting WUS config and NES cell entering OD-SIB1 should get to start at the same time. However, it’s impossible to achieve it via either of the signal above. For example, if Cell A sends the signal to NES cell at the time Cell A broadcasting UL WUS, NES cell may not be able to enter OD-SIB1 at once, it needs at least one SI modification period to start OD-SIB1 operation (as a cell in OD-SIB1 operation should broadcast UL WUS config of itself and make it effective, per RAN2’s agreement).

If the time alignment is necessary, Cell A and NES cell should negotiate during the WUS configuration provision procedure. Namely, NES cell can notify the estimated time when the WUS config will take effect (i.e., start OD-SIB1 operation), and Cell A can notify the estimated time when it will broadcast the UL WUS config, during this new Xn procedure. The indicated time information can be a time length, e.g., in unit of 10ms or ms.

For example, when SI modification period of Cell A is four times length of that of NES cell. NES cell notifies the Cell A that it expects to start OD-SIB1 at point A, and Cell A notifies NES cell that it expects to start broadcast UL WUS at point B. As point B is behind point A, NES cell can delay the time starting OD-SIB1 operation from point A to point B, after the WUS configuration provision procedure, in order to achieve the time alignment with Cell A.

Proposal 6: For UL WUS configuration provision, NES cell and Cell A can exchange the time of UL WUS config taking effect or the time broadcasting UL WUS config via the new Xn procedure.

2.1.2 F1AP signalling

In last meeting[4], it was agreed that WUS configuration is provided from DU to CU via Serving Cell Information IE:

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

The gNB-DU plans to operate OD-SIB1 in NES cell can provide the WUS config to gNB-CU, after that, the NES gNB can initiate the WUS Configuration Provision procedure. The BLCR to F1AP has introduced the UL WUS Configuration Information IE to Served Cell Information IE.

The NES gNB-CU decides whether OD-SIB1 can be operated in the NES cell based on the response from Cell A(s). So, the NES gNB-CU has to indicate its decision to the NES gNB-DU. If there is any cell allowed to operate OD-SIB1, NES gNB-CU can indicate the allowed cell(s) to the NES gNB-DU. Then the NES gNB-DU can activate the OD-SIB1 operation to those allowed cells.

An allowed list of OD-SIB1 IE can be introduced to gNB-CU Configuration Update message to include the identities of cells allowed to operate OD-SIB1.

Proposal 7: Add the allowed list of OD-SIB1 IE to include the identities of cell(s) allowed to operate OD-SIB1 in gNB-CU Configuration Update message.

2.2 UL WUS configuration update

2.2.1 XnAP signalling

If the WUS configuration is updated by NES cell, the new configuration should be delivered from NES cell to Cell A and broadcasted by Cell A. The new configuration delivery is similar with the first provision and thus can reuse the new Xn procedure (UL WUS configuration provision procedure). NES cell can provide the new WUS config to Cell A using the request message of the new Xn procedure. Cell A which has ever received WUS configuration of the NES cell can response whether it accept to broadcast the new WUS config for NES cell using the response message of the new Xn procedure. If the Cell A accepts the new WUS config, it overwrites the old WUS configuration with the new one for the affected NES cell and broadcast the new WUS config using SI modification procedure.

Proposal 8: If WUS configuration is updated, the new Xn procedure is reused for providing the new WUS configuration from NES cell to Cell A.

Proposal 9: For NES cell whose UL WUS config update is accepted, Cell A shall overwrite the previously stored UL WUS config and start to broadcast the new UL WUS config.

On the other hand, it was agreed by RAN2#127 meeting that the UE expects to receive UL WUS configuration updates from the NES Cell via legacy SI modification procedures for R19 UE camping on the NES cell. That means the NES cell should also broadcast the new UL WUS configuration for itself. However, as well known, the SI modification takes effect after one whole SI modification period. There would be mis-alignment between the time the new WUS configuration takes effect in NES cell and that in Cell A, e.g., if the SI modification periods are different in NES cell and Cell A. That would lead to failure in UE triggering OD-SIB1.

Take above figure as example, the SI modification period of NES cell is four times length as that of Cell A. After the new WUS config is provided to Cell A via the new Xn procedure, the new WUS configuration is expected to be broadcasted in Cell A at point A, whereas it is assumed taking effect after point B in NES cell. During the mis-alignment period, the UE camping in Cell A may fail in triggering OD-SIB1 due to using the new UL WUS configuration.

The time when the new WUS config takes effect in Cell A and in NES cell should be aligned. But this alignment is impossible to be achieved via specific Xn signalling, e.g., from NES cell to Cell A or vice versa. Cell A and NES cell should negotiate the time during the the new Xn procedure for new WUS config delivery. NES cell can notify the estimated time when the new WUS config will take effect (i.e., the time when the new WUS config is broadcasted in NES cell), and Cell A can notify the estimated time when it will broadcast the new WUS config, during the Xn procedure. The indicated time information can be a time length, e.g., in unit of 10ms or ms.

In the above example, if NES cell notifies Cell A that it expects to broadcast the new WUS config at point B, and Cell A notifies NES cell that it expects to start broadcast the new WUS config at point A, as point B is behind point a, Cell A can delay the time broadcasting the new WUS config from point A to point B in order to achieve the time alignment with Cell A.

Proposal 10: For UL WUS configuration update, NES cell and Cell A can exchange the expected time the new UL WUS configuration takes effect or the time broadcasting the new UL WUS configuration via the new Xn procedure.

2.2.2 F1AP signalling

When the WUS configuration is updated by gNB-DU, the NES gNB-DU should provide the new configuration to gNB-CU, so that NES gNB can provide the new configuration to the Cell A gNB. gNB-DU can send the updated WUS configuration to gNB-CU via the UL WUS Configuration Information IE.

Proposal 11: The UL WUS Configuration Information IE in Served Cell Information IE can be reused to include the updated UL WUS config from gNB-DU to gNB-CU.

As discussed above, the new Xn procedure can be reused for NES gNB to deliver the new WUS config to Cell A gNB. The NES gNB-CU can decide whether OD-SIB1 can be kept in the NES cell based on the response from Cell A(s). If OD-SIB1 cannot be kept, the NES gNB-CU has to indicate that to the NES gNB-DU and the relevant NES cell will get out of the OD-SIB1 operation.

As discussed above, the allowed list of OD-SIB1 IE is assumed to include the identities of cells allowed to operate OD-SIB1 in gNB-CU Configuration Update message. So, if any cell cannot keep OD-SIB1 operation, gNB-CU can update the allowed list of OD-SIB1 by gNB-CU Configuration Update message to exclude that cell. There is no extra F1 impact.

Proposal 12: The allowed list of OD-SIB1 IE in gNB-CU Configuration Update message can be reused to indicate which cell is no longer allowed to operate OD-SIB1 from gNB-CU to gNB-DU.

2.3 UL WUS Configuration transmission stop

2.3.1 XnAP signalling

In the last meeting, RAN3 agree that Cell A can initiate a class 2 Xn procedure to signal NES Cell gNB that it stops the UL WUS configuration broadcast.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

Upon receiving the indication of stopping UL WUS config broadcast from Cell A, NES cell can consider the UL WUS config is no longer broadcasted by Cell A.

Besides that, the NES cell can also decide to stop broadcasting WUS config in Cell A. When the NES cell plans to get out of OD-SIB1 operation, NES cell should request Cell A to stop broadcasting the WUS configuration. The request message of the new Xn procedure can be reused. Cell A which has ever received WUS configuration of the NES cell can response whether it accept or refuse to stop broadcasting WUS config via the response message of the new Xn procedure. If the Cell A accepts the request, it removes the stored UL WUS config for the NES cell and stops broadcast the WUS config using SI modification procedure.

It is FFS whether the request of stop WUS config provision is explicit or implicit signal, the implicit request such as using the release indicator included in the UL WUS Config IE.

Proposal 13: If NES cell decides getting out of OD-SIB1, the new Xn procedure is reused for NES cell requesting Cell A not to broadcast UL WUS config. FFS whether the request indicator is explicit or implicit.

Proposal 14: Cell A shall remove the stored UL WUS config and not broadcast the UL WUS config.

2.3.2 F1AP signalling

Cell A initiated WUS config provision stop:

If the NES gNB receives the indication from Cell A gNB saying that the Cell A has stopped broadcasting the WUS configuration for NES cell, it can determine whether any NES cell previously allowed to operation SIB1 is no longer allowed. If it is, gNB-CU has to indicate that information to the NES gNB-DU and the NES gNB-DU will stop the OD-SIB1 operation of the cell. In that case, gNB-CU can send the updated allowed list of OD-SIB1 via gNB-CU Configuration Update message to exclude the affected cell. There is no F1 impact.

NES Cell initiated WUS provision stop:

When the NES gNB-DU plans to stop OD-SIB1 operation in NES cell, it should indicate the release of UL WUS config to NES gNB-CU, so that NES gNB can initiate a UL WUS Configuration Provision procedure to request CellA gNB to stop broadcasting the WUS config.

As discussed above, during the first WUS config provision or WUS config update, UL WUS config info IE included in existing Served Cell Information IE can be used to convey the WUS configuration from DU to CU. So, there are two methods to indicate the release of WUS config:

- Using explicit release indicator in WUS Config Info IE

- No explicit indicator, using the absence of WUS Config Info IE

For method B), given no explicit indicator of release, gNB-DU should always include the WUS Config Info IE in Served Cell Information IE when delivering the gNB-DU Configuration Update message to gNB-CU (the WUS config will be assumed released otherwise). If that’s the case, gNB-CU cannot know whether the WUS config is updated or not. Thus, method B) is not feasible. A) is preferred, i.e., the WUS Config Info IE should be able to indicate the WUS config release.

Proposal 15: The WUS Configuration Information IE in the Served Cell Information IE should be able to indicate the WUS config release.

3 Conclusion

In the previous sections we made the following observations and proposals:

XnAP siganlling

Proposal 1: UL WUS Config IE is included in each item of the list of NES cells in the request message.

Proposal 2: For NES cell whose UL WUS config is accepted, Cell A shall store the UL WUS config and start to broadcast the UL WUS config.

Proposal 3: Include a list of cells whose UL WUS config is accepted and another list of cells whose UL WUS config is not accepted in the response message.

Proposal 4: Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS config for any NES cell.

Proposal 5: Provision ID is included in the request message, response message and failure message.

Proposal 6: For UL WUS configuration provision, NES cell and Cell A can exchange the time of UL WUS config taking effect or the time broadcasting UL WUS config via the new Xn procedure.

Proposal 8: If WUS configuration is updated, the new Xn procedure is reused for providing the new WUS configuration from NES cell to Cell A.

Proposal 9: For NES cell whose UL WUS config update is accepted, Cell A shall overwrite the previously stored UL WUS config and start to broadcast the new UL WUS config.

Proposal 10: For UL WUS configuration update, NES cell and Cell A can exchange the expected time the new UL WUS configuration takes effect or the time broadcasting the new UL WUS configuration via the new Xn procedure.

Proposal 13: If NES cell decides getting out of OD-SIB1, the new Xn procedure is reused for NES cell requesting Cell A not to broadcast UL WUS config. FFS whether the request indicator is explicit or implicit.

Proposal 14: Cell A shall remove the stored UL WUS config and not broadcast the UL WUS config.

F1AP signalling

Proposal 7: Add the allowed list of OD-SIB1 IE to include the identities of cell(s) allowed to operate OD-SIB1 in gNB-CU Configuration Update message.

Proposal 11: The UL WUS Configuration Information IE in Served Cell Information IE can be reused to include the updated UL WUS config from gNB-DU to gNB-CU. No extra F1 impact is needed.

Proposal 12: The allowed list of OD-SIB1 IE in gNB-CU Configuration Update message can be reused to indicate which cell is no longer allowed to operate OD-SIB1 from gNB-CU to gNB-DU.

Proposal 15: The WUS Configuration Information IE in the Served Cell Information IE should be able to indicate the WUS config release.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- Draft Report of 3GPP TSG RAN3 meeting #125, MCC

- Draft Report of 3GPP TSG RAN3 meeting #126, MCC

5 Annex A (TP to BLCR of 38.423)

-------------------------------------Start of TP---------------------------------------------

### 8.x.x	UL WUS Configuration Provision

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 for NG-RAN node2 to transmit UL WUS configuration (FFS)”.

The procedure is also used to enable NG-RAN node2 not to transmit the UL WUS configuration.

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the NG-RAN node2. The NG-RAN node2 replies with the UL WUS CONFIGURATION PROVISION RESPONSE message.

Upon receipt, for the served cell indicated by Cell ID IE in the Cells to transmit UL WUS Config Item IE and each cell indicated by NR CGI IE in UL WUS Config Info Item IE, the NG-RAN node2 shall evaluate the acceptance of the UL WUS configuration by the served cell. If the UL WUS configuration is accepted, the NG-RAN node2 shall behave in the following way:

- [If UL WUS configuration included in the UL WUS Config Info Item IE does not indicate the release of UL WUS configuration,][FFS] the UL WUS configuration included in the UL WUS Config IE shall be stored and overwrite any previously stored UL WUS configuration for the cell, then transmission of the UL WUS configuration shall be started in the served cell.

- [If UL WUS configuration included in the UL WUS Config Info Item IE indicates the release of UL WUS configuration, any stored UL WUS configuration for the cell shall be removed, and the UL WUS configuration shall not be transmitted in the served cell.][FFS]

- If the Time to take effect IE is contained in the UL WUS Config Info Item IE, if supported, shall consider that the time period indicated by this IE is expected to be used for the UL WUS configuration to take effect in the cell.

The NG-RAN node2 shall report to the NG-RAN node1, in the UL WUS CONFIGURATION PROVISION RESPONSE message, the result for all cells in the following way:

-	A list of cells whose UL WUS configuration is accepted by the served cell shall be included in the UL WUS Config Accepted List IE.

-	A list of cells whose UL WUS configuration is not accepted by the served cell shall be included in the UL WUS Config Not Accepted List IE.

If the Time to take effect IE is contained in UL WUS Config Accepted Item of the UL WUS Config Accepted List IE in the UL WUS CONFIGURATION PROVISION RESPONSE message, the NG-RAN node1 shall, if supported, consider the time period indicated by this IE is expected to be used for the UL WUS configuration of the cell to take effect in the cell indicated by the Cell ID IE.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot accept UL WUS configuration for any cell it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with appropriate cause value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Stop Indication

#### 8.x.y.1	General

The UL WUS Configuration Transmission Stop Indication procedure is initiated by an NG-RAN node to notify another NG-RAN node that the UL WUS configuration previously transmitted by the NG-RAN node is no longer transmitted.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Stop, successful operation

Upon receipt, NG-RAN node1 shall consider that UL WUS configuration of the cell indicated by NR CGI IE in UL WUS Config Transmission Stop Item IE is no longer transmitted by the cell indicated by Cell ID IE in the Cells transmitting UL WUS Config Item IE.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

-------------------------------------Next Change---------------------------------------------

#### 9.1.x.a	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by a NG-RAN node1 to a peer NG-RAN node2 to request about UL WUS configuration provision.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1. | YES | reject |
| List of Cells to transmit UL WUS Config |  | 1 |  | List of cells requested to transmit UL WUS configuration. | - |  |
| > Cells to transmit UL WUS Config Item | M | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| >> Cell ID | M |  | 9.2.2.27 |  | - |  |
| >> UL WUS Config Info List |  | 1 |  | List of UL WUS configuration information for cells operating on-demand SIB1. | - |  |
| >>>UL WUS Config Info Item | M | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>>> NR CGI | M |  | 9.2.2.27 |  | - |  |
| >>>> UL WUS Config | M |  | OCTET STRING | The UL WUS configuration for the cell indicated by NR CGI IE. Includes the [FFS] IE as defined in TS 38.331 [10]. | YES | reject |
| >>>> Time to take effect | O |  | INTEGER (0.. 8192, ...) | Indicates the time period expected to be used for the UL WUS configuration being taken effect in the cell indicated by NR CGI IE in unit of 10ms. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.x.b	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by a NG-RAN node2 to a NG-RAN node1 to acknowledge the request of UL WUS configuration provision.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1. | YES | reject |
| List of cells to transmit UL WUS Config |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| > Cell ID | M |  | 9.2.2.7 |  | - |  |
| > UL WUS Config Accepted List |  | 0..1 |  | List of cells whose UL WUS configuration is accepted. | YES | reject |
| >> UL WUS Config Accepted Item | M | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>> NR CGI | M |  | 9.2.2.27 |  | - |  |
| >>> Time to take effect | O |  | INTEGER (0.. 8192, ...) | Indicates the time period expected to be used for the UL WUS configuration being taken effect in the cell indicated by Cell ID IE in unit of 10ms. | YES | ignore |
| > UL WUS Config Not Accepted List |  | 0..1 |  | List of cells whose UL WUS configuration is not accepted. | YES | reject |
| >> UL WUS Config Not Accepted Item | M | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>> NR CGI | M |  | 9.2.2.27 |  | - |  |
| >>> Cause | M |  | 9.2.3.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.x.c	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by a NG-RAN node2 to a NG-RAN node1 to notify the failure of UL WUS configuration provision.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.x.d	UL WUS CONFIGURATION TRANSMISSION STOP INDICATION

This message is sent by a NG-RAN node2 to a NG-RAN node1 to notify the stop of UL WUS configuration transmission.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| List of cells transmitting UL WUS Config |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  |  |  |
| > Cell ID | M |  | 9.2.2.7 |  | - |  |
| > UL WUS Config Transmission Stop List |  | 1 |  | List of cells whose UL WUS configuration transmission is stopped in cell indicated by Cell ID IE. | - |  |
| >> UL WUS Config Transmission Stop Item | M | 1 .. <maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>> NR CGI | M |  | 9.2.2.27 |  | YES | reject |
| >>> Cause | O |  | 9.2.3.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

-------------------------------------End of TP---------------------------------------------

6 Annex B (TP to BLCR of 38.473)

-------------------------------------Start of TP---------------------------------------------

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells allowed for on-demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to operate on-demand SIB1 in the indicated cells for network energy saving.

>>>>>>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-----------------------------------------------------Next Change-----------------------------------------------------

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Unchanged parts are skipped |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed for On-demand SIB1 |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed for On-demand SIB1 Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

-------------------------------------End of TP---------------------------------------------