3GPP TSG-RAN WG3 Meeting #127                                                                    R3-250363

Athens, Greece, 17th – 21st Feb, 2025

Source:	CATT, NEC, ZTE

Title:	Discussion on the impact of cell DTXDRX

Agenda Item:	31.2

Document for:	Discussion and Decision

1 Introduction

This contribution discusses the impact of cell DTXDRX considering the response paper in [1] to our contribution [2] in the last meeting.

2 Discussion

It is claimed by [1], the NES CHO/HO is usually configured to a UE before cell DTX/DRX is activated, we agree this is correct understanding. As [1] explained, the F1 signalling enhancement for CHO have been discussed by RAN3 and it was not pursued. Nevertheless, we think the gNB-CU should be allowed to know that the gNB-DU plans to activate cell DTX/DRX as well as the pattern, e.g., for carrying out NES CHO/HO. But we further believe that gNB-CU can at least get that information by implementation, e.g., reading cell DTX/DRX configuration in DU o CU RRC container which is generated by gNB-DU for UE and delivered to gNB-CU. So, there would be no impact to F1.

Observation 1: There would be no impact on F1, as gNB-CU can obtain the cell DTX/DRX information by reading the RRC container for cell DTX/DRX configuration which is generated and delivered by gNB-DU.

As claimed by [1], gNB may not apply cell DTX/DRX pattern to UEs and the UEs with QoS non-suitable to the cell DTX/DRX pattern can be served by the NES cell after the cell DTX/DRX is activated. Further, gNB has the full freedom to operate a highly dynamic change of cell DTX/DRX status.

R3-247760[1]: Observation 1: It is important that network signalling for cell DTX/DRX doesn’t limit the gNB-DUs possibilities to take autonomous and dynamic decisions on activation and deactivation of cell DTX and cell DRX operation. Observation 2: 3GPP specifications don’t mandate that cell DTX/DRX operation has to be activated and deactivated for all UEs in the cell.

In our view, although the gNB is not mandated to apply cell DTX/DRX to all UEs, serving the UE outside the on-duration or keeping to serve the unsuitable UEs in NES cell of cell DTX/DRX seems quite abnormal. gNB should try to serve the UEs within the on-duration time in order to sleep in non-active periods. In other words, the cell which plans to or has activated cell DTX/DRX should avoid to serve the unsuitable UEs like a normal cell. Otherwise, the gNB will be forced to serve outside the on-duration time and it will lead to a low network energy saving efficiency. In that case, it’s not worthy to keep the cell DTX/DRX operation, and the gNB should get the NES cell back to normal.

Please note here “non-suitable” refers to the QoS rather than the UE’s NES capabilities, which means the UEs not supporting cell DTX/DRX can still be served by the NES cell only if their QoS restrictions matching the cell DTX/DRX pattern. The gNB should serve those UEs during the active period of cell DTX/DRX so that it can sleep in the non-active time.

It’s recommended that the NES cell prevents to serve the UEs with non-suitable QoS. Although cell DTX/DRX activation/deactivation would up to gNB implementation, the highly dynamic change of cell DTX/DRX status can be avoided and the energy saving efficiency can be guaranteed, given the NES cell does not serve the unsuitable UEs.

For example, gNB would not activate cell DTX/DRX if any UE remains in the cell with non-suitable QoS to the cell DTX/DRX pattern. Alternatively, the cell which plans to activate cell DTX/DRX removes the unsuitable UEs (e.g., using legacy HO or NES CHO), otherwise, cell DTX/DRX is not activated. After the cell DTX/DRX is activated, the UE’s inbound mobility to the NES cell should be restricted.

Observation 2: The cell activating cell DTX/DRX should try to serve the UEs during the on-duration time and prevent serving the unsuitable UEs, in order to guarantee the energy saving efficiency.

If a UE with non-suitable QoS is handed over to the NES cell with cell DTX/DRX activated, the cell may deactivate cell DTX/DRX. Thus, the neighbour nodes should restrict the mobility of UEs with non-suitable traffic to the NES cell. Neighbour nodes can determine whether the UE is suitable for the NES cell based on the cell DTX/DRX information (e.g., status, type and pattern). Note that it’s not suitable for the target side to do the handover control due to NES status. It’s beneficial for the neighbour nodes to be aware of cell DTX/DRX information of the NES cell.

Observation 3: Indicating the cell DTX/DRX information (status, type and pattern) to neighbour nodes is beneficial for refraining the NES cell from serving non-suitable UEs.

Also, the information of cell DTX/DRX pattern can help the neighbour cells estimate the possible interference from NES cell, and thus to achieve interference mitigation, e.g., deriving their own cell DTX/DRX patterns by by adapting on-duration of their own cell DTX/DRX overlapping with non-active time of the NES cell.

[1] said the usage probability in the active duration could help to achieve a more efficient resource coordination, because the usage percent of active duration would probably not be 100%, e.g., 50%. The neighbour cells can therefore adapt the cell DTX/DRX pattern taking the usage probability into account. For example, if the usage probability of active duration of NES cell is only 50%, the neighbour cells can consider the interference from that NES less than those whose usage percent is 100%.

However, we believe this problem is not new. In legacy, the resource usage is exchanged through resource status report. As discussed above, if cell DTX/DRX is activated, gNB is expected to serve during the on-duration time and avoid serving the unsuitable UEs. Thus, the usage probability of non-active time is nearly 0. The usage probability of on-duration therefore can be derived by the resource status reporting in conjunction with cell DTX/DRX pattern. For example, if the cell DTX/DRX duty cycle occupies 30% and the PRB usage of the NES cell is 25%, the usage probability of active time will be around 75%, because the usage probability of non-active time is assumed as 0.

Based on the legacy resource status exchanging, there no need to introduce other resource usage information over Xn. Exchanging the existing cell DTX/DRX information (including status, type and pattern) is sufficient on achieving interference mitigation between NES cell and neighbours.

Observation 4: Exchanging cell DTX/DRX pattern over Xn is sufficient to achieve interference mitigation between NES cell and neighbours.

A Cell DTX/DRX information IE can be added to Served Cell Information and exchanged between NG-RAN nodes over Xn during Xn setup or NG-RAN node update. The cell DTX/DRX information IE is used to indicate the cell DTX/DRX pattern, type as well as status. The cellDTX-DRX-Config-r18 in ServingCellConfig IE defined in RRC spec[3] consists of all the parameters also the release indication, this IE can be used as cell DTX/DRX information in Xn spec, then Xn does not need to define any new IE.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

…

cellDTX-DRX-Config-r18              SetupRelease { CellDTX-DRX-Config-r18 }                                 OPTIONAL,   -- Need M

…

}

#### –	CellDTX-DRX-Config

The IE CellDTX-DRX-Config is used to configure cell DTX/DRX related parameters.

CellDTX-DRX-Config information element

-- ASN1START

-- TAG-CELLDTX-DRX-CONFIG-START

CellDTX-DRX-Config-r18 ::=             SEQUENCE {

cellDTX-DRX-onDurationTimer-r18        CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

cellDTX-DRX-CycleStartOffset-r18       CHOICE {

ms10                                   INTEGER(0..9),

ms20                                   INTEGER(0..19),

ms32                                   INTEGER(0..31),

ms40                                   INTEGER(0..39),

ms60                                   INTEGER(0..59),

ms64                                   INTEGER(0..63),

ms70                                   INTEGER(0..69),

ms80                                   INTEGER(0..79),

ms128                                  INTEGER(0..127),

ms160                                  INTEGER(0..159),

ms256                                  INTEGER(0..255),

ms320                                  INTEGER(0..319),

ms512                                  INTEGER(0..511),

ms640                                  INTEGER(0..639),

ms1024                                 INTEGER(0..1023),

ms1280                                 INTEGER(0..1279),

ms2048                                 INTEGER(0..2047),

ms2560                                 INTEGER(0..2559),

ms5120                                 INTEGER(0..5119),

ms10240                                INTEGER(0..10239)

},

cellDTX-DRX-SlotOffset-r18                 INTEGER (0..31),

cellDTX-DRX-ConfigType-r18                 ENUMERATED {dtx, drx, dtxdrx},

cellDTX-DRX-ActivationStatus-r18           ENUMERATED {activated, deactivated}           OPTIONAL   -- Need N

}

-- TAG-CELLDTX-DRX-CONFIG-STOP

-- ASN1STOP

Proposal 1: Cell DTX/DRX information IE can be added to Served Cell Information and exchanged between NG-RAN nodes over Xn during Xn setup or NG-RAN node update. The cell DTX/DRX information IE includes the cellDTX-DRX-Config-r18 contained in the ServingCellConfig IE as defined in RRC spec.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: There would be no impact on F1, as gNB-CU can obtain the cell DTX/DRX information by reading the RRC container for cell DTX/DRX configuration which is generated and delivered by gNB-DU.

Observation 2: The cell activating cell DTX/DRX should try to serve the UEs during the on-duration time and prevent serving the unsuitable UEs, in order to guarantee the energy saving efficiency.

Observation 3: Indicating the cell DTX/DRX information (status, type and pattern) to neighbour nodes is beneficial for refraining the NES cell from serving non-suitable UEs.

Observation 4: Exchanging cell DTX/DRX pattern over Xn is sufficient to achieve interference mitigation between NES cell and neighbours.

Proposal 1: Cell DTX/DRX information IE can be added to Served Cell Information and exchanged between NG-RAN nodes over Xn during Xn setup or NG-RAN node update. The cell DTX/DRX information IE includes the cellDTX-DRX-Config-r18 contained in the ServingCellConfig IE as defined in RRC spec.

4 Reference

- R3-247760, Response to R3-247276, R3-247277, R3-247278, Nokia, Ericsson, BT, RAN3#126

- R3-247276, Support cell DTXDRX for NES, CATT, ZTE, NEC, RAN3#126

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.4.0 (2024-12)