3GPP TSG-RAN WG3 Meeting #127	R3-250365

Athens, Greece, 17-21 Feb 2025

Agenda Item:	13.2

Source:	Ericsson

Title:	Further discussion on inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we continue to discuss support for inter-CU LTM based on the agreements and open points captured at previous RAN3 meetings. Scenarios to support LTM in NR-DC based on RAN2 agreements are also discussed. 
The corresponding TPs capturing the proposals can be found in R3-250366.

RAN3 agreements and open issues

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

The error handling of multiple UE associations need to be considered.

# Discussion

## Support for Early UL sync

Before we analyse what is needed for Early UL sync in case of inter-CU LTM, we observe that for intra-CU LTM, support for Early UL Sync is specified with a mix of UE associated and non-UE associated. These steps are used:

- Step 1. CU sends to candidate DU a UE CONTEXT SETUP REQUEST with a request for the CFRA related parameters that source DU will use in the PDCCH order sent to the UE

- Step 2. candidate DU sends to CU a UE CONTEXT SETUP RESPONSE with the information that can be used for Early UL sync

- Step 3. the CU sends to the source DU (UE CONTEXT MODIFICATION REQUEST) the CFRA related parameters received by the candidate DU

- Step 4. The source DU sends a PDCCH order to the UE for TA value acquisition.

- Step 5. The target DU calculates the TA value and sends it to the source DU via the CU: first a DU-CU TA INFORMATION TRANSFER is sent from candidate DU to CU, then CU sends to source DU a CU-DU TA INFORMATION TRANSFER.

The procedures to request and obtain the RACH resources to use for Early UL Sync are UE associated. Instead, the procedures to transfer the TA values are non-UE associated.

- In intra-CU LTM the procedures to request and obtain RACH resources for Early UL Sync are UE associated.

- In intra-CU LTM the procedures used to transfer the calculated TA value for a UE are non-UE associated.

Let’s now discuss the inter-CU LTM scenario. The CU1 of gNB1 wants to request RACH resources to support Early UL Sync in one of its DUs. Over Xn, gNB1 can use a UE associated procedure, which returns one or more preambles that can be used. The gNB1 (or CU1) does not know if the candidate gNB is a split gNB or a non-split gNB. Namely, the Xn signaling between gNBs is (and must be kept) unaware of the logical entities constituting the gNBs themselves (gNB-DU, gNB-CU). On the other hand, we need to ensure that the same logic used in Rel-18 for intra-CU LTM solution is preserved. The logic in the Rel-18 solution is that the source DU can differentiate that a certain Preamble Index was provided by a certain “provider” of Preamble Indexes. The “provider” is a candidate DU, but what matters from source DU point of view is to distinguish the provider, not that this is a certain DU. So, while for intra-CU LTM, the LTM gNB-DU ID IE in F1AP was agreed as the mean to identifier the provider of the Preamble Index, this does not imply that a DU ID has not be used in Xn. On the contrary, since a gNB is not aware of the internal logical structure of its neighbours (it does not know if it is a split gNB or a non-split gNB), in Xn we need to use another identifier, that serves the same purpose as the gNB-DU ID in F1. For the purpose of requesting and providing Early UL Sync, we support to introduce in Xn a new identifier, e.g., an “Early UL Sync Configuration Provider ID”.

- For inter-CU LTM, use a new identifier to support Early UL sync (e.g., an “Early UL Sync Configuration Provider ID”).

Both for intra-CU LTM and inter-CU LTM, RACH-less LTM, is only possible if RACH resources are allocated by the candidate (DU or gNB). But RACH resources are limited (even in case the same Preamble Index is reused with multiple PRACH occasions) and the candidate (DU or gNB) can decide to make use of them in another way. For example, in inter-CU LTM, a gNB1 can be an LTM candidate for gNB2 and gNB3, and it can decide to release some or all the PRACH resources allocated for gNB2 and use them for some other purposes (for example, non LTM UEs). If gNB2 is not informed of this change, it may still send PDCCH orders to UEs and the Early UL Sync will fail. So, a source (DU or gNB) needs to know when the PRACH resources it received from a candidate (DU or gNB) cannot be used anymore. We think a good way is that the candidate (DU or gNB) explicitly informs the source (DU or gNB) when the PRACH preamble indexes assigned during the LTM preparation cannot be used anymore.

- For inter-CU LTM, a candidate gNB can inform the source gNB about the release of Preamble Indexes assigned for Early UL Sync.

- For intra-CU inter-DU LTM, a candidate DU can inform the source DU (via the CU) about the release of Preamble Indexes assigned for Early UL Sync.

As observed above, for intra-CU LTM the TA values are transferred from a candidate DU to source DU using non-UE associated signaling. Because of that, to ensure a RACH-less LTM, the source DU can send the LTM Cell Switch Command to a certain UE only when it has collected all the TA values of the candidate cells.

With inter-CU LTM additional delays are introduced compared to intra-CU LTM. Let’s assume that the TA information transfer in Xn is a non-UE associated signaling. The candidate gNB may control more candidate cells, controlled by more than one candidate DUs. The delay we can incur before the source DU collects all the TA values for a certain UE from all the candidate cell is the sum of various components: 1) in the candidate gNB, the delay due to the CU grouping the TA values of potentially multiple UEs from all the candidate DUs; 2) the delay over Xn; 3) the delay at the source gNB, where the CU can group the TA values of potentially multiple UEs coming from different candidate gNBs; 4) the delay over F1.

Considering the above, even if the existing F1AP messages (DU-CU/DU-CU TA Information Transfer) are non-UE associated, we think that reusing the same approach for Xn can nullify the Early UL Sync advantage. To avoid that, we prefer to use a UE-associated procedure to transfer TA values for multiple cells.

- The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

If Proposal 4 is not agreeable, we believe it should at least be possible in Xn signaling to identify that certain TA values are transferred for a given UE. This can help to avoid unnecessary delays in transferring TA values that can be outdated before being sent.

- Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Regarding whether the procedure should be a class 1 or a class 2, in this case we think the same approach used in F1 can be kept, i.e. use a class 2 procedure.

- The Xn procedure to transfer a TA information is a class 2 procedure.

## Handling of multiple UE associations

One of the point still unresolved with respect to initial LTM configuration is the following:

The error handling of multiple UE associations need to be considered.

RAN3 agreed that separate Handover Preparation procedures are used for each LTM candidate cell. Let’s assume that we have 2 candidate cells in one candidate gNB. When the source CU sends the first HANDOVER REQUEST message, it assigns a first Source NG-RAN node UE XnAP ID and receives from the candidate gNB the corresponding Target NG-RAN node UE XnAP ID. At this point, a first UE association is established, pointing to the first LTM candidate cell (first LTM cell relation). The source CU now sends the second HANDOVER REQUEST for the same UE, and the candidate gNB could assign the same or a different Target NG-RAN node UE XnAP ID. With different gNB implementations, nothing precludes that different pairs of “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID” are chosen for the same UE. If that happens, there will be a second UE association established for the same UE, pointing to the second LTM candidate cell (second LTM cell relation).

After initial preparation, there are two established UE associations, each one tightly coupled with one LTM candidate cell. Based on the current Xn baseline where the LTM Configuration Update is executed for one specific UE association (see tabular below), every time an update is needed that concerns a certain UE, which UE association is to be used? Also, if an update is needed for all the LTM candidate cells of one candidate gNB, multiple LTM Configuration Updates are needed. Since all the UE associations are pointing to the same UE, this leads to redundant signaling and complex error handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |

To avoid the complex error handling, we propose that, once the first Handover Preparation to prepare the first LTM candidate cell is done, and the UE association established with the pair “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID”, the following Handover Preparation procedures that prepares the other LTM candidate cells the first Target NG-RAN node UE XnAP ID is reused. With this information, the target gNB can understand that there is an already existing UE association for the UE and avoids assigning a new Target NG-RAN node UE XnAP ID.

Therefore, we propose to confirm the WA “For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.” and resolve the error handling concerning multiple UE associations in stage 3.

- Resolve the error handling concerning multiple UE association in the stage 3 by reusing the same Target NG-RAN node UE XnAP ID assigned when the first LTM candidate is prepared, when preparing for the same UE other LTM candidate cells of the same candidate gNB.

## Subsequent inter-CU LTM

At last meeting, RAN3 agreed that “To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.”

In addition, if the old source gNB still has at least one candidate cell, the new source gNB may want to update the LTM configuration for the old source gNB, for example in relation to Early UL sync. To achieve this, the new source, instead of sending UE CONTEXT RELEASE to the old source gNB, it can initiate an LTM Configuration Update. To capture all the above, we propose to introduce in TS 38.300 a signaling sequence for subsequent LTM, as captured in our companion contribution in R3-250366.

- Introduce a signaling sequence in TS 38.300 for subsequent LTM operations.

## FFS on data forwarding

At last meeting, the following open point was captured: “Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?”

Regarding the normal data forwarding we can make the following observations. In contrast to CHO, the LTM cell switch is initiated by the source gNB and not the UE, so the source gNB can initiate data forwarding as soon as it sends the LTM Cell Switch Command to the UE. Alternatively, the source gNB can wait until it receives the HANDOVER SUCCESS message. We believe that this should be left up to implementation. So, we propose that normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, but when exactly it is initiated, it is left to implementation.

- Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

## Coordination for semi-persistent CSI-RS transmission

At last meeting, a reply LS was sent to RAN1 in R3-247911, confirming that it is feasible to specify network signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).

Working Assumption
In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective

In this contribution we provide further analysis on how to achieve the coordination.  In the original LS sent by RAN1 to RAN3 in R3-247006 (and to which RAN3 replied in R3-247911), the following RAN1 Working Assumption was captured:

For LTM (intra-CU or inter-CU), the source DU prepares lower layer configuration, so it knows the CSI-RS resources for which it wants to receive UE measurements. Since the source DU also decides when to initiate the LTM cell switch based on the L1 measurements received from the UE, the source DU needs to be sure that semi-persistent CSI-RS transmission is ongoing in the candidate cell. Otherwise, the UE will not report any L1-RSRP measurements and the LTM cell switch cannot be triggered. We think that a simple way to realize the needed coordination is that the source DU requests the candidate DU to activate (or deactivate) the semi-persistent CSI-RS transmission. And to be sure that the source DU has the correct understanding of the semi-persistent CSI-RS transmission, the candidate cell needs to confirm (or acknowledge) the request. The same approach can be used for intra-CU LTM and inter-CU LTM.

For intra-CU LTM, the request of the source DU and the response from the candidate DU need to be sent via the CU, therefore some F1 signaling impact can be expected. For inter-CU LTM, we think the same F1 signaling can be reused, and of course an Xn signaling impact can be expected, the request from the source DU to the candidate DU (and the response from the candidate DU to source DU) needs to go via the source CU and the candidate CU.

The next two figures show a very high-level flow chart of how to achieve the coordination respectively for the intra-CU LTM case and the inter-CU LTM case. The detailed impact on F1 and Xn signaling are discussed in the next section.

- For intra-CU inter-DU LTM, the source DU can request to a candidate DU (via the CU) to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

- For intra-CU inter-DU LTM, the source DU can receive from a candidate DU (via the CU) the confirmation of its request to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

- For inter-CU LTM, the source CU can request a candidate CU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

- For inter-CU LTM, the source CU can receive from a candidate CU the confirmation of its request to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

Figure 1. Intra-CU inter-DU LTM - Coordination between source DU and candidate DU for semi-persistent CSI-RS transmission.

Figure 2. Inter-CU LTM - Coordination between source DU and candidate DU for semi-persistent CSI-RS transmission.

### F1 and Xn signaling for coordination

In this section we discuss our proposals on F1 and Xn signaling to implement the coordination between source DU and candidate DU to use semi-persistent CSI-RS transmission for LTM.

For intra-CU LTM, as part of the LTM preparation, the CU setup a UE context towards the candidate DU and modifies an existing context towards the source CU (see below).

Figure 3. Inter-gNB-DU LTM

In the existing flow chart, the first message where the source DU could send a request that the CU can forward to the candidate DU is the UE CONTEXT MODIFICATION RESPONSE in step 6. However, there is no “response” message for a UE CONTEXT MODIFICATION RESPONSE to confirm that the request has been accepted or not. So, the source DU will not know if the transmission of the semipersistent CSI-RS is activated or not, and it cannot rely on this method to initiate the LTM cell switch execution. Therefore, we prefer to use a different approach, and one option is to use the existing UE Context Modification Required procedure. The source DU could send its request to activate/deactivate the CSI-RS transmission in a UE CONTEXT MODIFICATION REQUIRED message, and the CU can confirm the activation/deactivation in the UE CONTEXT MODIFICATION CONFIRM message. If the request to activate/deactivate fails, the CU can send a UE CONTEXT MODIFICATION REFUSE to inform the source DU.

- The source DU can request its CU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the UE CONTEXT MODIFICATION REQUIRED message.

- The source DU can receive from its CU information about the activation or deactivation of transmission of semi-persistent CSI-RS in an LTM candidate cell using the UE CONTEXT MODIFICATION REQUEST message.

The request from the source DU to the CU can be forwarded by the CU to the candidate DU using existing signaling, e.g. in steps 7 and 8 of the existing inter-gNB-DU LTM signaling. So, we propose to reuse the UE Context Modification procedure to forward the request originated by the source DU to the candidate DU, and to receive back the response.

- The CU can send to a candidate DU a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell controlled by the candidate DU using the UE CONTEXT MODIFICATION REQUEST message.

- The CU can receive from a candidate DU the response to a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell controlled by the candidate DU using the UE CONTEXT MODIFICATION REQUEST message.

Regarding the Xn signaling impact needed to achieve the coordination for the inter-CU LTM case, we think that the coordination between source CU and candidate CU can be realized in two ways: 1) as part of the Handover Preparation or 2) as part of the LTM Configuration Update. Both options are workable for us.

- The source CU can send to a candidate CU a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the HANDOVER REQUEST message or the LTM CONFIGURATION UPDATE message.

- The source CU can receive from a candidate CU the response to a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the HANDOVER REQUEST ACKNOWLEDGE message or the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

## Inter-CU LTM with NR-DC

RAN3 received an LS from RAN2 in R3-247111, including a number of agreements related to NR-DC support for inter-CU LTM. Additional agreements taken at RAN2#128 are also listed below.

RAN2#127 agreements:

Agreements on inter-CU LTM (…) Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

RAN2#127-bis agreements:

Agreements on inter-CU LTM (…) 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. 11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

RAN2#128 agreements

Agreements on inter-CU LTM: 1. Rel-19 Set ID is configured for a candidate configuration. For DRB: - Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, > UE performs PDCP re-establishment to all radio bearers. - Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, > For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer. > For SN terminated bearer without change of termination point, UE performs PDCP re-establishment. > If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment. 2. For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. 3. For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard). 4. RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. 5. RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. 6. From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM 7. In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. 9. RAN2 to support intra-CU MCG LTM with SCG configuration. 10. It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. 11. RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Note that Inter-MN LTM and Inter-SN LTM cannot be configured (and of course not executed) at the same time. Considering the RAN2 agreements above, we think that RAN3 should discuss the 4 scenarios listed below, which in our view summarize the options agreed in RAN2:

-	Inter-Master Node LTM without Secondary Node release: this scenario enables “Inter-MN LTM with SCG reconfiguration” and “Inter-MN LTM with Intra-SN PSCell change”

-	Inter-Master Node LTM with Secondary Node release: this scenario enables “Inter-MN LTM with SCG release”

-	Inter-Master Node LTM with Secondary Node addition: this scenario enables “Inter-MN LTM with SCG addition”

-	SN initiated Inter-SN Change with LTM: this scenario enables “SN-initiated inter-SN LTM when intra-MN LTM is not configured”, and “SN-initiated inter-SN LTM when intra-MN LTM is configured”, where intra-MN LTM can be performed, but not at the same time as SN-initiated inter-SN LTM.

- RAN3 to discuss the following scenarios to support LTM with NR-DC:

- Inter-Master Node LTM without Secondary Node release

- Inter-Master Node LTM with Secondary Node release

- Inter-Master Node LTM with Secondary Node addition

- SN initiated Inter-SN Change with LTM

- Agree on the TPs for 38.300, 38.423 and 37.340 in R3-250366 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Inter-CU LTM

Observation 1	In intra-CU LTM the procedures to request and obtain RACH resources for Early UL Sync are UE associated.

Observation 2	In intra-CU LTM the procedures used to transfer the calculated TA value for a UE are non-UE associated.

Proposal 1	For inter-CU LTM, use a new identifier to support Early UL sync (e.g., an “Early UL Sync Configuration Provider ID”).

Proposal 2	For inter-CU LTM, a candidate gNB can inform the source gNB about the release of Preamble Indexes assigned for Early UL Sync.

Proposal 3	For intra-CU inter-DU LTM, a candidate DU can inform the source DU (via the CU) about the release of Preamble Indexes assigned for Early UL Sync.

Proposal 4	The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

Proposal 5	Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Proposal 6	The Xn procedure to transfer a TA information is a class 2 procedure.

Proposal 7	Resolve the error handling concerning multiple UE association in the stage 3 by reusing the same Target NG-RAN node UE XnAP ID assigned when the first LTM candidate is prepared, when preparing for the same UE other LTM candidate cells of the same candidate gNB.

Proposal 8	Introduce a signaling sequence in TS 38.300 for subsequent LTM operations.

Proposal 9	Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

Proposal 10	For intra-CU inter-DU LTM, the source DU can request to a candidate DU (via the CU) to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

Proposal 11	For intra-CU inter-DU LTM, the source DU can receive from a candidate DU (via the CU) the confirmation of its request to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

Proposal 12	For inter-CU LTM, the source CU can request a candidate CU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

Proposal 13	For inter-CU LTM, the source CU can receive from a candidate CU the confirmation of its request to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell.

Proposal 14	The source DU can request its CU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the UE CONTEXT MODIFICATION REQUIRED message.

Proposal 15	The source DU can receive from its CU information about the activation or deactivation of transmission of semi-persistent CSI-RS in an LTM candidate cell using the UE CONTEXT MODIFICATION REQUEST message.

Proposal 16	The CU can send to a candidate DU a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell controlled by the candidate DU using the UE CONTEXT MODIFICATION REQUEST message.

Proposal 17	The CU can receive from a candidate DU the response to a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell controlled by the candidate DU using the UE CONTEXT MODIFICATION REQUEST message.

Proposal 18	The source CU can send to a candidate CU a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the HANDOVER REQUEST message or the LTM CONFIGURATION UPDATE message.

Proposal 19	The source CU can receive from a candidate CU the response to a request originated by the source DU to activate or deactivate transmission of semi-persistent CSI-RS in an LTM candidate cell using the HANDOVER REQUEST ACKNOWLEDGE message or the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 20	RAN3 to discuss the following scenarios to support LTM with NR-DC:

-	Inter-Master Node LTM without Secondary Node release

-	Inter-Master Node LTM with Secondary Node release

-	Inter-Master Node LTM with Secondary Node addition

-	SN initiated Inter-SN Change with LTM

Proposal 21	Agree on the TPs for 38.300, 38.423 and 37.340 in R3-250366 that capture the above proposals.