3GPP TSG-RAN WG3 Meeting #127	R3-250367

Athens, Greece, 17-21 Feb 2025

Agenda Item:	13.3

Source:	Ericsson

Title:	Intra-CU Conditional LTM

Document for:	Discussion, Agreement

# Introduction

This is the first meeting in where RAN3 discuss conditional LTM. According to the WID scope, RAN3 work on conditional LTM in rel-19 should only focus on specifying support for intra-CU conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In rel-19, RAN3 work on conditional LTM is only for conditional intra-CU LTM.

RAN2 already spent quite some time on the topic, so it is good to consider the existing RAN2 agreements, listed below.

RAN2#127-bis agreements:

Agreements on C-LTM 1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

RAN2#128 agreements

Agreements on C-LTM 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we propose an initial discussion on Conditional LTM to evaluate the impact in network signaling.

# Discussion

## Scenarios

We propose that RAN3 defines the solution for the same type of scenarios described for legacy LTM, i.e. the following scenario should be discussed:

- Intra-gNB-DU Conditional LTM

- Inter-gNB-DU Conditional LTM

- Conditional LTM with gNB-CU-UP change

Within those, we think the first two scenarios, where CU-UP remains the same, should be discussed first, as the considerations derived for them can impact the third scenario. The first two scenarios are discussed in next subsections.

RAN3 to design network signaling solution for the following intra-gNB-CU mobility scenarios: 1) Intra-gNB-DU Conditional LTM, 2) Inter-gNB-DU Conditional LTM, 3) Conditional LTM with gNB-CU-UP change.

## CLTM preparation

In this section we discuss some fundamental aspects related to CLTM preparation.

A first question to answer is: who decides to configure CLTM? The CU or the DU? For non-CLTM, the responsibility is at the CU. We don’t see any reason why the solution for CLTM should be different in this aspect. So, we propose to confirm that CU is responsible of the decision to configure CLTM.

The CU decides to configure Conditional LTM.

A second question to answer is: who generates the CLTM execution conditions? On this, RAN2 reached several agreements:

- Source cell and each candidate cell provides its own execution condition for conditional LTM

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

  - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

  - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s).

CLTM execution conditions can be based on L1 or L3 measurements. We observe that there has always been a clear separation of concerns with respect to which logical entity is responsible to handle L1 or L3 measurements. Namely, the the DU configures the L1 measurements for the UEs and receives the corresponding measurement reports, and the CU configures L3 measurements for the UEs and receives the corresponding measurement reports. According to this principle, L3 execution conditions and L1 execution conditions need to be prepared by different entities. RAN2 agreed that DU generates the L1 execution condition, and it is FFS whether DU can generate L3 execution condition (when L3 measurements are used). To respect the principle above, we propose to confirm that DU generates L1 execution conditions, and CU generates L3 execution conditions.

DU generates L1 execution conditions, CU generates L3 execution conditions.

A third question to answer is: who decides whether CLTM is triggered based on L1 or L3 measurements?

If it is agreed that CU decides to configure CLTM, it is a natural choice that CU also decides whether CLTM should be triggered based on L1 or L3 execution conditions. RAN2 is evaluating whether the execution conditions for L1 and L3 can be configured for a CLTM candidate cell. In principle, we believe that the same type of execution conditions should be configured simultaneously. On the basis of that assumption, if CU decides for “L1”, then it asks the DU to prepare the L1 execution conditions, otherwise (the CU decides for “L3”) CU prepares itself the L3 execution conditions.

CU decides if CLTM execution conditions are based on L1 or L3 measurements.

If CLTM execution conditions are based on L1 measurements, CU requests DU to generate the L1 execution conditions. If CLTM execution conditions are based on L3 measurements, CU prepares the L3 execution conditions.

## Early Sync

RAN2 reached several agreements on Early Sync for CLTM:

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- Candidate cell TA is maintained by a new timer.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

Based on the above, we can assume that:

- TA acquisition for Early UL sync is supported.

- TCI state activation for candidate cells for Early DL sync is supported.

- mechanisms existing for legacy LTM are largely reused.

- a new candidate cell TA value is maintained by a new timer.

From the above, it seems that existing network signaling can be largely reused for intra-CU LTM. One aspect that we think RAN3 needs to discuss is whether there is a network signaling impact due to the new timer. In legacy intra-CU LTM, the UE relies on network implementation to guarantee the TA validity.

For Intra-CU Conditional LTM, the issue is how to maintain the validity of the received TA value. In LTM, the serving DU delivers the TA within the LTM cell switch MAC CE, and the serving DU checks whether the TA value is still valid before forwarding it to the UE. However, in CLTM, the UE receives the TA value before a CLTM cell switch procedure is triggered, so it we can assume that UE checks and maintains the validity of a received TA value. The source DU sends the TA value to the UE (network can inform the candidate cell’s TA information to UE via new MAC CE) and this is valid until the new Time Alignment Timer (TAT) expires. However, it is reasonable to assume that the TAT value for a candidate DU is decided by the candidate DU, i.e., the source DU does not know this value. This calls for a coordination between network nodes to support the transfer of TAT (Time Alignment Timer).

When a TA value is available for a CLTM candidate cell, the source DU signals this value to the UE. The UE then should start a Time Alignment Timer (TAT) associated with this configuration, with the way for signaling the TAT value still under discussion. If the TAT expires before a CLTM cell switch is executed, the UE performs a RACH-based CLTM cell switch. Otherwise, if the TAT is still running at the time of the CLTM cell switch execution, the UE proceeds with a RACH-less switch, without restarting the TAT. Additionally, if a new TA value is received from the source DU before the CLTM cell switch occurs, the UE starts or restarts the TAT accordingly. The signaling exchange between the source DU and the candidate DU about TAT for determining and updating the TA value needs to be discussed in RAN3.

Network coordination between the source DU and candidate DU via CU needs to be considered when transferring the LTM Time Alignment Timer (TAT).

Finally, for early sync the intra-CU inter-DU CLTM scenario requires to request and transfer RACH resources and TA values between CU and source DU, and between CU and candidate DU, as for intra-CU legacy LTM. The existing F1 signaling can be reused for these purposes, so we don’t see any RAN3 impact to support intra-CU CLTM compared to legacy intra-CU LTM.

To support Early Sync for conditional LTM, reuse the same F1 signaling defined to support Early Sycn for non-conditional LTM.

## Cell switch execution

The LTM cell switch execution in CLTM presents quite a few differences compared to legacy LTM solution. From network signaling point of view, we can expect:

- Source DU does not send the LTM cell switch command to the UE,

- Source DU does not send the DU-CU CELL SWITCH NOTIFICATION message to CU (in TS 38.401, step 14 of the intra-gNB-DU LTM signaling procedure and step 19 of the Inter-gNB-DU LTM signaling procedure).

- CU does not send the CU-DU CELL SWITCH NOTIFICATION message to candidate DU (in TS 38.401, step 20 in Inter-gNB-DU LTM signaling)

One important aspect to consider is that there is a period of time during which the source DU is not aware that UE has left the source cell. For example, in the current inter-gNB-DU LTM signaling procedure the source DU knows that the UE has left the source cell, so it does not need to receive any signaling to be informed about it. However, in inter-gNB-DU CLTM, when the UE leaves the source cell, the source DU does not know about it, and remains in a sort of limbo. It still has resources allocated for the UE, until, for example, it declares itself that the UE is lost (which is incorrect, with consequences in inaccurate observability), or until the CU releases the UE context.

A simple way to resolve the issue for inter-gNB-DU CLTM is that when the target DU detects the UE access, it informs the CU and CU forwards this information to source DU. We think RAN3 should discuss this issue and agree on a solution. To achieve this, it is possible to reuse already existing F1AP messages: first the target DU sends to CU a DU-CU CELL SWITCH NOTIFICATION message (to indicate the target cell ID), and then CU forwards the information to source DU in a CU-DU CELL SWITCH NOTIFICATION message.

RAN3 to discuss possible solutions on how to inform source DU that UE has left the source cell.

## Cell switch completion

According to RAN2 agreement “The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.”. When LTM cell switch is completed, the CU may release some of the prepared cells at the source DU. This, in our view, does not relate to whether LTM is conditional or not, so we think the existing network signaling can be reused.

For cell switch completion, LTM network signaling is reused for CLTM.

Considering the differences described between legacy LTM and conditional LTM for a split gNB, we propose to add a new signaling procedure for C-LTM in TS 38.401, as detailed in Appendix A.

Add in TS 38.401 a new signaling procedure for Inter-gNB-DU conditional LTM, as in Appendix A.

# 3 Conclusions

In this contribution the following observations and proposal were derived:

Observation 1	In rel-19, RAN3 work on conditional LTM is only for conditional intra-CU LTM.

Proposal 1	RAN3 to design network signaling solution for the following intra-gNB-CU mobility scenarios: 1) Intra-gNB-DU Conditional LTM, 2) Inter-gNB-DU Conditional LTM, 3) Conditional LTM with gNB-CU-UP change.

Proposal 2	The CU decides to configure Conditional LTM.

Proposal 3	DU generates L1 execution conditions, CU generates L3 execution conditions.

Proposal 4	CU decides if CLTM execution conditions are based on L1 or L3 measurements.

Proposal 5	If CLTM execution conditions are based on L1 measurements, CU requests DU to generate the L1 execution conditions. If CLTM execution conditions are based on L3 measurements, CU prepares the L3 execution conditions.

Proposal 6	Network coordination between the source DU and candidate DU via CU needs to be considered when transferring the LTM Time Alignment Timer (TAT).

Proposal 7	To support Early Sync for conditional LTM, reuse the same F1 signaling defined to support Early Sycn for non-conditional LTM.

Proposal 8	RAN3 to discuss possible solutions on how to inform source DU that UE has left the source cell.

Proposal 9	For cell switch completion, LTM network signaling is reused for CLTM.

Proposal 10	Add in TS 38.401 a new signaling procedure for Inter-gNB-DU conditional LTM, as in Appendix A.

# Appendix A – TP to 38.401 – Conditional LTM

-------------------------------------- CHANGES START HERE ----------------------------------

#### 8.2.x1.y1	Intra-gNB-DU conditional LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for conditional LTM. Figure 8.2.x1.y1-1 shows the intra-gNB-DU conditional LTM procedure for intra-NR.

Editor’s note: Details on the signaling procedure needs to be continued.

Figure 8.2.x1.y1-1: Intra-gNB-DU conditional LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the gNB-DU. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the gNB-DU. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration.

4.	If the gNB-DU accepts the request of conditional LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted candidate cell.

NOTE 1:	Steps 3 and 4 may be initiated multiple times for conditional LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU which may include the LTM configuration ID mapping list and/or the updated CSI resource configuration. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

NOTE 2:	In case of subsequent conditional LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	 An execution condition to trigger initiation of conditional LTM is fulfilled and the UE initiates LTM cell switch.

13.	The gNB-DU detects the UE access in the target cell as specified in TS 38.300 [2].

14.	The gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU including the target cell ID.

15.	The gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

16.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

18.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

19.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

#### 8.2.x2.y2	Inter-gNB-DU conditional LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for conditional LTM. Figure 8.2.x2.y2-1 shows the inter-gNB-DU conditional LTM procedure for intra-NR.

Figure 8.2.x2.y2-1: Inter-gNB-DU conditional LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell.

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16.	 An execution condition to trigger initiation of conditional LTM is fulfilled and the UE initiates LTM cell switch.

17.	The target gNB-DU detects the UE access in the target cell as specified in TS 38.300 [2].

18.	The target gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU including the target cell ID and the TCI state ID(s). The TA value(s) related information applicable for subsequent conditional LTM may also be included.

19.	The gNB-CU forwards in the CU-DU CELL SWITCH NOTIFICATION message to the target gNB-DU the target cell ID, the TCI state ID(s), and the TA value(s) related information received in step 18.

20.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

21.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

22.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

23.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

24.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

-------------------------------------- END OF CHANGES ----------------------------------