3GPP TSG-RAN WG3 Meeting #127		R3-250368

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on LP-WUS for NR

Agenda Item:	18.2

Document for:	Discussion

- Introduction

In RAN#105 meeting, the updated WID of Rel-19 low-power Wake-up Signal and Receiver (LP-WUS/WUR) for NR in [1] was approved with the below objective involved RAN3:

For IDLE/INACTIVE modes • Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

RAN3 has the following agreements in RAN3#125 meeting [2], RAN3#125bis meeting [3] and in RAN3#126 meeting [4]:

RAN3#125 [Agreement] New IEs should be introduced to support LP-WUS on NG, Xn and F1. FFS For RRC idle/inactive UEs, RAN3 to discuss the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU is FFS. For RRC idle/inactive UEs, RAN3 to discuss whether Entry/exit condition is generated at the gNB-DU or the gNB-CU. RAN3#125bis The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups. The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups. The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups. Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping. The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring. RAN3#126 WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID. It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE. NG-RAN node can acquire the total number of CN assigned subgrouping via OAM. WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

In this contribution, we discuss the corresponding RAN3 impacts on LP-WUS subgrouping, including CN assigned subgrouping and UE_ID based subgrouping.

- Discussion

  - LP-WUS subgrouping impact on F1 interface

RAN3 has the following agreement on F1 interface enhancement to ensure the DU know whether to send the LP-WUS when paging a UE.

RAN3#125bis Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping. RAN3#126 WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

Since the UE capability on PEI, i.e., pei-SubgroupingSupportBandList indicates whether the UE supports receiving PEI for a list of frequency bands. Thus, UE supporting the UE_ID based subgrouping/CN assigned subgrouping could only apply the UE_ID based subgrouping/CN assigned subgrouping in a cell within the frequency band that the UE indicates support the PEI, while UE couldn’t apply the UE_ID based subgrouping or CN assigned subgrouping in a cell not within the frequency band that the UE indicates support the PEI even though the cell indicates supporting the PEI with the SIB information. In our understanding, the same UE capability mechanism will be introduced for LP-WUS.

Thus, it is reasonable to introduce LP-WUS subgrouping support indication per-cell not a per UE indication in F1 paging message for both UE_ID based subgrouping and CN based subgrouping, since the DU not only needs to acquire the UE capability information on whether supporting LP-WUS but also needs to know whether a cell could support the LP-WUS for this UE.

Proposal 1: Confirm the WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

  - UE_ID for UE_ID based LP-WUS subgrouping

The UE Identity Index Value IE with10 bits length is used to calculate the Paging Frame and Paging occasion and corresponding to the UE_ID as specified in TS 38.304 [x] which is included in the following IE:

- Core Network Assistance Information for RRC INACTIVE IE with the direction that AMF  gNB-CU as described in TS 38.413[5]

- RAN Paging messagewith direction that NG-RAN node1  NG-RAN node2 as described in TS 38.423[6]

- Paging message with the direction that gNB-CU  gNB-DU as described in TS 38.473[7]

While in Rel-17 with the introduction of PEI, the UE_ID will also be used for UE_ID based subgrouping calculation, the legacy UE_ID with 10bits is not enough, thus the UE_ID is extended with the following principle:

- For UE_ID used in UE_ID based PEI subgrouping with DRX: since the maximum number of UE_ID based subgrouping is 8, UE_ID will be extended 8 times, i.e., the UE_ID is 5G-S-TMSI mod X, where X is 1024*8;

- For UE_ID used in UE_ID based PEI subgrouping with eDRX: the UE_ID should be extended 4 times further for eDRX, since the maximum value of eDRX (i.e.,10.24s) is 4 times of DRX (i.e., 2.56s) in this case, i.e., the UE_ID is 5G-S-TMSI mod X, where X is 1024*8*4;

Correspondingly, the UE_ID should be extended in the network interface, thus, Extended UE Identity Index Value with size 16bits is introduced in the above Core Network Assistance Information for RRC INACTIVE/ RAN Paging/Paging IE for the Paging Frame and Paging occasion calculation in eDRX case or for UE_ID based subgrouping calculation.

Observation 1: Extended UE Identity Index Value with the size of 15bits is introduced within the network interface in Rel-17 due to eDRX and UE_ID based PEI subgrouping.

Similarly, for UE_ID based LP-WUS subgrouping, the UE_ID for the calculation would also be extended. Since RAN1 has agreed the maximum subgroup number of LP-WUS is 32, then additional 5 bits are needed for the UE_ID within LP-WUS with UE_ID based subgrouping compared to the Rel-15 UE_ID for PF and PO calculation.

Observation 2: Since the maximum subgroup number of LP-WUS is 32, 15 bits are needed for the UE_ID within LP-WUS with UE_ID based subgrouping.

It seems the current Extended UE Identity Index Value with a size of 16 bits is enough for UE_ID based LP-WUS subgrouping. However, according to RAN2 discussion in RAN2#127 meeting:

In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately (if UE is configured to use both PEI and LP-WUS and if UE support both) in order to reduce the false paging rate. FFS on detailed configurations.

The PEI and LP-WUS may be used together, to achieve the purpose that LP-WUS subgroup ID is separate from the PEI subgroup ID, the UE_ID for UE_ID based LP-WUS subgrouping should be extended not only consider the maximum number of LP-WUS based subgrouping, i.e., 32 but also consider the maximum number of PEI based subgrouping, i.e., 8. Thus, additional 8 bits are needed for the UE_ID for UE_ID based LP-WUS subgrouping in case LP-WUS and PEI are used together, i.e., 18 bits may be needed for the UE_ID within LP-WUS with UE_ID based subgrouping.

Thus, the current Extended UE Identity Index Value IE with 16 bits may be not enough. In this case, the “Extended UE Identity Index Value” similar IE with 18bits should be introduced in the Core Network Assistance Information for RRC INACTIVE IE, RAN Paging IE and F1 Paging IE.

Proposal 2: Introducing new IE with 18 bits length and derived from 5G-S-TMSI for UE_ID based LP-WUS subgrouping into Core Network Assistance Information for RRC INACTIVE IE, RAN Paging IE and F1 Paging IE.  It could be updated based on further RAN2 agreement.

  - Emergency PDU session

In Rel-17 PEI, RAN2 discussed whether UE monitors PEI when it has an emergency PDU session [8]. In detail, for UE that supports CN assigned subgrouping, if there is an active emergency PDU session, the UE won’t inform the CN that it has the corresponding UE capability on CN assigned subgrouping.

When the UE has an active emergency PDU Session: - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

In this way, the CN won’t assign the subgroup ID for this UE [9]. Hence there won’t be paging latency for the CN assigned subgrouping. We think a similar method could be used for LP-WUS to avoid the paging latency during the emergency PDU session. However, it should be up to the SA2/CT1 decision.

Proposal 3: For CN assigned subgrouping, RAN3 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

For UE_ID based subgrouping, in the case of CN triggered paging for a UE with an emergency PDU session, the RAN doesn’t know if there is an emergency PDU session, thus the UE_ID based subgrouping for PEI is not able to be restricted which will introduce the paging latency. After the discussion within a few meetings in RAN2, RAN2 concluded that disabling UE_ID based PEI is not supported. It means that UE still monitors the PEI with UE_ID based subgrouping when it has an emergency PDU session. Considering the time offset between PEI and its corresponding PO is small, e.g., 50ms, RAN2 thinks the latency introduced by UE_ID based PEI, i.e., the emergency PDU session arrives within the time offset, could be accepted. Besides, SA2 confirmed that the service impact is limited even without any enhancement on UE_ID based subgrouping since it only has impacts on the emergency call back scenario. Besides, other features in addition to the PEI may also cause the MT call set up delay, thus it is not a big deal.

Observation 3: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

However, since the time offset between LP-WUS and its corresponding PO may be much larger than the PEI scenario, the paging latency may not be acceptable when UE has an emergency PDU session. We need to discuss and collect the companies’ views on whether to resolve this paging latency issue during an emergency PDU session.

If the latency issue on the use of UE_ID based subgrouping for LP-WUS during the PDU session needs to be addressed, we think the following options could be considered:

- Option 1: Introducing an indication in NG paging, RAN paging (if needed) and F1 paging to disable the LP-WUS function.

- Option 2: Leaving it to UE implementation, i.e., if a UE supporting UE_ID based subgrouping indicates its capability to gNB, when there is an active emergency PDU session, the UE should update its UE capability via indicating the gNB that it doesn’t support the UE_ID based subgrouping to gNB by UE implementation. After the emergency PDU session is released, the UE could update its UE capability again and indicate it supports the UE_ID based subgrouping to gNB by UE implementation. Similar to the legacy, when UE reports the capability, the RAN will indicate UE RADIO CAPABILITY INFO INDICATION to the CN, and CN will indicate this capability in CN paging for RRC_IDLE UEs, or in NGAP core Networking assistance information for RRC_INACTIVE UEs.

Proposal 4: RAN3 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

  - SA2 LS

In SA2#166 meeting, SA2 sent an LS to RAN2 and RAN3 [10]. Firstly, SA2 asks RAN3 to consider whether the terminology of “LP-WUS” is appropriate since SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile.

In our understanding, there is no conclusion in RAN WGs whether the WUS is transmitted with high power. On the other hand, the LP-WUS could only be used in the area which may be smaller than a cell due to the entry/exit condition introduced for whether UE could use LP-WUS. Besides, the terminology has been used for a long time in RAN WGs, including R18 study item and R19 work item, and the meaning is clear in RAN side without any misunderstanding. Thus, we suggest to keep the original terminology, i.e. “LP-WUS” without any change.

Proposal 5: RAN3 will still use the term of “LP-WUS” for discussion and how to capture it in the specification(s) will be further synced between different WGs in RAN.

Regarding the second issue in the LS [10] about how UE to report the UE capability on LP-WUS supporting in case the initial Registration is performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS). In this case, the legacy gNB does not understand the LP-WUS capability and thus cannot include this capability in the UERadioPagingInformation inter-node message to CN or other gNBs for paging, i.e. the LP-WUS capability reported by the UE is not maintained by the network, which results in inconsistent paging operations between the UE and the network and causes paging issues.

The same issue on PEI capability maintenance had been discussed in RAN2#120 meeting as follows [11].

RAN2#120 meeting R2-2211603 Discussion on the PEI capability maintenance Huawei, HiSilicon discussion Rel-17 NR_UE_pow_sav_enh-Core - Xiaomi think that the network deployment fixes this. The network is required to support homogenously, OPPO agrees, VDF agrees as well. HW think we didn’t agree homogenous requirement for this case. - Ericsson think there is some truth to the HW observation. Legacy gNB us required to support the latest ASN.1 Noted, not agreeable (no support)

To resolve the misalignment between the gNB and the UE on the UE capability on PEI supporting which will result the paging lost as above, R2-2211603 proposed the following 2 solutions:

- Solution#1: Introduce a capability filter for the PEI capability reporting where the UE only reports the PEI capability

- Solution#2: Introduce an additional bit to indicate whether the gNB has been upgraded to support the PEI feature, UE decides whether to report the PEI capability based on this indication.

However, during the online discussion, other companies thought the issue does not exist considering the network is required to support homogenously, or legacy gNB may require to support the latest ASN.1. Finally, no conclusion on the solutions was agreed for this issue.

In our understanding, it could be up to the network implementation to avoid this issue, e.g. the network support on LP-WUS is homogenously in the TA/RNA or all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS.

Proposal 6: Reply LS to SA2 that it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g., all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS. No specification impact is foreseen.

- Conclusions and Proposals

In this contribution, we discussed the RAN3 impacts to support paging subgrouping for LP-WUS, UE_ID based LP-WUS subgrouping for interface signalling, emergence PDU session and SA2 LS. Based on the discussion, we have the following observations and proposals.

Observation 1: Extended UE Identity Index Value with the size of 15bits is introduced within the network interface in Rel-17 due to eDRX and UE_ID based PEI subgrouping.

Observation 2: Since the maximum subgroup number of LP-WUS is 32, 15 bits are needed for the UE_ID within LP-WUS with UE_ID based subgrouping.

Observation 3: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

Proposal 1: Confirm the WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

Proposal 2: Introducing new IE with 18 bits length and derived from 5G-S-TMSI for UE_ID based LP-WUS subgrouping into Core Network Assistance Information for RRC INACTIVE IE, RAN Paging IE and F1 Paging IE.  It could be updated based on further RAN2 agreement.

Proposal 3: For CN assigned subgrouping, RAN3 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

Proposal 4: RAN3 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation, e.g. UE updates its capability of UE_ID based subgrouping for LP-WUS.

Proposal 5: RAN3 will still use the term of “LP-WUS” for discussion and how to capture it in the specification(s) will be further synced between different WGs in RAN.

Proposal 6: Reply LS to SA2 that it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g., all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS. No specification impact is foreseen.

- References

RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

RAN3 #125 Meeting Report

RAN3 #125-bis Meeting Report

RAN3 #126 Meeting Report

3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) ".

3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

R2-2209018 Report of [AT119-e][004] [ePowSav] Subgrouping/PEI (MediaTek)

3GPP TS 23.501: " System architecture for the 5G System (5GS); stage 2".

R3-250021/S2-2412876, 	LS Reply on LP-WUS subgrouping

R2-2300002, Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France

Annex – [draft] reply LS to SA2 on LP-WUS operation

Title:	[Draft] reply LS on to SA2 on LP-WUS operation

Response to:	R3-250021/ S2-2412876

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	TBD [to be RAN3]

To:	SA2

Contact Person:

Name:	              TBD

E-mail Address:	 TBD

1. Overall Description:

RAN3 would like to thank SA2 for the LS on LS Reply on LP-WUS subgroupingin [R3-240021/ S2-2412876].

For the terminology and name, RAN WG(s) will still use the term of “LP-WUS” for discussion. How to capture it in the specification(s) will be further synced between different WGs in RAN. Thus, it is up to SA2 whether to introduce another term in SA2 specification, but RAN3 recommends SA2 to use the same term as RAN side.

For updating UE Radio Capability for Paging Information in the mentioned scenario, it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g, the legacy gNB is upgraded to support the latest ASN.1 to guarantee that all capabilities (e.g. LP-WUS, PEI, (e)RedCap) of UE is not lost. No specification impact is forseen in RAN3.

2. Actions:

To RAN SA2:

RAN3 kindly request SA2 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG3 Meeting #128      7th Apr. – 11th Apr. 2025	Wuhan, China

TSG RAN WG3 Meeting #129	      19th May. – 23rd May. 2025	St Julian’s, Malta