3GPP TSG-RAN WG3 Meeting #127									       R3-250379

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	20.1

Source:	Huawei

Title:	([Draft] Reply LS to SA2) Discussion on LS from SA2 on LMF-based AI/ML Positioning for Case 3b

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution addresses the ACTION in the LS from SA2 in [2] on LMF-based AI/ML Positioning for Case 3b.

# Discussion

In their last meeting SA2 sent the LS in [2] concerning the LMF-based AI/ML Positioning aspects captured in TS 23.273 v.19.1.0 [3] for which some Editor’s Notes related to RAN3 work still exist and need to be resolved. To be specific, the following can be excerpted from [2]:

# 1 Overall description The WID AIML_CN defines that TS 23.273 includes that the LMF can perform UE Positioning using an AI/ML Model, this corresponds to case 3b as currently being worked on by RAN WGs in NR_AIML_Air WID. The AIML_CN WID obtained an exception to finalize normative work at SA2#167 that contains some Editor´s Note related to RAN3 work. SA2 asks RAN3 to provide feedback for the purpose to resolve the following Editor´s Notes in Clause 5.18 (“Support for UE positioning based on a ML Model at the LMF “) and Clause 6.22.3 (“Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements”) in TS 23.273 v.19.1.0: Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Please provide a reference to the RAN TS that includes the procedure for data collection by LMF from NG-RAN, so that SA2 can update TS 23.273 and remove these Editor´s Note (or an estimation of the time plan in RAN3). *** skip non-relevant parts *** # 2 Actions To RAN3 ACTION: SA2 asks RAN3 to provide feedback on the Editor’s Notes in Clause 5.18 and Clause 6.22.3 in TS 23,273 v.19.1.0 and to provide references to the corresponding procedures in RAN TSs (or an estimation on when those procedures can be available). *** skip non-relevant parts ***

In the LS, SA2 asks RAN3 to provide feedback on the following Editor’s Notes (ENs):

*** from clause 5.18 “Support for UE positioning based on a ML Model at the LMF” Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. *** from clause 6.22.3 “Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements” Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS.

Concerning the EN in clause 5.18, it relates to the data collected from UE and NG-RAN to be used as input data for the AI/ML model located in the LMF, and hence it is applicable to Case 2b (data collected from the UE) and Case 3b (data collected from the NG-RAN), respectively. As correctly captured by the EN itself, this needs to be decided by RAN1. However, and considering Case 3b only, how to convey the input data from the NG-RAN to the LMF falls in the realm of RAN3. In our view, the first EN in clause 6.22.3 which is applicable to Case 3b only (as it only refers to NG-RAN) captures the same SA2 understanding, that is, SA2 need some RAN3 feedback in order for the LMF to collect the input data from the NG-RAN for AI/ML based positioning calculation by the LMF.

In the past two meetings RAN3 discussed mainly AI/ML-based positioning Case 3a, and the RAN3 agreements which are relevant to Case 3b have been reached in RAN3#125bis [4]:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

From the agreements above it is clear that RAN3 intends to re-use the Measurement procedure in NRPPa so that the gNB can perform and report measurements which are relevant to the LMF for AI/ML-based positioning Case 3b, but the details of which measurements are pending RAN1 progress. We think that these agreements can be included in the reply LS to SA2 – a draft LS is provided in the Annex of this paper.

Proposal 1: RAN3 to agree to send the Reply LS to SA2 as drafted in the Annex which lists the RAN3 agreements for Case 3b so that SA2 can address the Editor’s Notes in clauses 5.18 and 6.22.3 of TS 23.273.

For the second EN in clause 6.22.3, instead, our understanding is that it refers to the scenario where the NG-RAN is not able to provide the LMF with the requested information due to an overload situation in the NG-RAN. This represents a significant issue especially in the context of AI/ML model training at the LMF – and clause 6.22.3 refers to model training aspects – for which a massive amount of data (related to a multitude of UEs) could be requested to be reported by the NG-RAN: in this case, the LMF could be made aware of the overload situation in NG-RAN which may end-up in terminating the whole data collection procedure.

In light of the RAN3 agreement that the NRPPa’s Measurement procedure is re-used to request and report the measurements to the LMF, we think that NRPPa already allows the gNB to indicate to the LMF that it cannot provide the requested measurements via an appropriate cause value (e.g., “Requested Item Temporarily not Available”) carried in the MEASUREMENT FAILURE message. If the LMF receives multiple failure messages (corresponding to multiple Measurement procedures) this could be interpreted by the LMF that the whole data collection procedure to train a model in the LMF should be terminated. Hence, in our view, there is no impact on the NG-RAN side for resolving the second EN as described by SA2 in clause 6.22.3.

Proposal 2: RAN3 to agree that, in NRPPa, it is already possible for the gNB to indicate to the LMF that measurement reporting cannot be performed by also indicating an appropriate cause value. There is no need for further enhancements to cover the scenario described in the Editor’s Note in clause 6.22.3 of TS 23.273.

# Conclusions

Proposal 1: RAN3 to agree to send the Reply LS to SA2 in the Annex which lists the RAN3 agreements for Case 3b so that SA2 can address the Editor’s Notes in clauses 5.18 and 6.22.3 of TS 23.273.

Proposal 2: RAN3 to agree that, in NRPPa, it is already possible for the gNB to indicate to the LMF that measurement reporting cannot be performed by also indicating an appropriate cause value. There is no need for further enhancements to cover the scenario described in the Editor’s Note in clause 6.22.3 of TS 23.273.

# References

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Qualcomm, RAN#106

- R3-250074 (S2-2501341), LS on LMF-based AI/ML Positioning for Case 3b, SA2 (Ericsson)

- 3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

# Annex – Draft Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #127	   R3-25xxxxx

Athens, Greece, 17th – 21st February, 2025

Title:	[Draft] Reply LS on LMF-based AI/ML Positioning for case 3b

Response to:	S2-2501341 / R3-250074

Release:	Rel-19

Work Item:	AI/ML_CN

Source:	Huawei [To be RAN3]

To:	SA2

Cc: 	RAN2, RAN1

# Contact Person:

# Name:	Damiano Rapone

# Email:		damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 thanks SA2 for LS on LMF-based AI/ML Positioning for case 3b.

RAN3 would like to provide feedback on the relevant Editor’s Notes in TS 23.273 v.19.1.0.

For the Editor’s Note in clause 5.18 and the first Editor’s Note in clause 6.22.3, RAN3 thinks that the following agreements have relevance for SA2 discussions:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

For the second Editor’s Note in clause 6.22.3, and in light of the RAN3 agreement that the NRPPa’s Measurement procedure is re-used to request and report the measurements to the LMF, the NRPPa protocol already allows the gNB to indicate to the LMF that it cannot provide the requested measurements via an appropriate cause value. Hence, there is no impact on the NG-RAN side for resolving the concerned Editor’s Note in clause 6.22.3 of TS 23.273.

# 2. Actions:

# To SA2:

ACTION: RAN3 kindly asks SA2 to take the above feedback into consideration.

# 3. Date of Next RAN3 meetings:

RAN3#127bis	7th Apr – 11th April 2025					China, CN

RAN3#128			19th May – 23rd May 2025					Malta, MT