3GPP TSG-RAN WG3 #127								R3-250389

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	9.2

Source:	China Telecom

Title: 	Discussion on Cell Information for UAV and ATG over Xn interface

Document for:	Discussion

Introduction

Operators are exploring the deployment of 5G networks to support low-altitude and aviation mobile communications. In Release 18, new features such as air-to-ground (ATG) communication and unmanned aerial vehicle (UAV) support were introduced to the 5G system. In 2024, China Telecom successfully conducted trials for UAV and ATG functionalities and plans to upgrade existing 5G cells to support these features in the commercial 3.5 GHz and 2.1 GHz spectrum bands. To accommodate both legacy user equipment (UE) and ATG/UAV UEs, operators may consider implementing dedicated mobility and radio resource management (RRM) policies for ATG/UAV UEs. Since upgraded and non-upgraded cells operate on the same frequency, it is crucial to identify which cells support ATG and UAV features. Currently, cell-related information can be exchanged between adjacent nodes for self-organizing network (SON) purposes. However, information regarding whether a cell supports ATG and UAV features is not available over the Xn interface. To address this issue, this paper proposes introducing cell-specific information about ATG and UAV capabilities over the Xn interface.

Discussion

- ATG Cell

ATG was introduced in Release 18 to support air-to-ground (ATG) communication. In operational networks, ground-based gNBs are configured with both legacy NR cells and ATG-capable cells simultaneously. According to TS 38.423, information about all serving cells must be exchanged with neighboring cells. Since ATG-capable cells are equipped with phased array antennas and customized active antenna units (AAUs), operators may consider dedicating resources for ATG UEs and excluding legacy UEs from accessing ATG cells. Therefore, it is necessary to determine whether ATG-related information should be included in the serving cell information. According to TS 38.331, ATG cell information is included in SIB1 and SIB22. The cellBarredATG IE in SIB1 indicates whether a cell supports ATG connectivity for UEs. The definition of cellBarredATG is as follows:

cellBarredATG Value barred means that the cell is barred for connectivity to ATG, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to ATG. If not present, the UE considers the cell is not allowed for connectivity to ATG, as defined in TS 38.304 [20]. This field is only applicable to ATG-capable UEs.

It can be concluded that an ATG-capable UE can access only cells where the cellBarredATG IE is set to notBarred. Additionally, SIB22 provides assistance information, including the ATG gNB location.

SIB22 information element

-- ASN1START

-- TAG-SIB22-START

SIB22-r18 ::=                         SEQUENCE {

atg-Config-r18                        ATG-Config-r18                                     OPTIONAL,     -- Need R

hs-ATG-cellReselectionSet-r18         ENUMERATED {true}                                  OPTIONAL,     -- Need R

atg-NeighCellConfigList-r18           ATG-NeighCellConfigList-r18                        OPTIONAL,     -- Need R

lateNonCriticalExtension              OCTET STRING                                       OPTIONAL,

...

}

atg-Config Provides parameters needed for ATG access such as ATG gNB location information, cell Specific Koffset, TA Report indication.

atg-NeighCellConfigList Provides ATG assistance information of ATG neighbour cells.

To prevent the handover of a legacy UE to an ATG cell, the source node must determine whether the neighboring node supports ATG connectivity. Additionally, according to TS 38.300, the legacy mobility procedure is supported in ATG networks, which implies that Xn-based handover is also supported. To facilitate the exchange of ATG information between adjacent ATG cells, it is necessary to introduce a new cell type indicator and ATG configuration for ATG cells.

Proposal 1: to introduce a new cell type indicator and ATG configuration for ATG cell over Xn interface.

- UAV cell

Similar to ATG, the NR frequencies utilized in our UAV trial are 2.1 GHz and 3.5 GHz. Consequently, UAV cells and legacy or non-upgraded cells adopt intra-frequency deployment to provide communication services for drones. From a network optimization perspective, operators aim for UAV UEs to remain camped on UAV cells. In the event of radio link failure (RLF) or handover failure, legacy or non-upgraded cells should, if feasible, facilitate the handover or redirection of UAV UEs to UAV cells. To support this, it is necessary to identify whether the target cell supports UAV connectivity. Additionally, to enable aerial operations, the network can configure a list of additionalPmax and additionalSpectrumEmission values for UAVs. According to TS 38.331, the nr-NS-PmaxListAerial information element (IE) can be configured per cell, along with a neighbouring cell list in SIB4. To acquire the nr-NS-PmaxListAerial IE of neighbouring cells, a new cell type indicator and corresponding nr-NS-PmaxListAerial configuration must be introduced for UAV cells.

Proposal 2: to introduce a new cell type indicator and corresponding nr-NS-PmaxListAerial for one UAV cell.

Based on the above analysis, we kindly ask RAN3 to agree the corresponding CRs [3][4].

Proposal 3: to agree the corresponding CRs [3][4] for Xn interface.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to introduce a new cell type indicator and ATG configuration for ATG cell over Xn interface.

Proposal 2: it is need to introduce a new cell type indicator and corresponding nr-NS-PmaxListAerial for one UAV cell.

Proposal 3: to agree the corresponding CRs [3][4] for Xn interface.

Reference

- TS38.423

- TS38.331

- R3-25xxxx  Correction on ATG cell information over Xn interface

- R3-25xxxx  Correction on UAV cell information over Xn interface