3GPP TSG-RAN WG3 Meeting #127	R3-250401x

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	14.3

Source:	Huawei,Ericsson

Title:	(TP for TS 38.300) Support of regenerative payload

Document for:	discussion

# 1	Introduction

In the previous meetings, the following agreements and FFS are given for the regenerative payload [1]:

RAN3#125bis The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. RAN3#126 RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used Whether to support NG suspend/Resume?

In this paper, we continue to discuss the NG impact and the inactive support in the regenerative payload, and we also provide our views on the SA2 LS.

# 2	Discussion

## 2.1 NG impact

The NG removal procedure was already agreed and captured by the BLCRs. The previous meetings mainly discussed whether to introduce the NG suspend/resume procedure also, which was first proposed by the paper [2] for the case of hard feeder link switchover. Since RAN3#125bis this was discussed and noted by chair notes that [3]:

- RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

This topic was discussed during several meetings without conclusion, we focus in this contributions on some main arguments.

The claimed advantage for the NG suspend/resume procedure may be a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG configurations). But it is doubtful whether the same NG configurations can be actually reused when the satellite gNB accesses the same AMF again. For example, when the gNB resumes NG connection from different location, the supported TA information should be different, so the old stored supported TA is useless; and, if the gNB resumes NG connection via a different GW, the current TNL address is different from the old TNL address, so the old NG-U tunnel configuration is also outdated.

Considering the feeder link switch is generally caused by the changing of satellite coverage, and as analyzed above, if the coverage area changes and GW changes, the NG configurations cannot be reused directly.

Observation 1: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

In addition, the NG suspend/resume operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

Based on the observations above, we propose:

- No need to introduce the NG suspend/resume towards the AMF.

## 2.2	INACTIVE supporting

The paging and UE context relocation for INACTIVE UEs are relying on the Xn interface. Due to the satellite mobility when there is no Xn connection between the UE’s last serving gNB and the UE’s current camped gNB, the RAN paging would fail. Companies discussed candidate solutions in the previous meetings, and the solutions can be generally classified into three directions:

- Proactive context pushing

In this solution, the last serving gNB can proactively push the UE context to the next satellite (we call it candidate gNB in our following analysis). But as we proposed in our papers for the previous meetings, there exists the following problem for pushing the UE context:

When the INACTIVE UE resuming connection, the UE’s verification is executed by the candidate gNB via comparing the shortMAC-I reported by the UE’s RRCResumeRequest and included in the RETRIEVE UE CONTEXT REQUEST, and the shortMAC-I computed by itself. Specifically, the candidate gNB computes the shortMAC-I using the Krrcint as the key, and the source C-RNTI, source PCI, target cell ID as the input parameters. The key Krrcint should be the key of the gNB releasing the UE to INACTIVE, i.e., the last serving gNB, which is different from the candidate gNB. But normally the key of a gNB cannot be transferred to another gNB. Therefore, to make the context pushing solution feasible, the last serving gNB should infer all the possible target cells where the UE may resume the connection, and generate the possible shortMAC-I values using the Krrcint of itself and the possible target cell IDs, and send these together with the UE context to the candidate gNB, which is hard to be implemented and with heavy signalling overhead.

- Implementation based via Stage-2 descriptions

The INACTIVE UE’s last serving gNB can simply initialize the RAN paging for the UE, when the Xn interface is to be disconnected between the last serving gNB and a member in the gNBs relevant to the UE's RNA. In this way, the UE context can be guaranteed to be relocated to its current camped gNB, which is the UE’s current serving gNB after the paging, and the key is also refreshed.

This enhancement causes RAN paging signalling before the UE context “may” be lost. It is essentially a solution to sacrifice some signalling overhead to ensure the reliability as well as Inactive benefits. But generally, the Xn can be maintained either via the ISL or via the feeder link. Hence the RAN paging can happen less than the expected, and such signalling overhead is at the tolerable degree.

All thoughts this solution is already possible today without any change on the current specification, the specification does not preclude than the gNB initiate a RAN paging by it own.

- UE contexts stored on ground or kept in the last serving gNB for NG fetch

There were some proposals to store the INACTIVE UE context on the ground gNB or the ground AMF, or the UE’s current gNB fetches the context from the anchor gNB with the help of ground AMF.

First, the context storage at the ground gNB is unworkable, considering the TN-NTN Xn interface and TN-NTN connected mode mobility is out of scope for this release.

Second, for the context storage at ground AMF, this is a totally new function from the AMF perspective. Also, it may happen that the UE’s current gNB may be inaccessible to the ground AMF storing the UE context.

At last, for the “context fetching from the anchor gNB via ground AMF”, even if the route path is possible, it comes to the question why the logical Xn connection cannot be maintained while the NG connection is available. It seems easier that the Xn can be maintained via the feeder link.

Based on the analysis above, direction 2 seems to be simple and with little cost and spec impact.

- RAN3 to agree the implementation-based solution via stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

## 2.3	OAM/pre-configuration and the SA2/SA5 LS

The RAN3#125bis made the following agreement, sent the LS to SA2 and SA5 respectively, and received the LS back in [4] and [5].

- The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration

The SA2 LS provide the following understandings, where it can be observed they need further confirmation of RAN3 solutions.

SA2 discussed the OAM based solution and aspects related to it are unclear to SA2. SA2 understands that the TAIs broadcast by an NTN gNB are earth static, and the TAIs broadcast by the NTN gNB may change with the movement of the satellite. SA2 also understands the NG Removal procedure is defined to remove interface between a gNB and an AMF. SA2 believes after receiving a NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMFs serving area (e.g., the serving PLMN allowed operation area). Therefore the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway. SA2 would like confirm these understanding with RAN3. - SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts. - The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs. SA2 would like to confirm whether RAN3 and SA5 are aligned on the frequency and contents of the OAM updates to the AMF. Some companies in SA2 have concerns with the solutions agreed by RAN3, which changes the functional split between RAN and the core network.

Also, SA5 provided the reply LS as follows, where SA5 agreed RAN3 solutions and would support it in the normative work.

# 1 Overall description 3GPP SA5 thanks RAN3 for the incoming LS on OAM requirements to support regenerative payload. SA5 would like to inform RAN3 that SA5 Rel 19 study “SP-231733 New SID on Management Aspects of NTN Phase 2” has investigated and agreed the potential solutions of providing the information of supported TAIs for NTN gNB to AMF in TR 28.874. Further, SA5 will consider the solution in normative work in Rel-19. # 2 Actions To RAN3 ACTION: 3GPP SA5 asks RAN3 to take the provided information into account.

When further looking at the TR 28.874, SA5 already agreed to include NTNgNBCapability (including gNB ID and TAI list) towards the AMF, which is associated with a time window.

To support REQ-NTN-FUN-02, new IOC NTNgNBCapability is introduced which is name contained by AMFFunction IOC. Different instances of NTNgNBCapability can be associated to time windows which reflects to their valid durations. Figure 5.2.2.3.2-1: NRM fragment for NTNgNBCapability The NTNgNBCapability IOC includes attributes defined in the Table 5.2.2.3.2-1 and 5.2.2.3.2-2. Table 5.2.2.3.2-1: attributes for NTNgNBCapability IOC Attribute name; S; isReadable; isWritable; isInvariant; isNotifyable. nTNgNBId; M; T; T; F; T. supportedTAIList; M; T; T; F; T.

Observation: SA5 already agreed to include NTNgNBCapability (including gNB ID and TAI list) towards the AMF, which is associated with a time window.

From the above, it seems to us that the main question from SA2 can be resolved. On the impacts on the existing procedures, we think RAN3 and SA5 are aligned,  there is no any impact of the OAM solution in RAN3, the updating TAI(s) via the NG configuration update procedure is not necessary due to the time windows in SA5 solution.

On the frequency of updates by OAM, from the TR 28.874 defined in SA5, the OAM based solution should be one-time configuration, including the mapping of the supported TAI list and the time window information, based on the deterministic movement of the satellite. And the configuration towards the AMF works when the related gNBs is setup the NG interface with the AMF, and before the NG removal. Hence for the AMF, the supported TA List can be initially sent in the NG setup message, afterwards, the TA list can be acquired from the configuration information from the OAM.

Clearly, the benefit of the OAM reconfiguration is to reduce the additional signalling cost due to the satellite movement.

- Reply to SA2 including the impacts on the existing procedures and the frequency of updates by OAM.

- On the impacts on the existing procedures, RAN3 concludes no any impact of the OAM solution, except not updating TAI(s) via the NG configuration update procedure

- On the frequency of updates by OAM, RAN3 understands that the OAM configuration for the gNB supported TAI list on the AMF is one-time configuration including the mapping of the supported TAI list and the time window information. The details would be up to SA5 final decision. Typically, for the AMF, the supported TAI List can be initially sent in the NG setup message, afterwards, the TAI list can be determined from the configuration information from the OAM before the NG removal procedure.

We provide the TS 38.300 TP for capturing the OAM based configuration in the Annex, and provide the reply LS in our paper [6].

- RAN3 to agree the stage 2 TP for capturing the OAM based configuration.

# 3	Conclusion

This contribution discusses the regenerative payload for the NR NTN. It is proposed:

Observation 1: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

- RAN3 to agree the stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

- The OAM configuration for the gNB supported TAI list on the AMF is one-time including the mapping of the supported TAI list and the time/location information, after the NG setup and before the NG removal.

# 4	References

- RAN3#126 Chair notes.

- R3-245434, (TP for TS 38.300) Discussion on RAN Signaling impacts for NR NTN Regenerative Payload, Qualcomm Incorporated, RAN3 125b.

- RAN3#125bis Chair notes.

- S2-2413030, LS on reply to LS on OAM requirements to support regenerative payload, SA2#166

- S5-247311, LS reply on OAM requirements to support regenerative payload, SA5#158

- R3-250402, [draft] Reply LS on OAM requirements to support regenerative payload, Huawei, RAN3#127

# 5	Annex: TP for BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Changes begin>>>>>>>>>>>>>>>>>>>>

### 16.14.6	AMF (Re-)Selection

The gNB implements the NAS Node Selection Function specified in TS 38.410 [16].

For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE).

For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network.

NOTE:	UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

NEXT CHANGE

### 16.14.X	INACTIVE support in regenerative payload

In case that the Xn disconnection between the two on-board gNBs is foreseeable, the UE's last serving gNB may send RAN paging messages to candidate gNB(s) included in the UE's RNA, and transfer the UE context to the UE’s current serving gNB when receiving the RETRIEVE UE CONTEXT REQUEST message.

NEXT CHANGE

### 16.14.Y	Supported TAI in regenerative payload

The supported TAIs of the regenerative payload may be configured to the AMF by the O&M. Under the configuration from the O&M, the on-board gNB may not update the supported TAI list towards the AMF when the TAI(s) changes.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>