3GPP TSG-RAN WG3 Meeting #127	R3-250405

Athens, GR, 17-21 Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1963 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Full eNB as Regenerative Payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Thales, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the WID in RP-243278, this CR provides the necessary signalling support for full eNB as regenerative payload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Regenerative payload with eNB on board the satellite, including: - introduce S1 Removal procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The full eNB as Regenerative Payload is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.x.1, 8.7.x.2, 8.7.x.3, 8.7.x.4, 9.1.8.x1, 9.1.8.x2, 9.1.8.x3, 9.3.2, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

### 8.7.x	S1 Removal

#### 8.7.x.1	General

The purpose of the S1 Removal procedure is to remove the interface instance between the eNB and the MME in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: S1 Removal, successful operation

The eNB initiates the procedure by sending the S1 REMOVAL REQUEST message to the MME. Upon reception of the S1 REMOVAL REQUEST message, the MME shall reply with the S1 REMOVAL RESPONSE message. After receiving the S1 REMOVAL RESPONSE message, the eNB shall initiate removal of the TNL association towards the MME and may remove all resources associated with that interface instance. The MME may then remove all resources associated with the eNB.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.x.3-1: S1 Removal, unsuccessful operation

If the MME cannot accept to remove the information for a TNL association with the eNB, it shall respond with an S1 REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.8.x1	S1 REMOVAL REQUEST

This message is sent by the eNB to initiate the removal of information for the interface instance.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |

#### 9.1.8.x2	S1 REMOVAL RESPONSE

This message is sent by the MME to acknowledge the initiation of removal of information for the interface instance.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | M |  | PrintableString(SIZE(1..150,…)) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.x3	S1 REMOVAL FAILURE

This message is sent by the MME to indicate that removing information for a TNL association cannot be accepted.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.2	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UEContextResumeFailure,

ConnectionEstablishmentIndication,

NASDeliveryIndication,

RetrieveUEInformation,

UEInformationTransfer,

ENBCPRelocationIndication,

MMECPRelocationIndication,

SecondaryRATDataUsageReport,

UERadioCapabilityIDMappingRequest,

UERadioCapabilityIDMappingResponse,

HandoverSuccess,

ENBEarlyStatusTransfer,

MMEEarlyStatusTransfer,

S1RemovalFailure,

S1RemovalRequest,

S1RemovalResponse

. <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-UERadioCapabilityIDMapping,

id-HandoverSuccess,

id-eNBEarlyStatusTransfer,

id-MMEEarlyStatusTransfer,

id-S1Removal

FROM S1AP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

S1AP-ELEMENTARY-PROCEDURES S1AP-ELEMENTARY-PROCEDURE ::= {

S1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

S1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

S1AP-ELEMENTARY-PROCEDURES-CLASS-1 S1AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation				|

handoverResourceAllocation		|

pathSwitchRequest 				|

e-RABSetup						|

e-RABModify						|

e-RABRelease					|

initialContextSetup				|

handoverCancel					|

kill							|

reset							|

s1Setup							|

uEContextModification			|

uEContextRelease				|

eNBConfigurationUpdate			|

mMEConfigurationUpdate			|

writeReplaceWarning				,

...,

uERadioCapabilityMatch						|

e-RABModificationIndication					|

uEContextModificationIndication				|

uEContextSuspend							|

uEContextResume								|

uERadioCapabilityIDMapping					|

s1Removal

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

eNBEarlyStatusTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBEarlyStatusTransfer

PROCEDURE CODE			id-eNBEarlyStatusTransfer

CRITICALITY				reject

}

mMEEarlyStatusTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEEarlyStatusTransfer

PROCEDURE CODE			id-MMEEarlyStatusTransfer

CRITICALITY				ignore

}

s1Removal S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		S1RemovalRequest

SUCCESSFUL OUTCOME		S1RemovalResponse

UNSUCCESSFUL OUTCOME 	S1RemovalFailure

PROCEDURE CODE			id-S1Removal

CRITICALITY				reject

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- UE RADIO CAPABILITY ID MAPPING PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Radio Capability ID Mapping Request

--

-- **************************************************************

UERadioCapabilityIDMappingRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityIDMappingRequestIEs} },

...

}

UERadioCapabilityIDMappingRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UERadioCapabilityID		CRITICALITY reject	TYPE UERadioCapabilityID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE Radio Capability ID Mapping Response

--

-- **************************************************************

UERadioCapabilityIDMappingResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityIDMappingResponseIEs} },

...

}

UERadioCapabilityIDMappingResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UERadioCapabilityID				CRITICALITY reject	TYPE UERadioCapabilityID				PRESENCE mandatory	}|

{ ID id-UERadioCapability					CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- S1 REMOVAL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- S1 Removal Request

--

-- **************************************************************

S1RemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1RemovalRequestIEs} },

...

}

S1RemovalRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Global-ENB-ID				CRITICALITY reject	TYPE Global-ENB-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- S1 Removal Response

--

-- **************************************************************

S1RemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1RemovalResponseIEs} },

...

}

S1RemovalResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MMEname						CRITICALITY ignore	TYPE MMEname					PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

##### -- S1 Removal Failure

--

-- **************************************************************

S1RemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1RemovalFailureIEs} },

...

}

S1RemovalFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-M5ReportAmount									ProtocolIE-ID ::= 347

id-M6ReportAmount									ProtocolIE-ID ::= 348

id-M7ReportAmount									ProtocolIE-ID ::= 349

id-TimeBasedHandoverInformation						ProtocolIE-ID ::= 350

id-Bearers-SubjectToDLDiscarding-Item				ProtocolIE-ID ::= 351

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseUELocationRequested						ProtocolIE-ID ::= 353

id-CoarseUELocation									ProtocolIE-ID ::= 354

id-S1Removal										ProtocolIE-ID ::= aaa

END

-- ASN1STOP

Change Ends