3GPP TSG-RAN WG3 #127	R3-250407

Athens, Greece, Feb 17-21, 2025

Agenda Item:	10.2

Source:	Ericsson

Title:	(TP for BL CR to 38.473 for SON) MRO enhancements for LTM

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM and for and CHO with candidate SCGs.

# 2	MRO enhancements for LTM

During RAN3#126, the following agreements and FFSs were taken on MRO for LTM:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

…

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

Whether network based solution is needed? What’s the UE context to be stored in RAN side?

…

UHI and ping-pong:

Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)?

CU sends UHI to DUs?

The next sections will discuss the scenarios to be discussed in RAN3.

## 2.1	Source C-RNTI over F1 for RLF Report

During RAN3#126, there was a discussion on addition of the source C-RNTI to the ACCESS AND MOBILITY INDICATION message, for the case where that message transfers a RLF Report from the CU to the DU. The source C-RNTI has already been added to the message in the BL CR to 38.473, however the documentation of the attribute containing the C-RNTI is still under discussion.

The C-RNTI attribute is specified to be optional. The reason is that the attribute is only relevant to send if the source DU is different from the target DU, i.e. a for a failure during an Inter-DU cell switch.

There are several possible places for specifying the details on when to include the source C-RNTI in the ACCESS AND MOBILITY INDICATION message. RAN3#126 contribution R3-247159 cl. 2.3.1 proposes that the documentation for this attribute is located in 38.401, NG-RAN; Architecture description. This proposal suggests specifying details of the attribute in a Stage 2 document. We believe that this is not a proper place for such details.

Instead, we propose to place the specification of the details of the Source C-RNTI in the Stage 3 document where this attribute is actually detailed, i.e. in TS 38.473, NG-RAN; F1 Application Protocol (F1), see TP for BLCR in Annex 1.

Avoid specifying the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.401

Specify the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.473, clause 9.2.10.1. See TP for BLCR in Annex 1.

## 2.2	Optimization of early UL synchronization

Early UL synchronization is an important part of LTM, since it is one of the mechanisms that allows the LTM cell switch execution to take place with really short interruption. To achieve this benefit, the source DU has to ensure that the early UL synchronization takes place with some margin before the LTM cell switch execution is actually triggered. On the other hand, if the source DU lets the TA obtained through early UL synchronization become too old, there is a risk that it is outdated and incorrect at the time of the LTM cell switch execution, potentially leading to an LTM failure. How fast a TA obtained through early UL synchronization becomes outdated is highly dependent on the circumstances, e.g. if UEs generally move fast or slow in the source cell or in certain beams in the source cell. This implies that it would be useful for the source DU to receive feedback on how well the early UL synchronization it triggers perform, e.g. how large are the UEs' transmission errors when they access the LTM target cell? Was it close to being so large that successful reception/decoding would have been precluded in the target DU? It is therefore proposed to acknowledge this scenario and to propose solutions to optimize early UL synchronization.

Also, during RAN2#127bis it was discussed whether RAN3 will develop a network based solution for logging whether the UE used a network provided TA or a UE calculated TA during a LTM cell switch attempt. RAN2 took the following agreement:

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

A network based solution has the drawback that the RLF Report may be kept in the UE for up to 48 hours. For a network based solution to work, the gNB would therefore need to store the UE context for up to 48 hours. As the responsibility for choosing between a Network configured TA value and a UE measured TA value lies within the DU, the UE Context needs to be stored in the DU, at MAC level. The volume of UE contexts to store in the DU would be prohibitively large, and in general this possibility is only discussed for gNB-CUs. For this reason, a Network based solution is infeasible.

The drawback for a UE based solution is the need to store one bit on whether the last LTM cell switch used a network or UE determined TA value, and to keep this bit until a RLF Report is deleted.

For these reasons, we propose a UE based solution.

RAN3 to agree on a UE based solution for logging  whether the UE used a network provided TA or a UE calculated Tain a failed LTM cell switch attempt.

RAN3 to send a LS to RAN2 on accepting the RAN2 solution. RAN3 will not develop a network-based solution.

## 2.3	Optimization of LTM cell switch ping-pong

Ping-pong LTM cell switch is a condition that needs to be avoided. Optimizing configuration to avoid ping-pong is done differently depending on where the ping-pong occurs.

- For intra-DU ping-pong, the DU is expected to optimize ping-pong by itself. Therefore, no communication between nodes needs to be specified.

- For Inter-DU ping pong, it should be noted that both the source and target cells are LTM candidate cells. Therefore, both the CU and the source and target DUs have copies of the UE context, and will keep it until after a successful LTM cell switch. Inter-DU ping pong has two sub cases:

  - For Inter-DU ping pong, as both the source and target DUs keep a copy of the UE context after cell switch, both source and target DUs knows the mobility history of the UE and therefore can detect ping-pong.

  - As the CU keeps a copy of the UE context, and as DU sends a F1AP ACCESS SUCCES message (including the new target cell ID) to the CU after a LTM cell switch, the CU can also detect inter-DU ping pong.

For these reasons, there is no need to send a list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

There is no need to send any list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

# 4	References

R2-125bis_Maint_eRedCap_SON-MDT_(Mattias)_20240419_Final.docx

3GPP TS 38.300, "NR and NG-RAN Overall Description; Stage 2"

R3-243846, Summary of Offline Discussion on MRO for LTM

# 5	Conclusion

- Avoid specifying the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.401

Specify the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.473, clause 9.2.10.1. See TP for BLCR in Annex 1.

RAN3 to agree on a UE based solution for logging  whether the UE used a network provided TA or a UE calculated Tain a failed LTM cell switch attempt.

RAN3 to send a LS to RAN2 on accepting the RAN2 solution. RAN3 will not develop a network-based solution.

There is no need to send any list of recently visited cells, for example the UHI, between any nodes involved in LTM switch ping pong.

# Annex 1:	TP to BLCR 38.473

First change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU in case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

End of changes