3GPP TSG-RAN WG3 Meeting #127	R3-250409

# Athens, Greece, 17th – 21st February, 2025

Title: 	On MDT enhancements for Network Slicing

Source: 	Ericsson, Jio Platforms (JPL), Interdigital, FiberCop

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

In meetings leading up to TSG-RAN WG3 #126 the following agreements and FFSs were made with respect to the future work on SON/ MDT enhancements for Network Slicing:

- For the immediate MDT, it is not needed to add measurements related to the slice unavailability and the lack of resources. - The slice ID will be added as a filter to the area scope (for the both, management- and signalling-based immediate MDT related to slice). The intention is to limit the number of reports sent to the OAM, thus the UE selection will not be amended. - Not to enhance MDT area scope for logged MDT for slice. - RAN3 will further discuss mechanism enabling monitoring NSAG configuration and check with RAN2 if needed

In this contribution, we share our views on further MDT related enhancements for Network slicing, where we present the motivation and benefits of MDT measurement collection/reporting subject to conditions.

# 2	Discussion

- In management-based immediate MDT, the OAM configures individual RAN nodes with MDT measurement configuration and the RAN nodes perform UE selection and collection of MDT measurements. The collected measurements are then sent back to the OAM. The UE selection process is based on the configuration provided by the OAM, i.e., based on the provided area scope, and after taking considerations of UE capability, consent, etc. However, the MDT measurement/report collection starts immediately upon selection of a suitable UE.

- Similarly, in signalling-based immediate MDT, the OAM and the Core Network tasks the RAN to configure MDT measurements on a specific UE, and the UE is configured immediately. The measurement in this case too commences immediately at the UE and the results are then signalled back to the OAM.

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately upon receiving an MDT configuration.

- Depending on the existing network conditions during the period over which the OAM configures the RAN to report MDT measurements, it may happen that the utilization of a slice for which measurements are collected fluctuates between low and high, while the overall load on the RAN node may have followed a different or the same trend. The reported performance of UEs on the slice of interest may be good, e.g., during non-peak utilization of the slice, while it may have degraded when the slice utilization was high. It should also be noted that the peak and low utilization periods of a slice may be different when compared to the peak and low utilization periods of the overall RAN node. Furthermore, the performance of the UEs on a certain slice may or may not be dependent on the RAN node’s utilization, e.g., it may depend on the type of slice, policies associated with the slice/RAN node, etc.

Observation 2: If OAM collects MDT reports for a specific slice irrespective of the existing slice utilization or the RAN node utilization, the collected information may not be interpretable without additional information about slice utilization and RAN utilization. The OAM or the node performing the data analysis would need access to the existing load conditions on the slice and the RAN node to be able to derive useful insights.

- Depending on the situation, obtaining this additional information may be challenging, or involve considerable overheads, i.e., the collected MDT data must be correlated with the existing network conditions when the measurement was collected. This additional information shall be signalled to the OAM, and when analysed, it may reveal that the subset of data which is actually interesting to analyse further is a small fraction of the originally collected dataset.

- In the context of Network Slicing, where each slice is a logically isolated network on top of a shared physical network, observing the performance of each of slices to ensure that the slice-related QoS/QoE/SLA parameters are fulfilled is vital for the delivered services. However, the delivered service characteristics on a certain slice may depend on the underlying network/hardware/load-related conditions which may need to be carefully correlated with the data collected to monitor SLA guarantee/QoS monitoring.

- Networks are typically dimensioned such that they can fulfil a certain peak demand. The MNO derives the SLA it can offer to services and customers based on careful consideration of the above. In well-designed networks, breaches to SLA or QoS guarantees are rare events and depend on dynamic factors such as load, and other conditional events. Therefore, utilizing a general-purpose data collection framework for the purpose of identifying a small fraction of events is not well suited.

- Observation 3: Conditional observability into slice-level performance can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

- Thus, to increase slice specific observability and transmission of pertinent data, i.e., data that is collected when the necessary pre-conditions are met, it is important to have the capability to collect MDT measurements based on slice-specific KPIs available in RAN. For example, if the RAN nodes are provided with a criterion to collect MDT measurements for a slice only when the slice specific load is very high or the overall RAN node utilization is very high, it would be able to observe if there is a degradation in the slice performance. When the slice specific load or the overall RAN utilization is low, there may be no incentive to obtain detailed measurements, e.g., MDT, and the SLA/QoS monitoring may be performed via existing mechanisms.

Observation 4: By collecting detailed reports in periods where issues are likely to manifest, e.g., during high slice utilization period, and not collecting the detailed information when indicators of potential issues are absent will enable the network to perform focussed data collection.

In the context of management-based immediate MDT, the existing MDT framework allows the OAM to specify the Area Scope as a UE selection criterion. While this enables physically identifying and selecting UEs in certain cells, TAs, etc., the selection criteria does not determine when the measurements are done, or which of the collected measurements are signalled to the OAM. In the current MDT framework, measurements are always collected and transmitted to the OAM after a UE has been selected (as described in observation 1).

The points made in this paper identify and motivate for measurement/reporting criteria, where for a UE that has previously been selected (in accordance with a selection criteria), the measurements are collected/reported to the OAM only upon fulfilment of the measurement/reporting criteria, which may be defined in terms of thresholds on node-level or slice-level utilisation values.

Proposal 1: Enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on node-level or slice-level utilization to be considered for further normative work.

The above-mentioned discussion points, in terms of the need for slice-specific observability and collection of data only when certain conditions are met, is also true in the context of signalling-based immediate MDT, where the gNB knows the UE for which MDT measurements are to be collected. Compared to management-based immediate MDT, signalling-based immediate MDT is less scalable as the number of UEs selected increases. With the proposed conditional MDT enhancements, the scalability of signalling based immediate MDT can be improved, with the additional benefit of obtaining more focussed and granular measurements only when the specified pre-conditions are met.

Furthermore, in the current framework for both signalling and management based MDT, once a UE has been selected for MDT measurements collection/reporting by the gNB (in case of management-based MDT) or by the OAM/Core Network (in case of signalling-based MDT), the measurements are collected independently of the slice used by the UE (the slice on which the UE has one or more active PDU sessions). In the context of increasing the slice observability and more specifically in the context of signalling-based MDT, is highly important to ensure that measurements for the selected UE(s) are collected only while the UE(s) is using the slice(s) under observation.

Observation 5: Current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are actively used by selected UE(s).

Proposal 2: Enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on node-level and/or slice-level utilization and only while the UE is actively using a given slice(s) to be considered for further normative work.

It is also important to note that the Slice utilization and the overall RAN utilization can fluctuate significantly during a short period of time. In order to guarantee that the collected measurements pertain to the specified collection criteria, it should be possible to activate/deactivate or pause/resume the slice specific MDT measurements without affecting the MDT measurement configuration or requiring a reconfiguration.

Observation 6: To obtain slice/RAN load specific measurements, whenever the system-associated or UE-associated metrics are outside the specified collection conditions, the measurement reports should not be collected or not be reported to the OAM.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss the presented enhancements for MDT configurations.

# Conclusion

In the previous sections we made the following observations:

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately upon receiving an MDT configuration.

Observation 2: If OAM collects MDT reports for a specific slice irrespective of the existing slice utilization or the RAN node utilization, the collected information may not be interpretable without additional information about slice utilization and RAN utilization. The OAM or the node performing the data analysis would need access to the existing load conditions on the slice and the RAN node to be able to derive useful insights.

- Observation 3: Conditional observability into slice-level performance can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

Observation 4: By collecting detailed reports in periods where issues are likely to manifest, e.g., during high slice utilization period, and not collecting the detailed information when indicators of potential issues are absent will enable the network to perform focussed data collection.

Observation 5: Current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are actively used by selected UE(s).

Observation 6: To obtain slice/RAN load specific measurements, whenever the system-associated or UE-associated metrics are outside the specified collection conditions, the measurement reports should not be collected or not be reported to the OAM.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on node-level or slice-level utilization to be considered for further normative work.

Proposal 2: Enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on node-level and/or slice-level utilization and only while the UE is actively using a given slice(s) to be considered for further normative work.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss the presented enhancements for MDT configurations.

The proposals above are mirrored in the Text Proposal for BLCR for MDT for TS 38.413 presented in R3-250410.

# References

RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC