3GPP TSG-RAN WG3 Meeting #127	R3-250414

Athens, Greece, February 17 – 21, 2025

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	AI/ML based positioning accuracy enhancements

Document for:		Discussion and Decision

# 1.	Introduction

At RAN3#125bis and RAN3#126, the "positioning accuracy enhancements" objectives of the "Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface" work item [1] were discussed. The progress is summarized by the following set of agreements/conclusions.

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. - WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. - FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). - FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. - Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. - FFS if LMF should be aware that the reported measurements are AI/ML generated. Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we further discuss the procedures and signalling design to support network-based AI/ML assisted (Case 3a) and direct (Case 3b) positioning.

# 2.	Network-based AI/ML assisted positioning – Case 3a

## 2.1	Data Collection and Model Training

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The model output are "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements dependent on RAN1 progress.

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

The status of the RAN3 discussions related to Data Collection and Model Training is as follows (see section 1):

-	The AI/ML model (inference and training functions) is at the gNB.

-	NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

-	The details of the measurements are still pending and subject to further discussion in RAN1.

-	LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

-	How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

-	FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

-	FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

-	FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

-	WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

As can be seen above, most items for further study (under RAN3 responsibility) are related to the signalling and procedures related to "Part B information" ("ground-truth label" information) provided by an LMF. At RAN3#126, the following general Solutions have been discussed [2]:

-	Solution 1a: gNB-triggered Data Collection Request via a new NGAP message

-	Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message

-	Solution1c: gNB-triggered Data Collection Request via the existing NRPPa message

-	Solution 2: "Xn-like Data Collection" over NRPPa

The Moderator [3] suggested a harmonized solution which is repeated in Figure 1 below. In this procedure [3]:

A.	The LMF proactively indicates that data collection for a list of UEs/TRPs is available via new non-UE associated NRPPa message.

B.	The gNB determines that data collection is needed and sends a request to LMF via new non-UE associated NRPPa message.

C.	If not already configured, the gNB configures the UE with UL SRS to get Part A information. FFS if time stamp or other information (e.g., SRS configuration) can be used to correlate part A and part B info.

Figure 1: Proposed harmonized baseline procedure in [3].

We think the above procedure with some modifications can be used as baseline. In particular, we think Step 1 in this procedure should be optional and the procedure should also work in the case Step 1 does not occur. For example, the NG-RAN node should be allowed to select any connected/known UE for data collection even if not in the list of available UEs/PRUs provided at Step 1 or in the case Step 1 does not occur; e.g., a UE which is already transmitting SRS for non-data collection purpose, etc. Otherwise, Step 1 (potentially without Steps 2-8 being executed) may happen rather frequently – i.e., every time the presence status of UEs changes (i.e., a UE appears/disappears from the AMF in the defined area) – since the LMF does not know if/when a gNB needs to perform model (re-)training and/or performance monitoring.

The list of UEs provided at Step 1 to the NG-RAN node may also need to be on "cell-ID level", otherwise the NG-RAN node would not know which UEs in the provided list are in the coverage area of a particular TRP. This implies that the LMF must perform some filtering of UE IDs, Cell-IDs and TRP IDs, which may only be reliable for (RRC) connected UEs (i.e., UEs cell-ID locations known) or stationary PRUs.

The above complexity can be avoided if the NG-RAN node can "blindly" request data collection from an LMF for an already present UE. Note, this request may fail since the gNB may select a UE which has LCS privacy restrictions (i.e., at Step 4). However, this can also happen if Step 1 is being performed since the UE privacy settings are not stationary [4]. E.g., a UE may have subscribed to the LCS privacy notification verification and may explicitly accept/deny a location request at each attempt [4]. Therefore, even if Step 1 includes a list of privacy verified UEs, it may still happen that Step 4 will be denied by the UE.

Note, the "request" message towards an LMF (e.g., Step 3 in Figure 1) could also be a new NGAP message (Solution 1a), which however, seems to have more AMF and SA2/CT specification impacts. The AMF would have to understand the new NGAP message and trigger a corresponding service operation towards an LMF for the new request. Given that there is no common UE identifier between NG-RAN node and AMF/LMF, this would also only work for connected UEs/PRUs (i.e., NGAP ID available). With an NRPPa message (which is transported in an NGAP message), the AMF would only have to recognize the NG-RAN assigned Routing ID to select an LMF, but the NRPPa message is still transparent to the AMF, as also described below at Step 4, and could be both, UE-associated and non-UE associated.

A corresponding modification of the procedure in Figure 1 together with additional details is proposed in Figure 2 below.

Figure 2: General data collection procedure for gNB-side model training in Case 3a.

1.	[Optional] In the case the target is a UE, the LMF may subscribe to each AMF serving a specific area to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request operation [4],[6]. The area of interest may expand over multiple TAIs or cover multiple NG-RAN nodes. 
Each AMF then sends a Namf_EventExposure_Subscriber_Response including the list of SUPIs in the area of interest and the UE Positioning Capability for each UE, as defined in [4]. The LMF may also check the user privacy requirements (at this Step or at Step 5 below).

NOTE:	This Step has been defined by SA2 for Case 2b and 3b [4], which can also be applicable here to Case 3a, if needed. This Step also depends on further SA2 progress.

2.	[Optional] The LMF may provide the list of available UEs/PRUs in an area of interest to the NG-RAN. The LMF may repeat this step whenever the presence status of the UEs/PRUs changes (e.g., UEs/PRUs switched off, new UEs/PRUs become available, etc.).

This is a new Class 2 (non-UE associated) NRPPa message which may include the list of available UEs/PRUs, possibly together with other known information (e.g., stationary PRU location). The LMF may assign a Universally Unique Identifier (UUID) to identify each UE/PRU.

3.	The NG-RAN node determines that model (re-)training is required; e.g., based on the model monitoring result and selects a suitable UE.

4.	The NG-RAN node may send a request to an LMF for the ground truth label ("Part B Information") of the selected UE.

Case 1:	If Step 2 happened and if the NG-RAN node selects a UE from the list provided at Step 2, the NG-RAN node sends a new Class 1 NRPPa message to the LMF to request ground truth label information (UE location information). The NG-RAN node sends the NRPPa PDU to the AMF in an NGAP Uplink Non UE Associated NRPPa Transport message. The NG-RAN node includes the Routing ID related to the LMF received at Step 2.
The NRPPa message includes the UE ID received at step 2 for which the ground-truth label is requested.

NOTE: 	If a PRU is selected from the list of available PRUs received at Step 2, and if the PRU is known to be stationary, and if the PRU location has already been provided at Step 2, and if the PRU is already CM_CONNECTED, this step 4 may not be needed.

Case 2:	If Step 2 did not happen, or if the NG-RAN node does not select a UE from the list provided at Step 2, the NG-RAN node sends the NRPPa PDU to the AMF in an NGAP Uplink UE Associated NRPPa Transport message (i.e., the UE is identified by the corresponding NGAP ID). 
The NG-RAN node assigns a Routing ID (aka session ID) for this UE. The Routing ID could be a special/reserved value to indicate to the AMF that this is a NG-RAN triggered NRPPa message. A range of Routing ID values could be reserved for NG-RAN triggered NRPPa messages (i.e., a range of Routing IDs which cannot be assigned by an AMF would enable an AMF to distinguish a Routing ID assigned by a NG-RAN node from Routing IDs assigned by an AMF). 
The AMF then performs the LMF selection and forwards the NRPPa message to this selected LMF.

NOTE:	The NRPPa message is still transparent to the AMF. However, the AMF need to recognize the new Routing ID as a NG-RAN node assigned ID to perform the LMF selection. The LMF selection per se does not need to change; i.e., could be based on criteria already defined in [4], since this "ground-truth label request" could be seen as a normal UE location request. 
This functionality is in principle not different from LMF assigned Correlation IDs to support UE Event Reports for an deferred MT-LR (see e.g., section 6.3.1 of TS 23.273 [4], step 27, NOTE 11), but now assigned by the NG-RAN.

5.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [4] and 38.305 [5] to obtain the UE location (to determine/calculate the ground truth label information).
If the UE/PRU is in CM_IDLE state, the AMF initiates a network triggered Service Request procedure to establish a signalling connection with the UE/PRU, which then also allows the data collection at Step 7/8.
The LMF may also check the LCS user privacy requirements as specified in [4].

6.	The LMF provides the UE/PRU location ('ground-truth label' information) to the serving gNB (via the serving AMF) in response to Step 4 ("Part B Information").

7.	The serving gNB configures a required SRS for model training in the UE/PRU.

NOTE:	For Case 1 in Step 4 (non-UE associated NRPPa message), the gNB could know only after Step 6 for which UE the ground-truth information is provided at Step 6 by assuming Step 5 is a response to Step 4. I.e., the correspondence between LMF-assigned UE-ID from Step 2/Step 4 (Case 1) and the actual UE-ID known in the gNB to configure a required SRS, would only be known after some UE-associated messages have been exchanged during Step 5.

NOTE:	The gNB may need to train models for a variety of different SRS configurations. I.e., the model output during inference may be dependent on the used SRS parameter (e.g., bandwidth, number of symbols, etc.).

8.	The NG-RAN performs the UL SRS measurements and derives any data required for model training ("Part A Information").

NOTE:	For Case 2 in Step 4, Steps 4-6 may also happen after Step 8.

From the above description, the following observations and proposals are made (considering the FFS items summarized in section 1 above):

Observation 1:	The general procedure for data collection in Case 3a should also work in the case an LMF does not provide a list of available UEs/PRUs to the NG-RAN (i.e., Step 1 in Figure 1 or Step 2 in Figure 2 does not occur), because of the following disadvantages:

-	The message may have to be sent rather frequently (i.e., every time the presence status of a UE/PRU changes in the defined area or the UE privacy settings change).

-	The NG-RAN and LMF have no common understanding of the UE-IDs provided/selected by an LMF, which has multiple implications:

-	The list of available UEs/PRUs must be on Cell-ID level, otherwise it would be unclear how the NG-RAN node can select a particular UE/PRU (e.g., may select a UE/PRU outside gNB coverage). However, knowledge of Cell-ID location at AMF/LMF may only be reliable for RRC connected UEs/PRUs.

-	The gNB may not always be able to reliably correlate the LMF provided information (e.g., Part B ground-truth label information) to a particular UE to configure a required SRS for the UE.

Observation 2:	Using a NG-RAN node initiated NRPPa message to request information from an LMF (e.g., Part B ground-truth label information) seems simpler than using a new NGAP message. A new NGAP message may have more AMF impacts, since it would have to trigger a new or updated service operation at the AMF towards an LMF, and therefore, would also have SA2/CT impacts. The NRPPa message would still be transparent to the AMF and uses existing transport mechanisms.

Proposal 1:	The gNB determines that data collection is needed (confirm the WA).

Proposal 2:	A NRPPa transaction can also be triggered by the NG-RAN (not confirm the WA).

Proposal 3:	Define a new Class 1 NRPPa procedure which allows a gNB to request ground-truth label information from an LMF. The NRPPa transport can use UE associated and non-UE associated signalling.

Proposal 4a:	For non-UE associated signalling of the new NRPPa procedure according to Proposal 3, the LMF determines an identity for the UE (e.g., a Universally Unique Identifier (UUID)). This UE identity is provided to the NG-RAN before using this new Class 2 NRPPa procedure.

Proposal 4b:	For UE associated signalling of the new NRPPa procedure according to Proposal 3, the UE is identified by the corresponding NGAP ID.

In the example of Figure 2, a serving gNB collects data for training its own gNB-side model using UEs in the coverage area of this gNB (Step 7/8). However, positioning usually requires a multitude of TRPs. For positioning, a gNB/TRP usually measures UEs SRS transmission which are not in the coverage area of the measuring gNB/TRP (neighbour TRPs).  Therefore, the gNB-side models may need to be trained with SRS transmission of 'far away' UEs to obtain a similar situation as for the actual inference operation (UE position location operation). This may require that the LMF is involved in the initiation of training data collection (e.g., SRS configuration), as illustrated in Figure 3 below.

Figure 3: General data collection procedure for gNB-side model training in Case 3a.

0.	Steps 1-3 as in Figure 2 may occur between LMF and Requesting NG-RAN Node.

1.	The Requesting NG-RAN Node may select a UE for SRS transmission not in the coverage area of this gNB. This may be for example a PRU whose location is not close enough to the Requesting NG-RAN Node or a UE likely not in the coverage area of the Requesting NG-RAN Node (which may be inferred from the LMF provided list at Step 2 in Figure 2). The Requesting NG-RAN Node then sends a request to the LMF via the serving AMF for 'model training'.  This could be the same message as in Step 4 of Figure 2 or a separate Class 2 procedure.

2.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [4] and 38.305 [5] to obtain the UE location. (Note, the UE/PRU would be identified by the Requesting NG-RAN Node via the UE ID received at Step 2 in Figure 2. The LMF would know the corresponding SUPI from Step 1 in Figure 2 and can request the transfer of positioning LPP/NRPPa PDUs from the AMF to the UE/serving gNB for the particular UE.)

3.	The LMF requests the serving gNB of the selected UE to configure a required SRS in the UE.

4.	The LMF sends a NRPPa Measurement Request message to the Requesting NG-RAN Node and possibly also to the serving and other neighbour NG-RAN Nodes, even though they have not requested "Data Collection". It could be up to these NG-RAN Nodes whether to perform data collection or not with this UE. The LMF may include the UE location ('ground-truth label') obtained at Step 2 in the Measurement Request message ("Part B Information").

5.	The requesting (and possible other/neighbour NG-RAN nodes) perform the UL SRS measurements and derive any data required for model training ("Part A Information").

6.	The NG-RAN Node(s) send a NRPPa Measurement Response to the LMF without measurements indicating that the data collection for model training has finished. (Note, since there are no measurements to report in this case, the NRPPa Measurement Response message could indicate that data collection is complete and the LMF could e.g., deactivate SRS etc.).

Proposal 5:	Define a new NRPPa procedure which allows a gNB to request model training initiation from an LMF. This could be the same procedure as in Proposal 3.

## 2.2.	Model Performance Monitoring

According to TR 38.843 [7], Model Monitoring is defined as follows:

Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

Therefore, "model monitoring" is closely related to "model inference".

For model performance monitoring, RAN3 made the following agreement:

NG-RAN node performs monitoring metric calculation for its own model.

For performance monitoring, the gNB could measure the SRS and derive the model input data and perform the model inference to obtain the "SRS-based measurements" (e.g., AI/ML enhanced legacy measurements (e.g., RTOA, UE Rx-Tx Time Difference Measurements) or any new measurements, dependent on RAN1 progress). To obtain the 'ground-truth label', the gNB would need the 'true' UE location and the 'true' gNB/ARP location (for example, to determine a 'true' RTOA, which depends on the distance between UE and gNB/ARP). Assuming the gNB has its own gNB/ARP location coordinates available, the procedure for gNB-side 'Model Performance Monitoring' could be the same as in Figure 2 and 3 with an additional final step to derive the model performance metric based on the model output and UE/gNB/ARP location. The model performance result may trigger the gNB to (re-)train the models as described in Figure 2/3. Therefore, no additional procedure seems needed for model performance monitoring in Case 3a.

Observation 3:	The model performance monitoring at gNB-side for Case 3a can share the same procedures as for model training.

## 2.3.	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [7] defines in section 7.2.4 the following:

- Management: o For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. o For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB. o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For gNB-side models (Case 3a), the functionality control can be inside the gNB. The gNB may decide on a particular model based on e.g., requested measurement type, currently used SRS configuration, etc. When requesting measurements from a gNB, the LMF specifies the measurements needed and provides all relevant information to the gNB in the NRPPa Measurement Request [8], incl. e.g., the SRS configuration. Therefore, a gNB should have all information available to perform the functionality management operation of activating/deactivating a particular model or fallback to non-AI/ML based measurements.

Note, a fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements can not be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

Proposal 6:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

Observation 4:	A fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements cannot be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

# 3. 	Network-based direct AI/ML positioning – Case 3b

## 3.1	Data collection and Model Training

For Case 3b, the AI/ML model is deployed at the LMF. The model output is the estimated UE location. The NG-RAN/TRP measurements may be the existing UL SRS measurements or new measurements dependent on RAN1 progress.

Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

For Case 3b (LMF-side models), the LMF performs the 'Data Collection and Model Training' [4]. This also implies that an LMF is collecting the necessary data from the TRPs/NG-RAN to perform the 'Model Training'. The trigger for data collection and model training by an LMF is up to implementation [4].

A general procedure for Case 3b data collection is already specified in [4], which is repeated in Figure 4 below. The procedure has the following Editor's Note:

Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

In the LS in [9], SA2 asked RAN3 to provide feedback:

SA2 asks RAN3 to provide feedback on the Editor’s Notes in Clause 5.18 and Clause 6.22.3 in TS 23,273 v.19.1.0 and to provide references to the corresponding procedures in RAN TSs (or an estimation on when those procedures can be available).

Figure 4: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (according to [4]).

At Step 6 in Figure 4, the LMF performs the data collection from NG-RAN. This requires UEs/PRUs transmitting UL SRS for positioning. Existing NRPPa Positioning Information Exchange procedures [8] allow an LMF to request UL SRS transmission from a UE/PRU. Similar, existing NRPPa Measurement Information Transfer procedures [8] allow an LMF to obtain desired UL measurements from TRPs.

At Step 7 in Figure 4, the LMF obtains the UE/PRU 'ground-truth location' like in Step 5 in Figure 2. Based on the discussion above, an extension of the procedure in Figure 4 is shown in Figure 5.

Figure 5: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (extension of Figure 4).

1.-5.	As defined by SA2 in [4], section 6.22.3.

6.	The LMF performs the NRPPa Positioning Information Exchange procedure to request the serving NG-RAN node to configure a required SRS in the target (UE or PRU).

NOTE:	Additional parameter values for SRS configuration (if any) depend on RAN1.

7.	The LMF sends a NRPPa Measurement Request to the serving and possibly neighbour NG-RAN nodes (TRPs) to request the SRS-based measurements for model training. This may be a request for any of the existing measurement types and/or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.).

8.	The TRPs provide the measurement results for model training to the LMF in a NRPPa Measurement Response message.

9.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [4] and 38.305 [5] to obtain the UE/PRU location.

NOTE:	The positioning method used for obtaining the UE/PRU location should not be based on the same SRS as used for steps 6-8, since the UE/PRU location should be independent on the actual data collected for model training. E.g., the UE/PRU location may be determined via GNSS, DL-TDOA, etc.

NOTE:	In case the target is a PRU, the LMF may already have the PRU location available (e.g., from the PRU registration [4]) and step 9 may not be needed.

After step 9, the LMF would have all the data available for 'Model Training', i.e., the measurement data (SRS-based measurements; Part A) together with the ground truth label (UE location; Part B).

Proposal 7:		For Case 3b ground-truth label generation for model training, existing LCS [4] and UE positioning [5] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 8:		Agree/confirm the general procedure in Figure 5 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

For LMF-side model training (Case 3b) the existing NRPPa Positioning Information Request with the Requested SRS Transmission Characteristics (Step 6a in Figure 5) can be used to request a desired SRS for positioning configuration for the UE/PRU. A LMF may need to train models for a variety of different SRS configurations. I.e., the model output during inference may be dependent on the used SRS parameter (e.g., bandwidth, number of symbols, etc.). For the inference phase, existing positioning procedures can be used. In these procedures, the gNB may provide a particular SRS configuration to the LMF (decided by the gNB) and the LMF would need to search for an applicable model in its model repository suitable to this particular SRS configuration. Therefore, when using the Requested SRS Transmission Characteristics for model training, a LMF may need to specify in more detail a required (and not desired) SRS configuration, such that the LMF can train models for a variety of SRS configurations. The NRPPa Validity Area Specific SRS Information [8] already allows an LMF to specify a more detailed requested SRS characteristics, but currently only when area-specific SRS is requested. This may need to be extended to the 'Model Training' use case, and the 'desired SRS configuration' may need to be defined as a 'required SRS configuration'.

Proposal 9:	For LMF-side model training, the NRPPa Requested SRS Transmission Characteristics need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

## 3.2	Model Performance Monitoring

For LMF-side models (Case 3b) the LMF is performing the 'Model Performance Monitoring' [4]. Therefore, the same procedure as for data collection could be used for performance monitoring. The procedure in Figure 5 would then have an additional 10th step to determine the desired model performance metric using the calculated UE position with the measurements from Step 8 in Figure 5 and comparing the result with the 'ground-truth label' from Step 9 in Figure 5. As discussed in section 3.1 above, a LMF may have several trained models corresponding to several different SRS configurations available. Therefore, also for Model Performance Monitoring, a LMF may need to be enabled to specify a more detailed required SRS Configuration in Step 6a of Figure 5 (see Proposal 9).

Observation 5:	The model performance monitoring at LMF-side for Case 3b can share the same procedures as for model training.

## 3.3	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [8] defines in section 7.2.4 the following:

- Management: o For UE-side model, […] o For gNB-side model, […] o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For LMF-side model (Case 3b), the LMF can request measurements from the gNBs as usual. The requested measurements may be any of the existing measurement types (RTOA, AOA, gNB Rx-Tx Time Difference, etc.) or any new measurements defined by RAN1. The LMF performs the inference operation based on available models. All information required to perform model selection, activation, switching, fallback, etc. should be available at the LMF. Per the study item conclusion shown above, the model/functionality control may be performed by the LMF when the monitoring resides within the LMF, which is the case per SA2 agreements [4]. Therefore, it is proposed to confirm the study item conclusion:

Proposal 10:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

# 4.	Summary

In this contribution, we further discussed the signalling and procedures for Network-based AI/ML assisted positioning (Case 3a) and Network-based direct AI/ML positioning (Case 3b). The following Observations and Proposals were made.

Case 3a:

Data Collection and Model Training:

Observation 1:	The general procedure for data collection in Case 3a should also work in the case an LMF does not provide a list of available UEs/PRUs to the NG-RAN (i.e., Step 1 in Figure 1 or Step 2 in Figure 2 does not occur), because of the following disadvantages:

-	The message may have to be sent rather frequently (i.e., every time the presence status of a UE/PRU changes in the defined area or the UE privacy settings change).

-	The NG-RAN and LMF have no common understanding of the UE-IDs provided/selected by an LMF, which has multiple implications:

-	The list of available UEs/PRUs must be on Cell-ID level, otherwise it would be unclear how the NG-RAN node can select a particular UE/PRU (e.g., may select a UE/PRU outside gNB coverage). However, knowledge of Cell-ID location at AMF/LMF may only be reliable for RRC connected UEs/PRUs.

-	The gNB may not always be able to reliably correlate the LMF provided information (e.g., Part B ground-truth label information) to a particular UE to configure a required SRS for the UE.

Observation 2:	Using a NG-RAN node initiated NRPPa message to request information from an LMF (e.g., Part B ground-truth label information) seems simpler than using a new NGAP message. A new NGAP message may have more AMF impacts, since it would have to trigger a new or updated service operation at the AMF towards an LMF, and therefore, would also have SA2/CT impacts. The NRPPa message would still be transparent to the AMF and uses existing transport mechanisms.

Proposal 1:	The gNB determines that data collection is needed (confirm the WA).

Proposal 2:	A NRPPa transaction can also be triggered by the NG-RAN (not confirm the WA).

Proposal 3:	Define a new Class 1 NRPPa procedure which allows a gNB to request ground-truth label information from an LMF. The NRPPa transport can use UE associated and non-UE associated signalling.

Proposal 4a:	For non-UE associated signalling of the new NRPPa procedure according to Proposal 3, the LMF determines an identity for the UE (e.g., a Universally Unique Identifier (UUID)). This UE identity is provided to the NG-RAN before using this new Class 2 NRPPa procedure.

Proposal 4b:	For UE associated signalling of the new NRPPa procedure according to Proposal 3, the UE is identified by the corresponding NGAP ID.

Proposal 5:	Define a new NRPPa procedure which allows a gNB to request model training initiation from an LMF. This could be the same procedure as in Proposal 3.

Model Performance Monitoring:

Observation 3:	The model performance monitoring at gNB-side for Case 3a can share the same procedures as for model training.

Functionality Management/Control:

Proposal 6:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

Observation 4:	A fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements cannot be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

Case 3b:

Data Collection and Model Training:

Proposal 7:		For Case 3b ground-truth label generation for model training, existing LCS [4] and UE positioning [5] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 8:		Agree/confirm the general procedure in Figure 5 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

Proposal 9:	For LMF-side model training, the NRPPa Requested SRS Transmission Characteristics need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

Model Performance Monitoring:

Observation 5:	The model performance monitoring at LMF-side for Case 3b can share the same procedures as for model training.

Functionality Management/Control:

Proposal 10:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

# References

[1]	RP-243244, "Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	RAN3#126 Chair Notes.

[3]	R3-247803, "Summary of Offline Discussion on CB: # AIPHY", Ericsson (moderator).

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	3GPP TS 29.518: "Access and Mobility Management Services; Stage 3".

[7]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[8]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[9]	R3-250074 (S2-2501341), "LS on LMF-based AI/ML Positioning for case 3b", SA2.

[10]	R3-245461, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated, RAN3#125bis.

[11]	R3-247459, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated, RAN3#126.