3GPP TSG-RAN WG3 #127	 											R3-250416

# Athens, Greece, 17-21 February 2025

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

2. Discussion

2.1 Multiple UE association

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

In the last meeting, we reached a working agreement (WA) to use separate LTM request messages for each candidate cell, i.e., parallel requests. Another issue that came up is whether the source and candidate gNB can assign different Source/Target NG-RAN node UE XnAP ID reference pairs for the same UE when parallel requests are sent to the same candidate gNB.

While supporting multiple associations may offer more flexibility, there are several potential issues that need to be addressed. The candidate gNB may not realize that different Source NG-RAN node UE XnAP ID references point to the same UE, which could impact candidate resource reservation and complicate UE context handling. Moreover, the target node may not be aware when an error occurs.

To simplify the procedure, one approach is to request that the source cell and candidate cell assign the same Source/Target NG-RAN node UE XnAP ID reference pair for each LTM request

Proposal 1: In LTM, when parallel requests are made, the source and the candidate cell shall always assign the same Source NG-RAN node UE XnAP ID reference and Target NG-RAN node UE XnAP ID reference for the same IE.

Proposal 2: If it assume source and candidate shall always assign the same Source/target NG-RAN node UE XnAP ID reference, error handling can follow the legacy mechanism.

2.2 UE Association in-between candidate CUs

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

When considering subsequent LTM procedures, the UE association should also be taken into account. For example, if gNB1 is the source gNB with cell1 as the serving cell, and candidate cell2 is allocated in gNB2 while candidate cell3 is allocated in gNB3, the situation can evolve as follows: After the UE switches to cell1, gNB2 will become the new source gNB. To support the UE's further handover to other candidate cells, an association between gNB2 and gNB3 needs to be established before the subsequent LTM occurs.

For when and how to establish the association among candidate gNBs, there are three options on table.

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send to other candidate gNBs (e.g. via initial source gNB).

- Option 2: After each cell switch, the new serving gNB updates the Source node UE XnAP ID and send it to other candidate gNBs, and then the candidate gNB-CUs reply with the Target node UE XnAP ID.

- Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB.

For Option 1, it represents the most efficient approach for subsequent LTM procedures, but it significantly increases the load on the initial candidate cell preparation. For Option 3, it requires the new source gNB (gNB2) to reuse the UE XnAP ID pairs assigned by the previous source gNB (gNB3), which may lead to potential collisions of the Source Node UE XnAP ID.

We prefer to use the option 2. When UE execute cell switch form source gNB(gNB1)and access to the new source gNB(gNB2), the gNB2 establish UE association with other candidate cell. This procedure can encompass several functions, including at least:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

Proposal 3: After each cell switch, the new serving gNB and candidate gNBs establish the UE association. i.e., option 2.

Proposal 4: Along with establishing the UE association, This procedure can encompass several functions:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

Another issue pertains to UE context association. Suppose UE 1 has switched to gNB2, and gNB2 sends a request to gNB3 to establish a UE context. Typically, considering privacy, the interface message includes only the Source XnAP ID IE assigned by the source, rather than the UE’s permanent ID. Based on this, when gNB3 receives the request from gNB2, gNB3 can o nly recognize that the request is for resource allocation for a UE marked by Source XnAP ID X, but it cannot identify that the request is for UE 1, for which resources have already been reserved in a previous request (from gNB1 to gNB3)

There are three options on the table for realizing UE context association when gNB2 establishes a UE association with gNB3.

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- Option 3: gNB2 shall include the assigned target XnAP ID, which is assigned by gNB3 in initial LTM candidate cell request procedure ( gNB1 send HO request to gNB3 for request candidate cell3).

Proposal 5: Discuss how to realize the UE context association in-between candidate gNB. Potential solution includes: When the gNB2 establish UE association with gNB3

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- Option 3: gNB2 shall include the assigned target XnAP ID, which is assigned by gNB3 in initial LTM candidate cell request procedure ( gNB1 send HO request to gNB3 for request candidate cell 3).

2.3 Early sync

For the early sync information request, for avoiding early sync confliction, candidate DU shall provide RACH configuration in per source DU in Rel-18. Considering the same logic, in inter-CU case, besides DU ID, CU ID should also be transferred in early sync information request. However, as a gNB can not explore its own architecture to neighbour gNB, it is reasonable to let candidate gNB provides the early sync resource per source gNB, and source gNB-CU further split the resource into per-gNB-DU granularity, if needed.

Proposal 6: Candidate gNB provides RACH configuration per source gNB per candidate cell, and the source gNB-CU further splits the resource into per-gNB-DU granularity, if needed.

  - Semi-persistent CSI-RS resource

In the LS, RAN1 confirmed the periodic CSI-RS is supported, and asking RAN3 for the feasibility of supporting semi-persistent CSI-RS, including specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

- Periodic CSI-RS: CSI-RS is activated after configured, i.e., UE can measure the CSI-RS.

- Semi-persistent CSI-RS: after configured, Semi-persistent CSI-RS need an additional MAC CE for activation, and then UE can measure the semi-persistent CSI-RS.

From the perspective of configuration, there is no different between supporting Periodic CSI-RS and Semi-persistent CSI-RS. The source shall request candidate to provide CSI-RS information by UE association message, and candidate cells provided CSI-RS configuration to source gNB. After collected from all accepted candidate cells, source generating the common CSI-RESOURCE to each candidate cell or generate CSI-REPORT. After the interaction, source CU can configure the entire candidate configuration to the UE.

Proposal 7: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and the same procedure shall be used.

The key issue to support Semi-persistent CSI-RS is that the source node and the target node shall inform each other when the candidate cell active the semi-persistent CSI-RS. There are two possible options from RAN3 perspective to supporting Semi-persistent CSI-RS:

- Option 1: Source node trigger:

After configuration the RS to UE, source node decides to trigger the Semi-persistent CSI-RS of candidate cell, and sending XnAP message to candidate node for request candidate node active Semi-persistent CSI-RS.

- Option 2:Candidate node trigger:

After configuration the RS to UE, candidate node decides to trigger the Semi-persistent CSI-RS, and sending XnAP message to source node for informing source node the Semi-persistent CSI-RS is active.

From our perspective, the option 1 is more reasonable, as source node has the fully knowledge of measurement results and UE location information. When the source node aware UE is near to the candidate cell, it will consider triggering the candidate cell active its Semi-persistent CSI-RS. Accordingly, the source node also needs to indicate UE measurement for activated CSI-RS, which should be discussed in RAN2.

Proposal 8: RAN3 to discuss the activation/deactivation of Semi-persistent CSI-RS, slightly preferring the Source node to trigger and send request message to candidate node.

Proposal 9: Introduce a new Xn message form source node to candidate node for Semi-persistent CSI-RS activation/deactivation.

As CSI-RS is a physical layer signaling, when the source node uses a split architecture, it is reasonable to enable the source DU (Distributed Unit) to trigger the activation/deactivation of Semi-persistent CSI-RS. This approach aligns with the functional split options in 5G NR, where the DU can handle certain physical layer functions, including the activation of CSI-RS resources.

Proposal 10: For split architecture, source DU trigger activation/deactivation of Semi-persistent CSI-RS.

Proposal 11: Introduce a new F1 message from source DU to source CU, and candidate CU to candidate DU for Semi-persistent CSI-RS activation/deactivation.

2.5 Set ID

Rel-19 Set ID is introduced for inter-CU LTM, and UE decided whether need to execute PDCP re-establish based on the Rel-19 Set ID, e.g., when the set id of source cell and target cell is different, UE shall execute PDCP re-establish. RAN3 need enhancement for supporting Rel-19 Set ID transfer,

In Rel-19, the Rel-19 Set ID is introduced for inter-CU LTM. The UE decides whether to execute PDCP re-establishment based on the Rel-19 Set ID. For example, if the Rel-19 Set ID of the source cell and the target cell is different, the UE shall execute PDCP re-establishment. To support this functionality, RAN3 needs enhancements for transferring the Rel-19 Set ID.

- Issue 1: Set ID Assignment.

- Source gNB Assigns Rel-19 Set ID Independently:

The source gNB can assign the Rel-19 Set ID by itself. However, since the source gNB does not know the internal implementation of the candidate gNB, it can only assign the same Rel-19 Set ID for all cells within the candidate gNB.

- Candidate gNB Provides Assistance Information:

The candidate gNB can provide assistance information, and the source gNB can make the final decision to ensure the global uniqueness of the Rel-19 Set ID. This approach allows the candidate gNB to assign different Set IDs for each candidate cell group, providing more flexibility for network operations

The assistance information can be provided by the following format

:

Example 1: Candidate gNB A assigns sub-Set IDs for its candidate cells: Cell a1: sub-Set ID 1 Cell a2: sub-Set ID 2 Candidate gNB B assigns sub-Set IDs for its candidate cells: Cell b1: sub-Set ID 1 Cell b2: sub-Set ID 1 The candidate gNBs send this assistance information to the source gNB in the following format: {gNB A: cell a1, 1; cell a2, 2} {gNB B: cell b1, 1; cell b2, 1} Example 2: Candidate gNB marks candidate cells which shall assigned same Rel-19 Set ID in the same cell group: {gNB A: cell group 1 (cell a1); cell group 2 (cell a2) } {gNB B: cell group 1 (cell b1, cell b2) } Based on above assistance information, the source gNB can then re-assign Set IDs for all candidate cells to ensure global uniqueness: {cell a1, 1; cell a2, 2; cell b1, 3; cell b2, 3}

Proposal 12: Discuss in RAN3 how to assign the Rel-19 Set ID, i.e., Whether source gNB Assigns Rel-19 Set ID Independently, or source gNB Assigns Rel-19 Set ID based on Candidate gNB Provides Assistance Information

- Issue 2: interference signalling

- If source gNB assigns the Rel-19 Set ID independently, the Set ID will only be used by the UE, eliminating the need for interaction between candidate gNBs. From an interface perspective, no enhancements are required.

- If the source gNB assigns the Rel-19 Set ID based on assistance information, the candidate cells need to provide this assistance information to the source gNB. After the source gNB completes the final assignment, it should send the Rel-19 Set ID list to all candidate gNBs for candidate cells maintenance, e.g., adding new candidate cells.

Based on the above analyses, to reduce the interaction between the source gNB and candidate gNB, we believe it is preferable for the source gNB to make the decision independently.

Proposal 13: It is proposed that the source gNB to assign the Rel-19 Set ID independently.

2.6 TA validity

TA validity is discussed in Rel18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

In Rel-18, though not explicitly specified, the fundamental approach to maintaining TA validity is to initiate a timer at the source side. The range of this timer is typically on the same order of magnitude as TAT (Timing Advance Timer) because they serve the same purpose. As defined in TS 38.331, TAT is specified as ENUMERATED, with the minimum value being 500ms. Generally, the latency between Xn interfaces is considered to be around 10ms. It is evident that the latency of the Xn interface is significantly less than the TAT, i.e., the TA validity time. Therefore, it is still feasible to determine the validity of TA at the source DU and leave it to implementation as per Rel-18.

Proposal 14: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validity.

To avoid unnecessary early synchronization, in Rel-18, we enable the source to transfer the obtained TA (Timing Advance) values in the Cell Switch Notification message. In the intra-DUscenario, the movement range of the UE is limited, so the TA remains valid in the target cell with a high probability. However, extending this to the inter-CU case, where the UE's movement range is larger, it becomes difficult to assert that the TA obtained at the source is still valid at the target side, regardless of whether it is at the source or target. Using an unsuitable TA may lead to handover failure, and the drawbacks of transferring TA among different CUs outweigh the potential advantages.

Proposal 15: Due to the fact that TA is relative to UE location, transferring TA values among different gNB is not support in inter-CU LTM.

2.7 Synchronization among candidate cells

To support subsequent inter-CU LTM, at least the following information should be synchronized among candidate cells.

- TCI state information list

- Configuration ID list

- Early sync information

- The data forwarding information

This procedure can be accomplished either through a second round of interaction between the source CU and candidate CUs, or after the UE has switched to the new DU. In any case, the synchronization of this information should be completed no later than when the UE accesses the new serving cell.

Proposal 16: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

2.8 Reference configuration

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

In the last meeting, we concluded that in RAN3, we should adopt a similar approach to intra-CU LTM, meaning we will only support a single reference configuration. The next issue is which node should determine the reference configuration.

To provide more flexibility, we suggest allowing both the source gNB and the candidate gNB to provide the reference configuration. The reference-related procedure can be initiated by the source gNB, which can either provide the reference configuration to the candidate gNB or request the candidate gNB to provide reference configuration. The Handover request procedure can be reused for this reference configuration interaction.

Proposal 17: The reference-related procedure can be initiated by the source gNB, which can either provide the reference configuration to the candidate gNB or request the candidate gNB to provide reference configuration.

Proposal 18: Reuse HO request message for reference configuration interaction.

2.9 Path switch

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC. And usually, it should be finished as soon as UE access to the target gNB. In LTM, when UE access to the candidate DU, the candidate DU shall send a access success message to the CU among F1AP, which means UE had access to the target gNB. It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 19: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: In LTM, when parallel requests are made, the source and the candidate cell shall always assign the same Source NG-RAN node UE XnAP ID reference and Target NG-RAN node UE XnAP ID reference for the same IE.

Proposal 2: If it assume source and candidate shall always assign the same Source/target NG-RAN node UE XnAP ID reference, error handling can follow the legacy mechanism.

Proposal 3: After each cell switch, the new serving gNB and candidate gNBs establish the UE association. i.e., option 2.

Proposal 4: Along with establishing the UE association, This procedure can encompass several functions:

- Update the KgNB no matter which solution is adopted by SA3;

- Inform other candidate cells which cell is the current serving cell;

Proposal 5: Discuss how to realize the UE context association in-between candidate gNB. Potential solution includes: When the gNB2 establish UE association with gNB3

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- Option 3: gNB2 shall include the assigned target XnAP ID, which is assigned by gNB3 in initial LTM candidate cell request procedure ( gNB1 send HO request to gNB3 for request candidate cell 3).

Proposal 6: Candidate gNB provides RACH configuration per source gNB per candidate cell, and the source gNB-CU further splits the resource into per-gNB-DU granularity, if needed.

Proposal 7: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and the same procedure shall be used.

Proposal 8: RAN3 to discuss the activation/deactivation of Semi-persistent CSI-RS, slightly preferring the Source node to trigger and send request message to candidate node.

Proposal 9: Introduce a new Xn message form source node to candidate node for Semi-persistent CSI-RS activation/deactivation.

Proposal 10: For split architecture, source DU trigger activation/deactivation of Semi-persistent CSI-RS.

Proposal 11: Introduce a new F1 message from source DU to source CU, and candidate CU to candidate DU for Semi-persistent CSI-RS activation/deactivation.

Proposal 12: Discuss in RAN3 how to assign the Rel-19 Set ID, i.e., Whether source gNB Assigns Rel-19 Set ID Independently, or source gNB Assigns Rel-19 Set ID based on Candidate gNB Provides Assistance Information

Proposal 13: It is proposed that the source gNB to assign the Rel-19 Set ID independently.

Proposal 14: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validity.

Proposal 15: Due to the fact that TA is relative to UE location, transferring TA values among different gNB is not support in inter-CU LTM.

Proposal 16: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

Proposal 17: The reference-related procedure can be initiated by the source gNB, which can either provide the reference configuration to the candidate gNB or request the candidate gNB to provide reference configuration.

Proposal 18: Reuse HO request message for reference configuration interaction.

Proposal 19: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

4. Reference

[1]	R3-244272 Discussion for Inter-CU LTM

[2]	R3-247389 Discussion on support of semi-persistent CSI-RS resource (CATT)

[3]	R3-247006 LS on the support of semi-persistent CSI-RS resource for LTM candidate cells