3GPP TSG-RAN WG3 Meeting #127		R3-250419

Athens, Greece, 17th – 21st, Feb, 2025

Agenda Item:	11.4

Source:	CATT

Title:	Down selection for continuous MDT

Document for:	Decision

- Introduction

In last meeting, the issue list in SI further expounded to FFSs shown as below.

Further discussions are needed on how to define a temporary identifier for Continuous MDT

Further discussions are needed on the solution for measurement correlation in Continuous MDT

In this paper, we main focus on the correlation issues.

- Discussion

As list by moderator during last meeting, there are mainly four solutions for continuous MDT. We give analyse for the potential solutions.

- Solution 1: Based on RAN-OAM interaction (special TR/TRSR for continuous MDT).

- Solution 2: Based on OAM-RAN-AMF interaction (AMF store MDT context, and provide new defined identifier).

- Solution 3: Based on OAM-RAN-AMF interaction (new Job type).

- Solution 4: Based on RAN-OAM interaction (report assigned TR/TRSR to OAM).

There provide the Analyse of four candidate solutions.

| solution | Potential impact on UE | Potential impact on OAM | Potential impact on CN | Impact on RAN |
| --- | --- | --- | --- | --- |
| Solution#1 | Measurement continuity: in legacy logged MDT, when the UE turned to RRC connected, it shall report an available indicator to the RAN node. If solution#1 is adopted, UE needs to report TR/TRSR of logged MDT to RAN node once it turned to RRC connected. | Measurement continuity: SA5 need to define special TR/TRSR for continuous MDT. Trace Correlation: SA5 need to further study. | . N/A | N/A |
| Solution#2 | N/A | N/A | Measurement continuity: For logged MDT, AMF need to store continuous MDT related context. And send it back to gNB when UE back to connected. Trace Correlation: CN need to define an identifier for distinguish MDT report in different RRC states, | Trace Correlation: NG-RAN node need to contain the new defined identifier in MDT report |
| Solution#3 | Measurement continuity: UE need to understand new Job Type. | Measurement continuity: SA5 need to change the deification of Job Type, e,g., one Job Type can reference to two type of MDT. | N/A | N/A |
| Solution#4 | N/A | N/A | Measurement continuity: For logged MDT, AMF need to store continuous MDT related context. And send it back to gNB when UE back to connected. | Trace Correlation: NG-RAN node need to contain the assigned TR/TRSR in MDT report, |

Proposal: Discussion and down selection the candidate solutions in RAN3.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal: Discussion and down selection the candidate solutions in RAN3.

4. Reference

[1] TR38.743

Appendix A: Detail of candidate solutions.

- Solution 1: Based on RAN-OAM interaction.

  - The OAM configures Management Based MDT to the gNBs taking part in Continuous MDT. The configuration includes specific identifiers (e.g. Trace Record) dedicated to the Continuous MDT process, namely the NG-RAN deduces from such identifier that the Management Based configuration is for Continuous MDT.  The RAN configures parameters such as the Trace Record Session Reference, which are reassigned to the UE (in full or in part) across RRC States and RRC_Connected mobility, namely such parameters are assigned/reassigned to the UE throughput the duration of the Continuous MDT session.

  - Measurement continuity is ensured by

    - Including in the Handover signalling an indication that the UE has to be selected for continuous MDT, e.g. the TR/TRSR assigned at source to the Continuous MDT process for the UE

    - Checking the TR/TRSR in the Logged MDT reports signalled by the UE at RRC_Idel/Inactive to RRC_Connected transitions. TR/TRSR reveal that the UE should be selected for Continuous MDT

  - Trace Correlation is achieved by:

    - Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace.

- Solution 2: Based on OAM-RAN-AMF interaction.

  - The OAM configures Management Based MDT at a gNB that selects the UE for Continuous MDT. When UE transits to RRC_Idle, a context related to the MDT configuration is stored by the gNB serving the UE to the Core Network. Upon transition from RRC_Idle to RRC_Connected or after successful handover, the AMF indicates to the second gNB serving the UE that the MDT configuration used for the UE, including an indication that the UE should be selected for management-based immediate MDT. In case of handover, the source gNB sends the MDT context related to the data collection to the target gNB during handover preparation.

  - Measurement continuity is ensured by

    - Receiving at the handover target gNB the MDT configuration previously used for Continuous MDT for the UE (signalled from the source gNB to the target gNB)

    - Receiving at the new serving gNB the MDT configuration previously used for Continuous MDT for the UE (signalled from the AMF to the serving gNB)

  - Trace Correlation is achieved by:

    - Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the Core Network and maintained in the Core Network for the duration of the UE context.

    - The AMF assigned identifier is signalled to the serving gNB as part of the MDT configuration used for continuous MDT. The serving gNB includes such identifier in the Trace Records signalled to the TCE.

- -	Solution 3: Based on OAM-RAN-AMF interaction.

  - Introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE. Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE.
The gNB serving a Continuous MDT UE sends parts of the MDT configuration to the AMF, which stores such information. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration

  - Measurement continuity is ensured by

    - The serving gNB receiving from the AMF information concerning the Continuous MDT configuration for the UE (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR allocated by the old serving gNB) which were previously stored in the AMF/5GC by the old serving gNB, so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

    - Source gNB provides the target gNB with information concerning the Continuous MDT configuration (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB). The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

  - Trace Correlation is achieved by:

    - “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE

- Solution 4: Based on RAN-OAM interaction (Trace Correlation without RAN3 Impact).

Measurement continuity is ensured by similar way proposed by solution 2, e.g., OAM/AMF assign an identifier and stored in CN.

For MDT report:

Immediate MDT report: gNB sends the immediate MDT Records to TCE, including the TR1, TRSR1 of immediate MDT(legacy behavior), and the assigned TR2,TRSR2 for logged MDT when UE goes to idle.

Logged MDT report: gNB sends the received MDT Record to the TCE, including the TR2, TRSR2 of the logged MDT(legacy behavior), and the assigned TR1,TRSR3 for immediate MDT UE will execute during connected state.

  - Measurement continuity is ensured by

    - Similar with solution 2.

  - Trace Correlation is achieved by:

    - Correlation two MDT Record by the “assigned TRx,TRSRx”  in this MDT report.

Appendix B: Example of solutions#4.

- UE across RRC states

In the Fig.1, we illustrate an example of MDT measurement correlation when UE experience RRC_connected, RRC_idle and back to RRC_connected.

Fig.1 UE across different RRC states

Step1-2: The OAM decides to configure Management Based MDT for continuous MDT towards the NG-RAN. And configure immediate MDT configuration (TR1) and logged MDT configuration (TR2) to the NG-RAN.

Step3-5: NG-RAN selects suitable UE to execute MDT measurement. NG-RAN sends immediate MDT configuration and logged MDT configuration to the UE, separately.

Step 6-7: UE sends the MDT report of immediate MDT to the gNB, and gNB save MDT measurement in the MDT Record.

Step 8: gNB shall send received MDT Records to TCE, including the TR1, TRSR1 (current MDT Record, legacy). In addition, for MDT report correlation, it shall also include the TR, TRSR for the subsequent MDT assigned by the gNB, it indicate the TR, TRSR will be used when UE in RRC_idle/inactive, e.g., TR2, TRSR2.

Step 9-10: UE transits to RRC_idle, and execute logged MDT based on the previous configuration received at step 5.

Step 11-12: UE backs to RRC_connected, and based on network request, UE sends logged MDT report to the gNB.

Step 13: UE sends the measurement report of logged MDT to the gNB, and gNB save UE measurement in the MDT Record, and sending this MDT Record to TCE. In the MDT Record head, it shall include the TR2, TRSR2 (current MDT Record, legacy). In addition, for MDT report correlation, it shall also include the TR, TRSR for the subsequent MDT assigned by the gNB, it indicate the TR,TRSR will be used when UE in RRC_connected, e.g., TR1, TRSR3.

Step 14-16: when UE transits to RRC_connected, gNB shall re-configure the Management based MDT configuration the UE, at least the immediate MDT configuration, as soon as possible. The UE shall execute immediate MDT after configured.

Step 17: Similar with step8, when send the MDT report, the gNB shall also include assigned TR, TRSR for the subsequent MDT.

Based on the above, OAM can correlate the MDT report received at step 8, 13 and 17 by the current TR, TRSR (Trace ID) and subsequent TR, TRSR (Trace ID).

- UE across RRC states in different gNB

In the fig.2, we illustrate an example of MDT measuremennt correlation when UE experience RRC_connected, RRC_idle and back to RRC_connected at the different gNB.

Fig.2 UE across different RRC states in different gNB

Step 1-10: Similar with step 1-8 in figure 1, OAM configure both immediate MDT and logged MDT configuration to the NG-RAN, and gNB choose suitable UE and sending the configuration. UE send the measurement report of immediate MDT to the gNB, and gNB save UE measurement in the MDT Record. gNB send the MDT record to TCE, including the TR1, TRSR1 for the current MDT Record and the assign TR, TRSR for the subsequent MDT.

Step 11-13: UE transits to RRC_idle in gNB1, and execute logged MDT based on the previous configuration received at step 5. UE transits to RRC_connected in gNB2, and based on network request, UE sends logged MDT report to the gNB2.

Step 14-15: UE sends the measurement report of logged MDT to the gNB2, and gNB2 save UE measurement in the MDT Record, and sending this MDT Record to TCE. including the TR2, TRSR2 (current MDT Record, legacy) and the assigned TR, TRSR for the subsequent MDT by gNB2.