3GPP TSG-RAN WG3 Meeting #127		R3-250421

Athens, GR, 17th – 21st, Feb, 2025

Agenda Item:	15.2

Source:	CATT

Title:	Discussion about IoT NTN for regeneration payload

Document for:	Decision

- Introduction

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

In this paper, we provide our initial discussion about S&F scenario, and start with the attachment procedure.

- When the feeder link is not available, satellite shall work on S&F mode, UE can send the user data to the satellite when the service link available and stored in MME-onboard.

- When the feeder link is available, for previous stored user data during S&F operation mode, the MME-onboard shall transfer the user data to the MME-ground by the Feeder link 1, which is a link between MME-onborad and MME-ground, and defined by SA2.

- When the feeder link is available, for normal UE, satellite can provide service by the regenerative payload on board, and transfer the user data to the on ground CN by Feeder link 2, which is S1 interface and shall be enhance in RAN3.

- Discussion

  - S1 connection management

In NR NTN discussion, RAN3 has concluded to introduce NG Removal/Setup procedure for Regenerative Payload NG interface management. And for whether introduce suspend/resume procedure still FFS. For the IoT NTN scenario, we suggest to follow NR NTN agreements to introduce S1 Remove procedure for S1 interface management. And for suspend/resume procedure, we understand there is no big issue to support it, the final decision can follow the agreements in NR.

Proposal 1: introduce S1 remove procedure for regenerative payload S1 interface management.

  - Feeder link switch

Similar, in IoT NTN scerino, when considering the feeder link switch, it is assumed that the MME remains unchanged. And RAN3 has agreed that the current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. However, S1AP is currently designed to support only a single TNLA association. There are two possible approaches concerning the IoT NTN feeder link switch:.

- Option 1: Enhance S1 interface to support multiple TNL association, and support both soft feeder link switch and hard feeder link switch.

- Option 2: Only support hard feeder link switch in IoT NTN.

As we understand it, IoT NTN is primarily designed for mMTC applications, which typically involve UEs that can tolerate higher delays. Compared to a soft feeder link switch, a hard feeder link switch introduces an additional interruption when re-establishing the TNLA. However, this interruption is acceptable for IoT UEs given their higher delay tolerance..

Proposal 2: Considering IoT NTN mainly designed for UE with high delay tolerance, only support hard feeder link switch in IoT NTN is sufficient.

For the control plane, similar to NR NTN, all the related network function can know the feeder link available status based on OAM configuration. The activation and disconnection of the old and new TNLA can be based on the status of the feeder link. For example, MME, eNB and the Gateways all obtain when the old TNLA unavailable and when the new Gateway finish the establishment.

For the user plane, after new feeder link is connected, eNB could notify the MME about the new DL S1-U TNL Information and SMF could obtain the changed TNL address, the potential message could be RAN Configuration Update Procedure and/or Path Switch Procedure, if UP address change is needed.

Proposal 3: Reuse RAN Configuration Update Procedure and/or Path Switch Procedure for supporting UP address modification.

  - Location based HO over X2AP/S1AP

R18 has already enhanced the X2 interface to support Time based CHO. Handover with Time-based trigger condition and Location-based trigger condition are specified in Stage-2 TS 36.300. In the NR NTN discussion, some companies proposed to enhance location based HO. For inter-eNB mobility, enhancement shall also support in the similar manner. It is proposed that RAN3 considering whether and how to better support location based HO via the X2 and S1 interface. For example, source eNB can transfer the coarse UE location information or the condition of location based CHO to target eNB in the Handover Request. Thus the target eNB can decide when to reserve resources based on this information.

Proposal 4: For location based CHO, it is proposed to transfer some information from the source eNB towards the target eNB, e.g. the coarse location of the UE, for better resource handling.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: introduce S1 remove procedure for regenerative payload S1 interface management.

Proposal 2: Considering IoT NTN mainly designed for UE with high delay tolerance, only support hard feeder link switch in IoT NTN is sufficient.

Proposal 3: Reuse RAN Configuration Update Procedure and/or Path Switch Procedure for supporting UP address modification.

Proposal 4: For location based CHO, it is proposed to transfer some information from the source eNB towards the target eNB, e.g. the coarse location of the UE, for better resource handling.

4. Reference

[1]	TR 23.700 Study on integration of satellite components in the 5G architecture.

[2]	S2-2410918 Reply LS on Support of Regenerative-based Satellite Access