3GPP TSG-RAN WG3 Meeting #127	R3-250423

Athens, Greece, 17th – 21th Feb 2025

Title: 	(TP to TS36.413) Introduce S1 Removal for IoT NTN

Source: 	CATT, ZTE, CMCC, SAMSUNG

Agenda item: 	15.2

Document Type:	Discussion

- Introduction

This is the TP for S1 Removal to support Regenerative Payload in IoT NTN according to R3-250422.

2	TP to TS36.413

-----------------Start of Change-------------------

### 8.7.x	S1 Removal

#### 8.7.x.1	General

The purpose of the S1 Removal procedure is to remove the interface instance between the E-UTRAN node and the MME in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: S1 Removal, successful operation

The eNB initiates the procedure by sending the S1 REMOVAL REQUEST message to the MME. Upon reception of the S1 REMOVAL REQUEST message, the MME shall reply with the S1 REMOVAL RESPONSE message. After receiving the S1 REMOVAL RESPONSE message, the eNB shall initiate removal of the TNL association towards the MME and may remove all resources associated with that interface instance. The MME may then remove all resources associated with the eNB.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.x.3-1: S1 Removal, unsuccessful operation

If the MME cannot accept to remove the interface instance with eNB, it shall respond with an S1 REMOVAL FAILURE message with an appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

-----------------Next Change-------------------

#### 9.1.2.x1	S1 REMOVAL REQUEST

This message is sent by the eNB node to initiate the removal of the interface instance.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |

#### 9.1.8.x2	S1 REMOVAL RESPONSE

This message is sent by the MME to acknowledge the initiation of removal of the interface instance.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | M |  | PrintableString(SIZE(1..150,…)) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.x3	S1 REMOVAL FAILURE

This message is sent by the MME to indicate that removing the interface instance cannot be accepted.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

-----------------End of Change-------------------

5. Reference

[1] 	RP -240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	R3-250421	Discussion about IoT NTN for regeneration payload