3GPP TSG-RAN WG3 Meeting #127	R3-250434

Athens, Greece, 17 – 21 February 2025

Agenda item:	12.3

Source:	Nokia

Title:	Completion of Functional Aspects of NR Femto

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

At the last RAN3#126, baseline CRs were agreed for TS 38.300 and TS 38.413 which are still incomplete.

This paper investigates remaining open points and proposes some Text Proposals to fill the gap.

# 2	Discussion

- Completion of stage 2

The following conclusion was reached in the TR 38.799 for the NG interface:

### 6.2.1	Architecture for NG interface

Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible.

In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase.

It is clear from the above text that the agreement to have an optional NR Femto Gateway in the architecture was taken in consideration of smooth integration for an operator who has deployed HeNBs using HeNB GW. Therefore, it is assumed that the protocol and functional aspects of the NR Femto GW to be specified should offer an equivalent level of functionalities to the operators as the one of the HeNB Gateway architecture in 4g network.

At the last RAN3#126 meeting, the specific functional aspects related to NR Femtos were captured in the baseline CR for TS 38.300 along these lines.

At this meeting, the specification of the specific functional aspects of the NR Femto Gateway are proposed to be added to the 38.300 BL CR in [2] with a similar spirit.

Proposal 1: agree the TP in [2] to introduce functional aspects of NR Femto Gateway in the BL CR 38.300.

Similarly, some functional characteristics of the AMF need to be specified inn TS 38.300 for operation of NR Femtos which are proposed in [3].

Proposal 2: agree the TP in [3] to introduce functional aspects of the AMF in the BL CR 38.300.

- Completion of stage 3

Significant progress was achieved at RAN3#126 on the BL CR 38.413.

However, some specific aspects of the initial access control in the AMF are missing in the specification.

For the initial access control in AMF, the AMF needs to know if the accessed cell is shared CAG- PLMN cell or a CAG-only cell. This is because:

- If the accessed cell is CAG-only cell, initial access control in the AMF should be successful if the Allowed CAG List of the UE matches with the Supported CAG List received from the NR femto node in the Initial UE Message;

- If the accessed cell is a shared CAG-PLMN cell, then initial access control in the AMF should be successful if either:

  - The UE is not a CAG-only UE, or,

  - The UE is CAG-only UE and the Allowed CAG List of the UE matches with the Supported CAG List received from the NR femto node in the Initial UE Message.

As can be seen from above, the checks to be made in the AMF at initial access control depends on whether the accessed cell is shared with PLMN or not and the logic to acheive successful outcome of the access control therefore depends on the same.

It is therefore needed for AMF to learn whether the UE accesses from a CAG-only cell or a shared cell.

Proposal 3: agree the TP in [4] to add an indication of CAG-only cell or shared cell to the BL CR 38.413.

- Remaining open points

The functional Part of the HeNB subsystem in TS 36.300 section 4.6 reveals several security verification checks for HeNBs performed either by the HeNB GW or by the MME to validate the HeNB.

It FFS pending SA3 study whether the same verification checks would apply to the 5G NR Femto architecture agreed by RAN3. It would therefore be beneficial to emphasize explicitly to SA3 those security points which needs checking for parity with 4g so that SA3 can give a quick feedback.

Proposal 4: send the LS to SA3 proposed in Annex A of this paper to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

# 3	Conclusion and Proposals

This paper has evaluated the objectives and corresponding work to complete the NR Femto work item agreed in [1] and made the following proposals:

To progress the stage 2 and stage 3:

Proposal 1: agree the TP in [2] to introduce functional aspects of NR Femto Gateway in the BL CR 38.300.

Proposal 2: agree the TP in [3] to introduce functional aspects of the AMF in the BL CR 38.300.

Proposal 3: agree the TP in [4] to add an indication of CAG-only cell or shared cell to the BL CR 38.413.

To address remaining open points:

Proposal 4: send the LS to SA3 proposed in Annex A of this paper to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-250436, [TP for BL CR NR Femto 38.300] Functional aspects of NR Femto Gateway

- R3-250437, [TP for BL CR NR Femto 38.300] Functional aspects of AMF

- R3-250435, [TP for BL CR NR Femto 38.413] Completion of NR Femto stage 3

# 5	Annex A: LS to SA3 on security aspects

3GPP TSG-RAN WG3#127			R3-25xxxx

Athens, 17 – 21 February 2025

Title:	LS on Security verifications related to NR Femtos

Response to:

Release:	Rel-19

Work Item:	FS_WAB_5GFemto_NR

Source:	RAN3

To:	SA3

Cc:

Contact Person:

#### Name:	Philippe Godin

Tel. Number:	+33 1 6040 0304

E-mail Address:	philippe.godin@nokia.com

Attachments:

1. Overall Description:

As previously indicated, RAN3 has agreed to have an NR Femto architecture in release 19 with an optional NR Femto GW for NG interface aligned with the 4g architecture deployment with HeNB GW.

RAN3 noticed that in 4g TS 36.300 specified multiple security verification checks to be performed by HeNB GW (respectively MME) to verify certain information sent by the HeNB. RAN3 would like SA3 to feedback on whether similar verification checks need to be specified by RAN3 in 5G to be performed by the NR Femto GW (respectively AMF) as follows:

For the NR Femto Gateway:

1:	In case of NGAP INITIAL UE MESSAGE message, whether to verify, for a closed NR Femto, that the indicated cell access mode and CAG ID are valid for that NR Femto is pending SA3 study.

2:	In case of NG SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid and determining whether the access mode of the NR Femto is closed or not is pending SA3 study.

3:	In case of NG PWS RESTART INDICATION message and PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid is pending SA3 study.

4:	If an NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

For the AMF:

1:	In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message and determining whether the access mode of the NR Femto is closed or not is pending SA3 study.

2:	In case of an NR Femto directly connected, whether to verify, for a closed NR Femto, that the indicated cell access mode and CAG ID are valid when receiving the NGAP INITIAL UE MESSAGE message is pending SA3 study.

3:	In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NG PWS FAILURE INDICATION message is pending SA3 study.

4:	If routing ambiguities are to be avoided, a TAI used in an NR Femto GW should not be reused in another NR Femto GW.

5:	The AMF or NR Femto GW should not include the list of CAG IDs for paging when sending the paging message directly to an un-trusted NR Femto or gNB (to be checked by SA3).

2. Actions:

To SA3 group:

ACTION: 	RAN3 kindly ask SA3 to indicate whether the verification checks listed above are applicable in 5G for the NR femto GW (respectively AMF).

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#127-bis	                    07 – 11 April			Wuhan, China

TSG-RAN WG3#128	                    19 – 23 May			Malta, MT