3GPP TSG-RAN WG3 Meeting #127	R3-250444

Athens, Greece, 17th – 21st February 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.300 and 38.401] Architecture for A-IoT RAN

Document for:	Discussions & Approval

# 1	Introduction

This document discusses stage-2 related specification consequences of the tasks given to RAN3 in the WID approved in RP-243326:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

(with the location reporting handled in a separate paper for Agenda Item 16.3)

# 2	Discussion

The agreement to that the interface between the A-IoT RAN and the A-IoT CN is NG, and respective communication shall take place by means of NGAP, moreover “assuming an architecture of aggregated gNB”, allows the re-use of the NG-RAN architecture specified in TS 38.300 – with relevant adaptations and additions.

Proposal 1:	Introduce in TS 38.300 a new chapter for “Support of Ambient IoT” and the statement that the NG-RAN overall architecture as specified in section 4 is applicable for Ambient IoT with adaptations and additions.

The “adaptations and additions”, i.e. the conditions under which the NG-RAN architecture as specified in TS 38.300 is applicable, have to be explicated, taking into account the 2 options agreed by SA2 on how to connect A-IoT RAN with the AIOTF, either via the AMF or directly.

Applicability of the NG-RAN architecture implies that the NG interface stays as an interface between NG-RAN and the AMF (UPF is out of scope in Rel-19, see the agreed WID), irrespective the 2 connectivity options agreed by SA2.

First, it has to be stated, that obviously, in order to support A-IoT, the gNB and the AMF have to be “A-IoT enabled”. In addition, it also needs to be explicated that the means via which an A-IoT enabled gNB and an A-IoT enabled AMF to communicate with each other are specified within NGAP.

Proposal 2:	Specify in TS 38.300 that the NG-RAN architecture is applicable under the assumption that the gNB and AMF are “A-IoT enabled” and communicate by means of NGAP.

With regards to the 2 connectivity options, direct and indirect via AMF

-	the indirect option resembles the way how NG-RAN communicates with the SMF via the AMF by means of NGAP defined “SMF transparent containers”. It is possible to apply the same protocol structure for the indirect communication between the A-IoT enabled gNB and the AIOTF via an A-IoT enabled AMF.

-	the direct connectivity option allows us to regard the AIOTF as a logical node where AMF and AIOTF functions are co-located, similar to a deployment option where an AMF and an SMF are co-located. The protocol functions necessary for an A-IoT enabled gNB to connect to an AIOTF directly (as if it would connect to an AMF) require the AIOTF and the A-IoT enabled gNB to apply basic (applicable) NGAP Interface management functions (NG Setup, RAN/AMF Configuration Update, Error Indication) in addition to A-IoT specific functions. We would not expect that the A-IoT enabled gNB would realise any difference with regards to the connectivity option.

Proposal 3:	Specify in TS 38.300 that the NG-RAN architecture is applicable under the assumption that in case of direct connectivity the A-IoT enabled gNB regards the AIOTF as an A-IOT enabled AMF.

In case of indirect communication, it should be possible to apply the same connectivity cardinality as for NG-RAN node/AMF/SMF cardinality, i.e. to assume that an AIOTF is reachable by an A-IoT enabled gNB via all A-IoT enabled AMF the gNB is connected to. Such approach would have implications on 38.401 as well.

Proposal 4:	Specify in TS 38.300 that the NG-RAN architecture is applicable under the assumption that in case of indirect connectivity the A-IoT enabled gNB communications with the AIOTF via one or several A-IoT enabled AMFs and
in TS 38.401 that each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Nevertheless, the cardinality aspect in case of indirect connectivity is proposed to be marked as “FFS” for now. It seems appropriate to inform SA2 on these assumptions.

Proposal 5:	Inform SA2 about the cardinality assumptions in proposal 4 (see draft LS out in R3-250446).

Further, it should be specified that in Rel-19 (“in this version of the specification”) A-IoT specific user data is transported via the C-plane only (i.e. NG-U is not applicable for A-IoT)

Proposal 6:	Specify in TS 38.300 that the NG-RAN architecture is applicable under the assumption that in Rel-19 A-IoT specific user data is transported between the A-IoT enabled NG-RAN and A-IoT enabled 5GC takes place via the NG-C interface only.

Last but not least on the architectural aspects, we have gained the common understanding that Xn is not affected, do to no support of mobility or other co-ordination between A-IoT enabled gNBs.

Proposal 7:	Specify in TS 38.300 that the NG-RAN architecture is applicable under the assumption that in Rel-19 no A-IoT specific communication takes place between gNBs.

Stage 2 also includes depiction of basic communication flows, in case of NG-RAN architecture, communication between the A-IoT enabled gNB and the A-IoT enabled AMF/AIOTF. TR 38.769 already contains an excellent basis, and respective content can be used for a first draft version of message flows to be incorporated in TS 38.300.

Proposal 8:	For A-IoT related message flow, use TR 38.769 as a basis for Inventory and Command message flows.

Final proposal: Agree on the TPs for TS 38.300 and 38.401 as shown below

# 3	Text Proposal

## 3.1	Text Proposal for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.x	Support of Ambient IoT

### 16.x.1	Introduction

Editor’s Note:	<introduction> section content to be added.

### 16.x.2	Architecture

The NG-RAN overall architecture as specified in section 4 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs and AMFs are enabled to support A-IoT.

-	An A-IoT enabled gNB communicates with an A-IoT enabled AMF by means of NGAP protocol functions.

-	In case of direct communication between an A-IoT enabled gNB and an AIOTF, as specified in <reference to be added>, the A-IoT enabled gNB regards the AIOTF as an A-IoT enabled AMF.

-	In case of indirect communication between an A-IoT enabled gNB and an AIOTF, as specified in <reference to be added>, the A-IoT enabled gNB communicates with the AIOTF via one or several A-IoT enabled AMFs.

Editor’s Note:	Whether indirect communication between an NG-RAN node and an AIOTF may take place via several AMFs needs to be further discussed.

-	In this version of the specification, A-IoT specific user data is transported between the A-IoT enabled NG-RAN and A-IoT enabled 5GC takes place via the NG-C interface only.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.x.3	Inventory procedure

Figure 16.x.3-1 depicts the basic communication between the A-IoT enabled gNB and the A-IoT enabled AMF/AIOTF for the Inventory procedure.

Editor’s Note 1:	Security related details need to be added based on SA3 progress.

Figure 16.x.3: Inventory procedure – basic NG-C signalling

1.	The A-IoT enabled AMF/AIOTF initiates the Inventory procedure by sending the Inventory Request message to the A-IoT enabled gNB. The Inventory Request may include information concerning one or several A-IoT devices.

Editor’s Note 2:	Specification of further parameterisation of the Inventory Request message (assistance information from 5GC) needs further work.

2.	The A-IoT enabled gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The A-IoT enabled gNB confirms the request from the A-IoT enabled AMF/AIOTF by replying with the Inventory Response message.

NOTE 1:	If the A-IoT enabled gNB is not able to perform the Inventory procedure, it rejects the request.

4.	The A-IoT enabled gNB performs the Inventory procedure towards the A-IoT device over the A-IoT radio interface.

NOTE 2:	If the Inventory procedure concerns multiple A-IoT devices, step 4 is performed towards multiple A-IoT devices.

5./6.	Upon receiving the inventory result from the A-IoT device, the A-IoT enabled gNB sends the Inventory Report message to the A-IoT enabled AMF/AIOTF. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT enabled AMF/AIOTF.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

### 16.x.4	Command procedure

Figure 16.x.3-1 depicts the basic communication between the A-IoT enabled gNB and the A-IoT enabled AMF/AIOTF for the Command procedure.

Editor’s Note 1:	Security related details need to be added based on SA3 progress.

Figure 16.x.4: Command procedure – basic NG-C signalling

1.	Prior to the Command procedure the Inventory procedure is performed for the concerned devices. The A-IoT enabled gNB and the A-IoT device is informed that a Command procedure follows the Inventory procedure.

Editor’s Note 2:	Further details on the relation between the Inventory procedure and the Command procedure may need input from SA2/SA3/RAN2.

2.	The A-IoT enabled AMF/AIOTF initiates the Command procedure by sending the Command Request message to the A-IoT enabled gNB. The Command Request may include information concerning one or several A-IoT devices.

3.	The A-IoT enabled gNB allocates and co-ordinates the usage of A-IoT radio resources.

4.	The A-IoT enabled gNB confirms the request from the A-IoT enabled AMF/AIOTF by replying with the Command Response message.

NOTE 1:	If the A-IoT enabled gNB is not able to perform the Command procedure, it rejects the request.

Editor’s Note 3:	Whether rejecting the Command procedure should be regarded as an abnormal case due to A-IoT enabled gNB’s commitment to an Inventory Request with a follow-on indication for Command may need further discussions.

5.	The A-IoT enabled gNB performs the Command procedure towards the A-IoT device over the A-IoT radio interface.

NOTE 2:	If the Command procedure concerns multiple A-IoT devices, step 4 is performed towards multiple A-IoT devices.

6./7.	Upon receiving the command result from the A-IoT device, the A-IoT enabled gNB sends the Command Report message to the A-IoT enabled AMF/AIOTF. If the Command procedure concerns multiple A-IoT devices, multiple Command Reports may be sent to the A-IoT enabled AMF/AIOTF.

NOTE 3:	If the Command procedure concerns multiple A-IoT devices, a Command Report may include reports from multiple A-IoT devices.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

## 3.1	Text Proposal for TS 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.1.1	Overall Architecture of NG-RAN

Figure 6.1-1: Overall architecture

The NG-RAN consists of a set of gNBs connected to the 5GC through the NG interface.

NOTE 1:	As specified in TS 38.300 [2], NG-RAN could also consists of a set of ng-eNBs, an ng-eNB may consist of an ng-eNB-CU and one or more ng-eNB-DU(s). An ng-eNB-CU and an ng-eNB-DU is connected via W1 interface. The general principle described in this clause also applies to ng-eNB and W1 interface, if not explicitly specified otherwise.

An gNB can support FDD mode, TDD mode or dual mode operation.

gNBs can be interconnected through the Xn interface.

A gNB may consist of a gNB-CU and one or more gNB-DU(s). A gNB-CU and a gNB-DU is connected via F1 interface.

One gNB-DU is connected to only one gNB-CU.

NOTE 2:	In case of network sharing with multiple cell ID broadcast, each Cell Identity associated with a subset of PLMNs corresponds to a gNB-DU and the gNB-CU it is connected to, i.e. the corresponding gNB-DUs share the same physical layer cell resources.

NOTE 3:	For resiliency, a gNB-DU may be connected to multiple gNB-CUs by appropriate implementation.

NG, Xn and F1 are logical interfaces.

For NG-RAN, the NG and Xn-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. For EN-DC, the S1-U and X2-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. The gNB-CU and connected gNB-DUs are only visible to other gNBs and the 5GC as a gNB. A possible deployment scenario is described in Annex A.

The node hosting user plane part of NR PDCP (e.g. gNB-CU, gNB-CU-UP, and for EN-DC, MeNB or SgNB depending on the bearer split) shall perform user inactivity monitoring and further informs its inactivity or (re)activation to the node having C-plane connection towards the core network (e.g. over E1, X2). The node hosting NR RLC (e.g. gNB-DU) may perform user inactivity monitoring and further inform its inactivity or (re)activation to the node hosting control plane, e.g. gNB-CU or gNB-CU-CP.

UL PDCP configuration (i.e. how the UE uses the UL at the assisting node) is indicated via X2-C (for EN-DC), Xn-C (for NG-RAN) and F1-C. Radio Link Outage/Resume for DL and/or UL is indicated via X2-U (for EN-DC), Xn-U (for NG-RAN) and F1-U.

The NG-RAN is layered into a Radio Network Layer (RNL) and a Transport Network Layer (TNL).

The NG-RAN architecture, i.e. the NG-RAN logical nodes and interfaces between them, is defined as part of the RNL.

For each NG-RAN interface (NG, Xn, F1) the related TNL protocol and the functionality are specified. The TNL provides services for user plane transport, signalling transport.

In NG-Flex configuration, each NG-RAN node is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node. The AMF Set and the AMF Region are defined in TS 23.501 [3]. Each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Editor’s Note:	This cardinality related aspect may need further discussions, for now it follows non-A-IoT architecture principles.

If security protection for control plane and user plane data on TNL of NG-RAN interfaces has to be supported, NDS/IP TS 33.501 [13] shall be applied.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>