3GPP TSG-RAN WG3 Meeting #127	R3-250446

# Athens, Greece, 17th – 21st February 2025

Title:	[DRAFT] Reply LS on A-IoT Conclusions in SA WG2

Response to:	LS (S2-2413035/R3-250025) on A-IoT Conclusions from SA2

Release:	REL-19

Work Item:	FS_AmbientIoT, FS_Ambient_IoT_solutions, Ambient_IoT_Solutions

Source:	(will be RAN3)

To:	SA2, RAN2

Cc:

Contact Person:

#### Name:

Tel. Number:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	<potentially agreed TPs>

# 1. Overall Description:

RAN3 thanks SA2 for the LS in S2-2413035/R3-250025 containing explicit indications to conclusions concerning RAN3.

RAN WGs have meanwhile started a Rel-19 Work Item and the progress in RAN3 concerns the following topics highlighted in S2-2413035:

- Following issues need to finalize in coordination with other WGs

- Issues in coordination with RAN WG:

- The layer to enable AIoT Reader control, whether using NGAP or a layer above NGAP.

RAN3 answer:

RAN3 decided to specify A-IoT transaction management protocol function within NGAP, in a analogous way as communication between an NG-RAN node and the SMF is realised through definition of NGAP containers transparent to the AMF.

- Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1

RAN3 answer:

Please refer also to the answer provided in response to S2-2501343 in R3-25xxxx.

Further RAN3 decided to utilise applicable NG interface management functions, namely the NG Setup procedure to establish an NG-C interface instance between an A-IoT enabled gNB and an A-IoT enabled AMF or AIOTF (dependent on the connectivity option). The NG Setup Request message from an NG-RAN node informs the AMF about the Tracking Areas configured in the cells served by the NG-RAN node. Likewise, in re-use of this function, it is possible to configure an A-IoT enabled gNB with a dedicated Tracking Area that could serve the purpose of selecting the appropriate A-IoT enabled gNB, assuming the AIOTF is configured with the association of that Tracking Area and the A-IoT Service Area introduced as described in the response to S2-2501343.

- How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF

RAN3 answer:

RAN3 assumes that in Rel-19 this aspect is not applicable, as a transaction is always triggered by the 5GC, so passing UL data towards the AIOTF should be possible.

- Following issues still need to be finalized by SA2

- AIoT reader selection principles.

RAN3 answer:

Please refer also to the answer provided in response to S2-2501343 in R3-250442.

- AIoT data aggregation principles.

RAN3 answer:

RAN3 assumes that the new A-IoT NGAP protocol functions should allow A-IoT enabled gNB to aggregate responses from various A-IoT devices, if addressed in the same A-IoT transaction.

Additional RAN3 information:

RAN3 would like to inform about further outcome of the RAN3#127 meeting:

-	RAN3 assumes that, in analogy of connection cardinality principles specified in TS 38.401 (i.e. each NG-RAN node is connected to all AMFs of AMF sets within an AMF Region supporting at least one slice also supported by the NG-RAN node), each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region

-	RAN3 assumes the necessity of an A-IoT transaction identification that enables A-IoT enabled gNBs and A-IoT devices distinguishing requests from the same and different AIOTFs as well as recognising repeated transaction requests from the same AIOTF.

# 2. Actions:

To SA2 and RAN2 group.

ACTION: 	RAN3 asks SA2 and RAN2 to take the above RAN3 agreements and assumptions into account and comment where applicable and necessary.

# 3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN WG3 Meeting#127bis	4th – 11th April 2025, China

TSG RAN WG3 Meeting#128	19th – 23rd May 2025, St.Julien’s, Malta