3GPP TSG-RAN WG3 Meeting #127	R3-250453

# Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on packet loss as UE performance metric

Source: 	Ericsson, JIO Platforms (JIO), Deutsche Telekom

Agenda item:	9.2

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss aspects related to the Packet Loss metric, which is part of UE performance measurements. As part of the RAN3 WI on AI/ML for NG-RAN in Release-18, UE performance was intended as a feedback mechanism for handover decisions derived by AI/ML algorithms. While the scope of discussions in RAN3 in Release-19, in this context, is to bring the Release-18 features to split architecture deployments, we show in this paper that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric and revisit the inclusion of such a metric in Release-19 normative work.

# 2. Discussion

Obtaining feedback to actions derived via AI/ML is an important step in bridging the gap between an AI/ML model and the real-world system. In the context of AI/ML based use-cases such as mobility optimization, RAN3 specified a mechanism to collect UE performance after an AI/ML derived mobility action. The UE performance metrics include Average UE throughput DL, Average UE throughput UL, Average Packet Delay, and Average Packet loss DL, which is collected by a target gNB after handover and is signalled back to the source gNB upon fulfilment of specified exit conditions.

The source gNB, after receiving the metrics included in the UE performance feedback, may take into account the performance at the target cell and use it to tune AI/ML derived mobility actions, e.g. prioritizing actions that maximise UE performance. Such process may lead to model re-training/updating. It is important to note that the evaluation based on UE performance should be carried out at service/application-level, as this determines the end-user perception of the service. While metrics collected from lower protocol layers may be used to interpret the service/application-level performance, such metrics not always reflect how a service/application is performing.

In the above context, there are important distinctions to note when discussing the relationship between packet loss measurement at a lower layer and the service or application-level performance.

Firstly, compared to other performance metrics, such as throughput and delay, the packet loss rate measured in the RAN is not necessarily reflected by the packet loss rate experienced at a higher layer. For example, at RLC level, PDUs can be dropped but they can be retransmitted for timely reception at TCP level, hence becoming available at application level.
Secondly, delay and throughput are metrics directly linked to service performance at the higher layers, i.e., the packet delay and throughput at application layer are directly connected with the measured delay and throughput. The same is not valid for packet loss at application layer. 
Based on the above, it may be argued that while delay and throughput are absolute measures of service level performance, packet loss is not. Hence, UE performance feedback may show a high packet loss rate, while the packet loss rate at application layer is perfectly good.

Furthermore, it is also the case that compared to throughput and delay, packet loss is a much more volatile metric whose resulting value is connected to the design of the system. For example, the choice of a modulation scheme is made in relation to a target BLER, and as a consequence a certain base packet loss rate is the consequence of a design choice of the BLER target. BLER targets are implementation specific and may change in relation to, e.g. the current SNR, the overall resource availability, the scheduling policy of a RAN node. Therefore, relative changes in the packet loss rate may not give information of whether the target cell is a good mobility target or not. The mobility target cell may be a perfectly good cell, but due to implementation choices the target RAN node may decide to e.g. apply an aggressive modulation and coding scheme to the UE and end up in higher packet loss rates, while the performance at the application layer is still good.

Now we discuss the specifics about how the packet loss metric may be collected.

TS 28.552 and TS28.558 define the following measurements for packet loss rate:

DL Packet Loss rate on Uu: This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface.

For split gNBs only - UL PDCP SDU Loss Rate: This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface.

The first peculiarity to note is that the DL Packet Loss rate on Uu (RLC layer), which is valid for all type of gNB deployments (split or non split), is only available in DL, while the UL PDCP SDU Loss Rate (PDCP layer) is valid only for split gNBs and it is not available in DL.

Keeping aside what is available in the specifications, a packet loss rate may be performed either at the PDCP level or at the RLC level to derive the packet loss rate at different levels in the protocol stack. 
In the context of UE performance reporting for packet losses however, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. This is also better because it takes into account losses over the interfaces such as F1. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service.

Although PDCP level packet loss measurements are a better choice, there are drawbacks with those too. For a DRB delivered over RLC Unacknowledged mode, a PDCP packet loss metric as defined above, cannot be computed due to the fact that the PDCP entity cannot keep track of packets that were acknowledged in RLC UM but can only keep track of packets submitted to lower layers for transmission. Therefore, the PDCP entity does not know whether a RLC UM packet has been successfully delivered or not. On the contrary, it is possible to compute packet ross rate under RLC unacknowledged mode at lower layers, i.e., at the RLC entity in the gNB-DU. However, as explained above, the service level performance is not necessarily reflected by packet loss at RLC layer.

In addition to the above, packet loss rate at RLC layer is not available for uplink traffic. To make this measurement available for UL traffic, a change at the UE would be needed, implying high impact on UEs and standard specifications and also implying that the measurement would not be available for legacy UEs.

In summary, the use of Packet Loss rate for UE performance feedback purposes is subject to the following drawbacks:

- UL Packet Loss Rate is NOT available at RLC layer and for non split gNBs.

- DL packet loss rate at RLC indicates loss of packets at lower layers, which may not at all reflect in application packet losses, hence this metric is not useful to derive the performance with which the UE bearers in use are served.  Note that DL packet losses might be purposely accounted for in implementations that adopt aggressive modulation and coding, hence such packet losses are not necessarily an indication of low performance.

- For split gNBs, PDCP level DL packet loss rate is not defined. Even if this was defined for split and non split gNBs, it would not be able to monitor for loses in RLC Unacknowledged Mode. The existing UL PDCP SDU Loss Rate is subject from the same shortfall, i.e. it cannot monitor for loses in RLC Unacknowledged Mode.

Observation 1: Computing packet loss rate is not possible in UL for non split gNBs and for all use cases, and for DL it might not reflect the performance at application level.

Additionally, we reflect on the fact that release 18 discussions on packet loss and the inclusion of this IE in the UE performance feedback were largely derived from the QoS settings for packet loss rate. The specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%. In realistic scenarios, as specified for NR QoS parameters, the expected packet loss/error rate that the network is expected to offer to services is at most on the order of 10-2, e.g. 0.01%. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in a real network (as the value range of the IE is too coarse). In other words, the Average Packet Loss DL IE introduced in TS38.423 is of no use and ignoring this IE would bring no impact to the overall functionality.

Observation 2: the Average Packet Loss DL IE introduced in TS38.423 for UE performance feedback monitoring has an accuracy of 0.1%, which is far above a typical packet loss rate in a real network. Hence this parameter is of no use and ignoring this IE would bring no impact to the overall functionality.

On the basis of this, we conclude that fixing the shortfalls to enable measurements of the packet loss rate in all use cases implies big impacts to the standard and in the end, it does not deliver information that is crucial to understand whether a mobility target is good or bad. We therefore propose the following:

Proposal 1: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 2: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

We understand that there might be companies that still see Packet Loss Rate as a meaningful metric to consider. In that case, we would propose as a very minimum, that an LS is sent to SA5 and RAN2 tasking them to define an UL Packet Loss Rate, if feasible, given that such metric is the most critical when it comes to packet losses over the air.

Proposal 3: If RAN3 decides to maintain the Packet Loss Rate as a performance metric, send an LS to SA5 and RAN2 requesting for the definition of an UL Packet Loss Rate metric, if feasible.

# 3. Conclusion

# In this contribution we present our views in the context of Packet Loss as a metric in UE Performance feedback. The discussion presents the different reasons to why Packet Loss is not a suitable metric in its current form, and the following observations and proposals were made:

# Observation 1: Computing packet loss rate is not possible in UL for non split gNBs and for all use cases, and for DL it might not reflect the performance at application level.

# Observation 2: the Average Packet Loss DL IE introduced in TS38.423 for UE performance feedback monitoring has an accuracy of 0.1%, which is far above a typical packet loss rate in a real network. Hence this parameter is of no use and ignoring this IE would bring no impact to the overall functionality.

Proposal 1: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 2: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

Proposal 3: If RAN3 decides to maintain the Packet Loss Rate as a performance metric, send an LS to SA and RAN2 requesting for the definition of an UL Packet Loss Rate metric, if feasible

A CR reflecting the main proposals in this paper is presented in R3-250454