3GPP TSG-RAN WG3 Meeting #127	R3-250455

# Athens, Greece, 17th – 21st February, 2025

Title: 	Split architecture support for Release 18 use cases

Source: 	Ericsson, Nokia, JIO Platforms (JPL), Deutsche Telekom, FiberCop, Vodafone

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss the details of split architecture support for Release 18 use-cases based on the conclusions reached during the SI [1], the new WID [2], and agreements reached in the preceding RAN3 meetings. During RAN3#126, the following agreements were reached:

- In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

- Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

This paper focuses on the open questions within the UE performance collection topic, primarily discussing the collection and signalling of Delay and Throughput as part of UE performance reporting in split architecture.

# 2. Discussion

## 2.1 Measured UE performance

UE performance metrics in split architecture are computed in the different logical functions in the RAN depending on the information available at each entity based on its role in the protocol stack. These metrics should then be aggregated at the gNB-CU-CP for split architecture deployments to reach parity with the defined functionality for non-split architecture in Rel.18.

#### 2.1.1 Delay

In the context of downlink UE delay measurements, the metrics are defined in TS 28.558, this corresponds to the following metrics collected at different entities:

- D1: Average delay in DL over air-interface (available at gNB-DU)

- D2: Average delay in DL in gNB-DU (available at gNB-DU)

- D3: Average DL delay over F1-U (available at gNB-CU-UP)

- D4: Average DL delay in CU-UP (available at gNB-CU-UP)

In the uplink direction, the delay components consist of the following, as defined in TS 28.558:

- D1: UL PDCP packet average delay (available at the UE and gNB-CU-CP)

- D2.1: Average over-the-air interface UL packet delay (available at gNB-DU)

- D2.2: Average UL RLC packet delay (available at gNB-DU)

- D2.3: Average UL delay over F1-U (available at gNB-CU-UP)

- D2.4: Average UL PDCP re-ordering delay (available at gNB-CU-UP)

The following was agreed as part of the meeting outcomes from RAN3 #125:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase.

The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

During discussions in Rel-19, two main approaches emerged to collecting UE performance feedback metrics at the gNB-CU-CP. They were:

- The gNB-CU-UP-based approach, where the gNB-CU-UP aggregates the required metrics for delay measurements. For example, the gNB-CU-UP may re-use the UL and DL Delay Result computed for the UL PDU SESSION INFORMATION and signals the same to the gNB-CU-CP, and

- The gNB-CU-CP-based approach where both the gNB-CU-UP and the gNB-DU report the available metrics locally to the gNB-CU-CP.

There exist different trade-offs with both the above approaches, which we analyse deeper in the following paragraphs.

Within the gNB-CU-UP-based approach, we first note that there are two gNB-CU-UP-based solutions.

The first involves obtaining from gNB-DU the metrics measured/available at the gNB-DU (listed above) over new procedures/extending existing procedures. The gNB-CU-UP may then combine the collected values with the metrics available at the gNB-CU-UP and signal the cumulative value to the gNB-CU-CP. This approach is however more complex than re-using the results computed for UL PDU session information  as it involves additional signalling and it may also be argued that this approach has no significant advantages compared to the gNB-CU-CP-based approach.

The second approach consists of a gNB-CU-UP-based solution that re-uses UL and DL Delay Result available as part of UL PDU session information, it is beneficial that this information is already available at the gNB-CU-UP, although it is originally intended as a report for the UPF for QoS flow monitoring.

With data collection for AI/ML-related procedures, RAN3 agreed that the metrics have to be collected in accordance with the general principles of AI/ML data collection. One among these principles is the possibility where the requesting node can configure a reporting periodicity on the target. There is no defined reporting periodicity for the metrics computed for the UL PDU Session Information as it may be triggered/collected whenever packets corresponding to a particular PDU session needs to be transferred. Likewise, the QoS monitoring mechanisms currently standardised do not cover the possibility for the gNB-CU-UP to subscribe to the delay information from the gNB-DU or to stop receiving such information.

With respect to the gNB-CU-UP based option it should also be discussed how the gNB-CU-UP would collect delay metrics from the gNB-DU if QoS Monitoring is not enabled.

Observation 1: for gNB-CU-UP based solutions, delay metrics collection for QoS monitoring purposes should be amended to enable delay metric collection for AI/ML purposes.

Now, we discuss the gNB-CU-CP-based approach. In this approach, the gNB-DU and the gNB-CU-UP collect delay metrics in accordance with the definitions in TS 28.588. These metrics shall then be signalled to the gNB-CU-CP for aggregation and then be signalled to a neighbour node that requested UE performance feedback. For the UE performance signalled to a neighbour gNB, the granular components of the delay measurements, e.g., delay over the air interface, delay in the gNB-DU, and others listed above are not individually relevant to derive the UE performance at target cell. It is only the total end-to-end delay in the UL or DL per UE that is of interest in this context. It may therefore be efficient to signal a compound measurement that is generated by summing up the cumulative measurements from the gNB-CU-UP or the gNB-DU to the gNB-CU-CP. However, the delay measurements defined in TS 28.558 Sec. 6.3.1.1, and the corresponding MDT measurements defined in TS 37.320 are defined per QoS level, or per S-NSSAI, or at a granularity of per-DRB- per UE. UE performance, as the name suggests, is at the granularity of the UE. RAN3 in meeting #124 agreed that the UE performance per UE is derived by average over the per-DRB measurements of different metrics corresponding to the UE.

UE level performance is derived as the average value of per DRB level measurements of the UE.

With the above approach however, there exists different methods to average the different DRB level measurements corresponding to a metric. For example, one alternative may be individually generating an average delay in the gNB-CU-UP or the gNB-DU for each delay sub-component, or another alternative may be signalling the raw measurements to the gNB-CU-CP to average and generate the per-UE-level UE performance metric. Other options include averaging the individual delay metrics measured per-DRB across all the DRB sessions and generating an arithmetic average, or by summing all the numerator values, e.g., measuring the delay components received in a time-period at the CU-UP, CU-CP, or DU, before generating a per-UE performance metric for UL/DL delay by dividing by the number of samples or time duration. All the above listed values may represent a certain average value. However, when it comes to exchange of UE performance across gNBs, a single method must be defined and followed according to agreed principles such that the collected data and its interpretation is correctly understood. Based on the above discussion, we note the following:

Observation 2: It is unclear how the specifications in TS 28.558 allow to generate per-UE-level measurement reports

In addition to ensuring that the averages are computed in a well-defined manner, it is also vital that the different component measurements measured at the UE (i.e. UL PDCP packet average delay already available at the gNB-CU-CP), gNB-DU and the gNB-CU-UP follow the principles below:

- are measured over the same time-period, and

- are aligned with one another when a cumulative UL/DL performance measurement is generated at the gNB-CU-CP, i.e., the samples from the gNB-CU-UP and the gNB-DU are available at the gNB-CU-CP when a cumulative measurement is calculated.

For the alignment of measurements, the gNB-CU-CP, which configures the reporting of the metrics from the UE, gNB-DU and the gNB-CU-UP, must ensure that the measurements are configured with the same reporting periodicity. For measurements collected at the UE it might suffice that they are configured with a periodicity that is equal or smaller than the reporting periodicity for gNB-DU and gNB-CU-UP measurements. Whilst to ensure that the measurements reported are measured within the same time-period, it is essential that a reported measurement shall be taken within the previous time period and not in time periods before or after it. With these conditions, the gNB-CU-CP would be able to collate delay measurements coming from the UE, gNB-DU and from the gNB-CU-UP and received within the same time period, thereby forming a total value representing the delay experienced across the configured time period.

Observation 3: Timing and sync between the different logical functions should be carefully considered for the gNB-CU-CP-based approach.

Proposal 1: RAN3 to discuss the gNB-CU-UP-based and gNB-CU-CP-based approach to obtain delay measurement for UE performance feedback based on the following items.

For the gNB-CU-UP-based approach, further discussion is needed in the following areas:

- collection of the delay metrics at different periodicities

- start and stop for delay metrics reporting at gNB/CU/UP

- collection of metrics when QoS flow monitoring is not enabled

For the gNB-CU-CP-based approach, further discussion is needed in the following areas:

- computing per-UE-level measurement reports

- timing and sync between the gNB-CU-CP, gNB-CU-UP, and the gNB-DU.

#### 2.1.2 Throughput

As discussed in Ericsson’s previous contributions to R3#125 (R3-244417) and R3#126 (R3-247524), any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. TS 28.558 defines throughput measurements, however, the Average DL and UL UE throughput in gNB is defined as the ratio of successfully delivered RLC SDU data volume over time. This measure is available at the gNB-DU. The throughput metric should be analysed further in the context of UE performance. The intention with UE performance metrics is to compare the observed performance in the target immediately after the handover to evaluate an AI/ML-based handover decision. It is also well understood that the throughput at RLC-level does not take into account congestion, higher-layer packet losses (which led to a lower overall throughput in reality), or delays in interfaces upstream of the gNB-DU. These issues are also brought up in R3-247622.

An alternative to the above is to obtain the UE throughput measurements from the gNB-CU-UP, which is the termination point for PDCP. While there are no measurements defined for PDCP throughput measurements at the gNB-CU-UP, the specifications imply that such functionality is available at the gNB-CU-UP as part of the traffic policing functionality. This functionality is likely up to the implementation. However, if such a metric were to be used in inter-node signalling, this would require a more rigorous definition of the measurement.

Proposal 2: RAN3 to discuss the gNB-CU-UP-based (PDCP) and gNB-DU-based (RLC) approach to obtain UE throughput measurement for UE performance feedback.

## 2.2 Signalling aspects

In light of the solutions discussed in this paper for the collection of delay and throughput metrics, two distinct groupings emerge.

- gNB-CU-UP-based solutions: In the case of the gNB-CU-UP-based approach to obtaining delay metrics and using the gNB-CU-UP-based approach to obtain the throughput measurements, no new metrics need to be exchanged from the gNB-DU to the gNB-CU-CP or the gNB-CU-UP. The only impacted interface is E1.

- Solutions which impact the gNB-DU: with this group, i.e., the gNB-CU-CP-based solution to obtain delay metrics and the gNB-DU-based approach to obtain UE throughput at RLC level, there is a requirement to introduce new signaling procedures to both the F1 and the E1 interfaces.

The signaling impact of the solution chosen for collecting UE performance metrics should also be considered as a part of the discussion. On one hand we have the following agreement from RAN3 #125-bis, in the context of transmission of Energy Cost from the DU to the CU:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

On the other hand, one set of solutions discussed in this paper do not impact the F1 interface at all. While the discussions in RAN3 are moving ahead step-by-step, it is also important to choose enhancements such that the needed functionality is implemented with the least disruption to the specifications. Based on the above, we propose the following:

Proposal 3: RAN3 to revisit earlier agreement to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 for the transfer of measured EC from DU to the CU depending on the agreed outcomes on signalling of Packet delay and throughput for UE performance in split architectures.

However, if the solutions agreed are those that impact the gNB-DU and the F1 interface, then the agreed Data Collection Reporting Initiation and Data Collection Reporting procedures shall be extended with the other metrics.

Below we discuss the need for signaling enhancements between the gNB-CU-UP and the gNB-CU-CP to exchange the UE performance metrics.

Currently over the E1 interface, there already exists Resource Status Reporting procedure which is used by a gNB-CU to request from the gNB-CU-UP reporting of measurements which may be used for various purposes, e.g. as AI/ML inference inputs or feedback information. From an operational perspective, the gNB-CU-CP uses the information received over the Resource Status Reporting procedure for actions such as load balancing, resource planning, etc., and extending this procedure with UE performance metrics seems a viable option that does not violate the principles of the Resource Status Reporting procedure.

Another alternative, that is comparatively more disruptive, is to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UE performance metrics from the CU-UP to the CU-UP. This approach would mean that the same procedural design is preserved over the XN, F1, and E1 interfaces.

Proposal4: RAN3 to discuss and agree on procedure to signal UE performance metrics over E1 interface.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals.

Proposal 1: RAN3 to discuss the gNB-CU-UP-based and gNB-CU-CP-based approach to obtain delay measurement for UE performance feedback based on the following items.

For the gNB-CU-UP-based approach, further discussion is needed in the following areas:

- collection of the delay metrics at different periodicities

- start and stop for delay metrics reporting at gNB/CU/UP

- collection of metrics when QoS flow monitoring is not enabled

For the gNB-CU-CP-based approach, further discussion is needed in the following areas:

- computing per-UE-level measurement reports

- timing and sync between the gNB-CU-CP, gNB-CU-UP, and the gNB-DU.

Proposal 2: RAN3 to discuss the gNB-CU-UP-based (PDCP) and gNB-DU-based (RLC) approach to obtain UE throughput measurement for UE performance feedback.

Proposal 3: RAN3 to revisit earlier agreement to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 for the transfer of measured EC from DU to the CU depending on the agreed outcomes for on signalling of Packet delay and throughput for UE performance in split architectures.Proposal4: RAN3 to discuss and agree on procedure to signal UE performance metrics over E1 interface.

TPs to TS38.473 and TS37.483, mirroring the proposals above, are presented in R3-250456, R3-250457 and R3-250458.

Note that two TPs have been made available for TS38.473, each including a different signalling option.

# 4. References

- R3-244810 (TP to 38.473) Conclusion on AI/ML NG-RAN Enhancements

- RP-242385 New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN