3GPP TSG-RAN WG3 Meeting #127	R3-250465

Athens, Greece, 17-21 Feb 2025

Title: 	Further discussions on AI/ML enabled CCO

Source: 	Ericsson, Jio Platforms (JPL), InterDigital

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we discuss the following open points and WA from previous meetings:

RAN3#125-bis:

- Information to validate the prediction is needed.

RAN3#126:

- Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

- WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

# 2. Discussion

## Validate a prediction

With AI/ML enabled CCO, a predicted CCO issue can be the output of AI/ML inference, and regardless of whether a predicted CCO issue type is “coverage” or “cell edge capacity” the action that will be taken to prevent the issue is a change in coverage. When the CCO-related action results in one cell (or beam) shrinking and another one expanding, the fundamental effect that UEs will experience is a change in the signal strength and/or quality of the radio channel. Some UEs that were served by a cell will be served by another one, and all UEs in the vicinity of the CCO issue area will measure different levels of RSRP/RSRQ/SINR for the cells.

It can happen that the performance of the UE (e.g. throughput, delay) in the cells affected by CCO is different, but this is an indirect effect, potentially influenced by many other factors, like user behavior or the RRM policy for a given UE. Whether the CCO changes at radio environment level are good or bad is not reflected by UE performance metrics. Instead, radio measurements (RSRP, RSRQ, SINR) can provide a more precise and direct information to understand whether a CCO-related action resulted in a good or bad coverage for the UEs. Ultimately, a CCO capacity issue (caused by cross cell interference) or a CCO coverage issue (caused by lack of signal coverage) are directly reflected by radio measurements on cell reference signals.

For example, a CCO-related action to resolve a CCO capacity issue may lead to a contraction in coverage for cell 1 of gNB1 and to the expansion of coverage for a neighbor cell 2 of gNB2. Some UEs will be handed over from cell 1 to cell 2. To understand if the issue was prevented, UEs in cell 2 should not suffer high cross cell interference.
To determine if the CCO issue was prevented, gNB1 may request gNB2 to report UE measurements (RSRP/RSRQ/SINR) for those UEs which will be handed over to gNB2.

We focus our attention on the UEs that are handed over between cells involved in a CCO state change, because by doing this we can reuse the UE Performance reporting framework. Namely we can simply enhance the UE performance reported after a handover, hence collecting feedback information for CCO and limiting the impact on existing procedures.

According to this principle, gNB2 would send cell level measurements (RSRP, RSRQ) to gNB 1 at part of the UE Performance reporting after HO from gNB1 to gNB2. After gNB2 sends the UE radio measurements to gNB1, gNB1 uses them to understand if the predicted CCO issue was prevented by means of the CCO State changes. This also creates an understanding of whether the predicted CCO issue would have occurred if the CCO state changes were not applied.

For example, if the predicted CCO issue was an interference at cell edge, a good SINR reported by UEs in the area affected by the CCO state changes indicates that this was successfully prevented. If the predicted CCO issue was related to coverage, a good RSRP reported by UEs in the area affected by the CCO state changes indicates that this was successfully prevented.

We also would like to note that reporting of radio measurements on a per UE level, even if for a different purpose, is not new in RAN3. Over the X2, as part of Resource Status Reporting, RSRP measurements for a UE can be requested (and reported) between eNBs. We can reuse a similar approach for CCO, with a request to report UE radio measurements in the Data Collection Reporting. At handover preparation, the Data Collection ID is added to the Handover Request, and the target gNB collects the radio measurements for the handed over UEs. The reporting is UE-specific and can be provided in the Data Collection Update as part of UE Performance.

An example of how the Report Characteristics can be extended to request UE radio measurements is provided in the table below.

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = UE measured RSRP Twelfth Bit = UE measured RSRQ Thirtheen Bit = UE measured SINR Other bits are ignored by the NG-RAN node2. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

An example of UE Measurement Report that can be used to extend UE performance contained in the Xn Data Collection Update is provided in the table below.

9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| UE Measured RSRP | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |
| UE Measured RSRQ | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |
| UE Measured SINR | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |

Proposal 1: To request feedback for AI/ML based CCO, reuse the Report Characteristics IE in the Data Collection Reporting Request to request for UE radio measurements.

Proposal 2: To report feedback for AI/ML based CCO, extend the UE Performance IE to include UE radio measurements.

## Value 0 for Future Cell Coverage State

At the last RAN3 meeting, an Editor’s Note has been captured in the BL CR to 38.473 and 38.423, to indicate that value 0 for Future Cell Coverage State IE needs to be further discussed:

Editor’s Note: Value 0 of Future Cell Coverage State IE to be further discussed.

If a CCO issue is predicted at a gNB1, the node can send to a neighbour gNB2 the future CCO state that will be applied at gNB1 to prevent the issue, and in particular, the Future Cell Coverage State IE within the Future Coverage Modification List IE indicates to gNB2 the future coverage configuration that will be adopted for a given cell of gNB1.

In legacy CCO, both in LTE and in NR, the same logic is used for a detected CCO issue: the node (gNB1) detecting the issue can send to a neighbour gNB2 the CCO state that is applied at gNB1 to resolve the issue, and in particular, the Cell Coverage State IE within the Coverage Modification List IE indicates to gNB2 the coverage configuration adopted for a given cell of gNB1.

In legacy CCO, both for LTE and NR, the inter-node signaling supports the exchange of the Cell Coverage State IE as an INTEGER that includes value 0, where “Value ‘0’ indicates that the cell is inactive.”.

LTE (36.423)

| >Cell Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the cell is inactive. Other values Indicate that the cell is active and also indicate the coverage configuration of the concerned cell | – |  |
| --- | --- | --- | --- | --- | --- | --- |

NR (38.423)

| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

Observation 1: For LTE and NR CCO solution, inter-node signaling supports the exchange of Cell Coverage State IE as an INTEGER that includes value 0, where “Value ‘0’ indicates that the cell is inactive.”.

Therefore, according to legacy, it is possible that in relation to a CCO issue, a gNB1 sends to gNB2 a Cell Coverage State for a cell with value 0. In this case, it is likely that gNB2 needs to adjust the coverage of one or more of its cells to pick up some traffic previously served by gNB1. Also, gNB2 should not trigger handovers towards the inactivated cell.

In fact, in legacy CCO, the Cell Coverage State is a fundamental piece of information for the functionality. When a node receives the Cell Coverage State IE, it can understand the applied change in coverage for a cell of its neighbour and adopt a matching CCO state for its cells accordingly, so that e.g., coverage holes are avoided. This is particularly true when the value of Cell Coverage State is 0.

It should be emphasised that the importance of supporting Cell Coverage State = 0 in CCO is primarily to gather an understanding on cell coverage and how to adjust it, and not for energy saving purposes, as it was claimed during the last RAN3 meeting.

Observation 2: In legacy CCO, the Cell Coverage State is a fundamental piece of information to let a node understand how to change the coverage of its cells to adapt to the present coverage of cells of a neighbour node and avoid coverage holes.

The same principle holds for AI/ML based CCO: the Future Cell Coverage State is a fundamental piece of information for the functionality: when a node receives the Future Cell Coverage State IE, it can understand the future change in coverage for a cell of its neighbour, and can generate a future CCO state that will adapt the coverage of its cells accordingly, so that e.g., coverage holes are avoided.

Observation 3: In AI/ML based CCO, the Future Cell Coverage State is a fundamental piece of information to let a node understand how to change in the future the coverage of its cells to adapt to the future coverage of cells of a neighbour node and avoid coverage holes.

In summary, we think that the Future Cell Coverage State is a fundamental piece of information for AI/ML based CCO and value 0 is particularly important, to avoid coverage holes while attempting to prevent a CCO issue in the future. We therefore propose to keep the value 0 in the encoding for the Future Cell Coverage State IE.

Proposal 3: Keep the value 0 for the Future Cell Coverage State IE.

## Time information for predicted CCO issue and future CCO state

At last meeting a WA was reached: “WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.”.

The Requested Prediction Time IE is currently encoded as an INTEGER with range from 1 to 60 seconds:

| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

The range for the Requested Prediction Time was agreed on the basis ofthe fact that predictions related to rel-18 AI/ML enabled use cases (load balancing, mobility optimization, network energy saving) would be beneficial within that time horizon. There are no indications so far that larger time horizons should be considered for AI/ML enabled CCO. 
Along these lines, it can be assumed  that predictions on CCO issues and predictions for legacy Rel18 use cases are correlated. For example, if an implementation can predict the amount of load in a cell 60 seconds from now, the same implementation can also predict that in 60 seconds from now the cell where the load is predicted to increase will suffer from a capacity problem at the edge. 
Based on that, we think that a reasonable assumption is that a CCO issue can be predicted within the same time horizon used for the Requested Prediction Time. For simplicity, we propose then to reuse the same encoding for the time information for predicted CCO issue and for the future CCO state.

Proposal 4: In F1AP, the Time for Predicted CCO Issue and the Time for Future Coverage Modification IEs are encoded as INTEGER with range from 1 to 60 seconds.

Proposal 5: In XnAP, the Time for Future Coverage State is encoded as INTEGER with range from 1 to 60 seconds.

Proposal 6: Agree the TPs to F1AP and XnAP provided in R3-250467 and R3-250466.

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Proposal 1: To request feedback for AI/ML based CCO, reuse the Report Characteristics IE in the Data Collection Reporting Request to request for UE radio measurements.

Proposal 2: To report feedback for AI/ML based CCO, extend the UE Performance IE to include UE radio measurements.

Observation 1: For LTE and NR CCO solution, inter-node signaling supports the exchange of Cell Coverage State IE as an INTEGER that includes value 0, where “Value ‘0’ indicates that the cell is inactive.”.

Observation 2: In legacy CCO, the Cell Coverage State is a fundamental piece of information to let a node understand how to change the coverage of its cells to adapt to the present coverage of cells of a neighbour node and avoid coverage holes.

Observation 3: In AI/ML based CCO, the Future Cell Coverage State is a fundamental piece of information to let a node understand how to change in the future the coverage of its cells to adapt to the future coverage of cells of a neighbour node and avoid coverage holes.

Proposal 3: Keep the value 0 for the Future Cell Coverage State IE.

Proposal 4: In F1AP, the Time for Predicted CCO Issue and the Time for Future Coverage Modification IEs are encoded as INTEGER with range from 1 to 60 seconds.

Proposal 5: In XnAP, the Time for Future Coverage State is encoded as INTEGER with range from 1 to 60 seconds.

Proposal 6: Agree the TPs to F1AP and XnAP provided in R3-250467 and R3-250466.