3GPP TSG-RAN WG3 Meeting #127	R3-250471

Athens, Greece, 17-21 Feb 2025

Title: 	AI/ML enabled Network Slicing

Source: 	Ericsson

Agenda item:	11.2

Document Type:	Discussion and agreement

# Introduction

In the previous RAN3 meeting important progress was made in standardizing stage 3 details for the AIML enabled network slicing support. In this context the stage 3 details that require further discussions and agreement are:

- Requesting per slice UE Performance,

- Reporting per slice UE performance,

- Signalling slice measurement initiation failure.

Through this contribution we discuss these remaining topics and propose potential solutions for covering the required stage 3 details.

# 2. Discussion

## Requesting per slice UE Performance

For the source gNB to request from the target gNB the reporting of per slice UE performance after a successful UE handover the following agreement was made at RAN3#126:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

To implement this agreement the DATA COLLECTION REQUEST message must be enhanced in order to enable the source gNB to request per slice UE Performance reporting from the target gNB. In doing so, the enhancement to be made must be consistent with the design principles agreed in release 18 for UE performance reporting. Furthermore and most importantly, the per slice UE Performance must be expressed via the same set of measurements/ metrics as the release 18 UE Performance, and the UE performance metrics reported by the target gNB per slice shall be comparable to metrics that the source gNB can measure when the UE is served by the source gNB.

Based on the considerations above we propose adding four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message as shown below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| … |  |  |  |  |  |  |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| … |  |  |  |  |  |  |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

Proposal 1: RAN3 to agree adding the Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message, to enable the source gNB to request per slice UE Performance reporting from the target gNB.

In addition, the source gNB must be able to indicate to the target gNB which are the network slices for which the per slice UE Performance reporting is requested. In the absence of this indications, the target gNB will have no options but to report the per slice UE Performance for all the network slices for which the UE has active PDU session in the target cell despite that: i) the source gNB may not need the per slice UE Performance to be reported for this entire set of slices, and ii) some of the slices the UE is using in the target gNB may not be available or supported in the source gNB. To avoid unrequested transferring of per slice UE Performance reports from the target gNB to the source gNB we propose adding the list of slices for which the per slice UE performance reporting is requested by the source gNB into the UE Performance Collection Configuration IE as shown below:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Slice To Report List for UE Performance |  | 0..1 |  | S-NSSAI list for which UE performance measurements are requested. |
| >Slice To Report Item for UE Performance |  | 1 .. < maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI List |  |  | 1 |  |
| >>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>>S-NSSAI | M |  | 9.2.3.21 |  |

Proposal 2: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

## Reporting per slice UE Performance

The discussions on per slice UE Performance reporting have started at RAN3#125 when the following working assumption was turned into an agreement:

WA turned into agreement: Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase

Starting with RAN3#125-bis the focus of the discussions was set on defining the granularity of per slice UE Performance and the following working assumption and FFS was made during the meeting:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

At RAN3#126 the working assumption above was challenged by both, proposals aiming at defining a finer granularity for per slice UE Performance such as, e.g. per slice per QoS flow, and also by proposals claiming that a finer granularity is not needed. Finaly the following agreement was captured during the meeting:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

In light of this agreement and based on the previous meeting discussions and further considerations, we came to the conclusion that a per slice, i.e., per S-NSSAI, granularity for per slice UE Performance, is enough.

Indeed, the purpose of per slice UE Performance metrics is to enable an AI model running in the source gNB, to assess, in the context of network slicing operation, if a slice-centric mobility decision for a UE was rightfully or wrongfully taken. The purpose is not to assess the impact of the handover procedure on the quality of service for a service which would require a per PDU session granularity, nor to assess the RRM performance at the target gNB which will require a per QoS flow or even finer granularity.

For the main purpose of validating a slice-centric mobility decision for a UE, a per S-NSSAI granularity for the per slice UE Performance reporting is sufficient and no finer granularity needs to be defined. This approach also ensures that the per slide UE performance reported from source to target gNB is comparable withmeasurements that the source gNB can collect for the UE, namely per slice measurements. 
Furthermore, measuring the per S-NSSAI UE performance comes with marginal additional cost at the target gNB, as, according to the current standards, a gNB must continuously monitor for a given UE and for all the slices used by the UE the downlink and uplink UE’s slice throughput in order to make sure that the values are below the upper limits defined by the core network and sent to the gNB in the UE Slice Maximum Bit Rate, as part of the UE context as shown in the following INITIAL CONTEXT SETUP REQUEST message excerpt from [1].

IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Old AMF; O; AMF Name 9.3.3.21; YES; reject. UE Aggregate Maximum Bit Rate; C-ifPDUsessionResourceSetup; 9.3.1.58; YES; reject. …. UE Slice Maximum Bit Rate List; O; 9.3.1.231; YES; ignore. …. Ranging and Sidelink Positioning Service Information; O; 9.3.1.269; This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services.; YES; ignore. #### 9.3.1.231 UE Slice Maximum Bit Rate List This IE contains the UE Slice Maximum Bit Rate List as specified in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. UE Slice Maximum Bit Rate Item; 1..<maxnoof AllowedS-NSSAIs>; Applicable across all GBR and Non-GBR QoS flows.. >S-NSSAI; M; 9.3.1.24. >UE Slice Maximum Bit Rate Downlink; M; Bit Rate 9.3.1.4; This IE indicates the downlink UE-Slice-MBR as specified in TS 23.501 [9].. >UE Slice Maximum Bit Rate Uplink; M; Bit Rate 9.3.1.4; This IE indicates the uplink UE-Slice-MBR as specified in TS 23.501 [9]..

Considering the discussion above we propose to adopt a per S-NSSAI granularity for per slice UE performance reporting as shown in the example below, which is an excerpt of the DATA COLLECTION UPDATE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| … |  |  |  |  |  |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Slice UE Performance | O |  | 9.2.3.y |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 |  |

Proposal 3: RAN3 to adopt a per S-NSSAI granularity for per slice UE Performance reporting.

## Slice specific measurements Initiation failure indication

When designing the measurements initiation failure indication for slice specific measurements one should make the distinction between UE level slice measurements and cell level slice specific measurements. For UE level slice specific measurements such as Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, and Slice Average Packet Loss DL which are part of the per slice UE Performance measurements, we suggest to follow the example of the release 18 UE Performance measurements initiation failure indication and to indicate the initiation failure for these measurements by adding four new bits in the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message as exemplified here after.

Proposal 4: RAN3 to agree indicating the measurements initiation failure for slice UE Performance measurements via four new bits to be added into the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

For cell level slice specific measurements, there is a fundamental need to indicate the measurements initiation failure separately for each slice in the cell for which the measurements have been requested. As an example, lets consider now a gNB having a cell which supports two slices, slice A and slice B for which the reporting of Predicted Slice Available Capacity has been requested by a neighbour gNB. In case that one of the two slices, e.g. slice B, is configured with zero resources for the cell (slice B is supported but not available in the cell) the reporting gNB should fail initiating the measurement of Predicted Slice Available Capacity for slice B and indicate this failure to the requesting gNB. As the slice A is available and supported in the cell, this measurement initiation failure should not concern the slice A in the cell.

Following the example above, we propose that for cell level slice specific measurements, the indication / reporting of the measurements initiation failure from the reporting node to the requesting node with a slice -level granularity. A possible implementation in which the Slice Measurement Initiation Result List IE of the DATA COLLECTION RESPONSE message is used to carry the list of failed measurements for each slice in the cell is given below.

Proposal 5: RAN3 to adopt a “per-slice” granularity for the cell level slice specific measurements initiation failure indication.

IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NG-RAN node1 Measurement ID; M; INTEGER (1..4095,...); Allocated by NG-RAN node1; YES; reject. NG-RAN node2 Measurement ID; M; INTEGER (1..4095,...); Allocated by NG-RAN node2; YES; reject. Node Measurement Initiation Result List; 0..1; List of measurement objects that failed to be initiated in the node.; YES; reject. >Node Measurement Initiation Result Item; 1 .. <maxFailedMeasPerNode>; –. >>Node Measurement Failed Report Characteristics; M; BITSTRING (SIZE(32)); Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory, Seventh Bit = Slice Average UE Throughput DL, Eighth Bit = Slice Average UE Throughput UL, Nineth Bit = Slice Average Packet Delay, Tenth Bit = Slice Average Packet Loss DL, Other bits are ignored by the NG-RAN node1.; –. >>Cause; M; 9.2.3.2; Failure cause for measurement objects for which the measurement cannot be initiated.; –. Cell Measurement Initiation Result List; 0..1; List of measurement objects that failed to be initiated per cell.; YES; reject. >Cell Measurement Initiation Result Item; 1 .. <maxnoofCellsinNG-RANnode>; –. >>Cell ID; M; Global NG-RAN Cell Identity 9.2.2.27; Indicates an NR Cell Identity.; –. >>Cell Measurement Failure Cause List; 0..1; Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell.; –. >>>Cell Measurement Failure Cause Item; 1 .. <maxFailedCellMeasObjects>; –. >>>>Cell Measurement Failed Report Characteristics; M; BITSTRING (SIZE(32)); Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1.; –. >>>>Cause; M; 9.2.3.2; Failure cause for measurement objects for which the measurement cannot be initiated.; –. >>Slice Measurement Initiation Result List; O; 9.2.3.x; –. Criticality Diagnostics; O; 9.2.3.3; YES; ignore. #### 9.2.3.x Slice Measurement Initiation Result List This IE indicates the list of measurement objects that failed to be initiated per slice. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Slice Measurement Initiation Result List; 1..< maxnoofBPLMNs >. >PLMN Identity; M; 9.2.2.4; Broadcast PLMN. >S-NSSAI Measurement Initiation Result List; 1. >>S-NSSAI Measurement Initiation Result Item; 1 .. < maxnoofSliceItems>. >>>S-NSSAI; M; 9.2.3.21. >>>S-NSSAI Measurement Failure Cause List; 1; Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the slice.. >>>>S-NSSAI Measurement Failure Cause Item; 1..< maxFailedSliceMeasObjects >. >>>> S-NSSAI Measurement Failed Report Characteristics; M; BITSTRING (SIZE(32)); Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Slice Available Capacity. Other bits are ignored by the NG-RAN node1.. >>>> Cause; M; 9.2.3.2; Failure cause for measurement objects for which the measurement cannot be initiated..

# Conclusions

In this contribution we discuss the details of per slice UE Performance reporting and the indication of slice specific measurements failure initiation. Throughout the discussion we derived the following proposals:

Proposal 1: RAN3 to agree adding the Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message, to enable the source gNB to request per slice UE Performance reporting from the target gNB.

Proposal 2: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

Proposal 3: RAN3 to adopt a per S-NSSAI granularity for per slice UE Performance reporting.

Proposal 4: RAN3 to agree indicating the measurements initiation failure for slice UE Performance measurements via four new bits to be added into the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

Proposal 5: RAN3 to adopt a “per-slice” granularity for the cell level slice specific measurements initiation failure indication.

The proposals above are reflected in the Text Proposal on AI/ML enabled Network Slicing for (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN provided in R3-250472.

# References

[1]	3GPP TS 38.413 V18.4.0 (2024-12)