3GPP TSG RAN WG3 Meeting #127	R3-250476

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	8.1

Source:	Ericsson

Title :	Discussion on SA2 LS on XR enhancements

Document for:	Discussions & Approval

# Introduction

In this contribution, we provide our views on the LSes received from SA2 and SA on Rel-19 XR listed below:

| R3-250018 | LS Reply on FS_XRM Ph2 | SA2(Huawei) | 8.1 | Rel-19 | FS_XRM_Ph2 |
| --- | --- | --- | --- | --- | --- |
| R3-250032 | LS on multi-modality awareness | TSG SA(China mobile) | 8.1 | Rel-19 | XRM_Ph2, NR_XR_Ph3-Core |

# LS from SA2 on available Data Rate

The LS from SA2 in [1] provides feedback to RAN3’s previous questions to SA2 on the definition of available data rate. The SA2 answers are copied below:

SA2 thanks RAN3 for the LS on FS_XRM Ph2 (R3-244844/S2-2411282) and kindly asks RAN3 to consider the following SA2 feedback. - From RAN3 perspective, the control plane based solution providing periodicity per QoS flow as part of the TSCAI is sufficient for semi-static periodicity case. Other potential cases (e.g. dynamic case) need to be clarified by SA2, if any. Feedback from SA2: SA2 has agreed to exclude the in-band signalling of the periodicity in Rel-19. - For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate. RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting. Feedback from SA2: From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN. Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR). Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

There is a list of questions and ambiguities detected from the SA2 definitions and answers in the LS reply. We list them below:

- First, when SA2 mentions: “it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow”

This represents a misconception by SA2 that “data rate” can be regarded to be “proportional” to “RAN resources” provided for a QoS flow. Such definition may imply that the CN/AF would like to have information about the “cost” of a service in terms of necessary RAN resources, which is not possible with such an information, and which shouldn’t be introduced.

- SA2 says: “While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN”.

This is not clear and is questionable definition on the application usage of the “available data rate” information: e.g., "somewhat". The CN/AF shouldn’t consider “available data rate/resources” when it comes to admission control, e.g. by prioritising one QoS flow of the other etc., there is a clear worksplit between RAN and CN and introducing such feature will act as destabilisation of the 5G system.

- SA2 says: The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP.

This is also questionable. For a GBR QoS flow, the NG-RAN is anyway committed to provide the indicated bit rate for the associated QoS flow, unless Alternative QoS Profiles are enabled. Thus, it is redundant information. Speaking of which, the notion of available data rate seems to be very similar to the concept of Alternative QoS Profiles, which has been introduced specifically in Rel-16 to let the application know what alternatives parameters for the QoS flow can be fulfilled instead, and perform rate adaptation considering what can be fulfilled by RAN. Hence a questionable addition is if UP signalling would have any advantage over existing CP reporting.

Therefore, another question if the application considers that this bit rate may be too stringent for the RAN, then doesn’t it already assume a level of unfulfillment by the RAN? But then why not sending the AQPs together with in the QoS profile, so that the AF can be aware of what is supported, fulfillable and usable for the application for this service, rather than requesting bitrate updates whenever the latter changes in RAN.

Another question is how often such will change.

- SA2 says: The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR).

We note in SA2 discussion there were different views of definition of available data rate meetings (see S2-2410487 and S2-2410190, which provided contradicted definitions, meaning that even SA2 is not sure about the stage 2 design and is expecting a long meeting series to mend a proper definition).

- SA2 says: Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR

the “available data rate” should often be identical to the GFBR according to SA2, then does it mean sending the available bit rate when there is change is considered a very rare case? This questions if UP reporting is the right solution since it is not foreseen to change often.

- Considering the final paragraph from SA2: Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly

there is a number of specifications impacts not negligeable that we have to consider for the reporting the “available data rate” to UPF:

  - There is a first step needed for SMF to know about RAN capability of reporting available data rate via UP during the setup of the PDU session;

  - There is a need of a configuration phase, where SMF upon knowing of RAN capability signals the "thresholds” and triggering conditions for the reporting of “available data rate”.

  - CU-UP configuration is needed, DU configuration, Xn and MR-DC impacts.

  - The signaling framework is similar to ECN in terms of CP configuration with much more impacts and then in-band signalling of the data rates upon meeting the triggering requests.

All of the above impacts cannot be comprehensively discussed and addressed in a couple of meetings in RAN3. There will likely be mistakes and wobbly stage 2/3 design, The definition is also not clear and many questions remain unanswered.

Observation 1: SA2 has a misconception that “data rate” can be regarded to be “proportional” to “RAN resources” provided for a QoS flow and represents information about the “cost” of a service in terms of necessary RAN resources.

Observation 2:  The CN/AF shouldn’t consider “available data rate/resources” when it comes to admission control, e.g. by prioritising one QoS flow of the other etc., there is a clear worksplit between RAN and CN and introducing such feature will be a destabilisation of the 5G system.

Observation 3: there were contradicting views in SA2 about definition of available data rate (see S2-2410487 and S2-2410190)

Observation 4: For a GBR QoS flow, the RAN is committing to meet a bitrate requirement, thus the available data rate is supposed to remain the same. if the AF assumes the bitrate is too stringent, it can signal AQP in the QoS profile.

Observation 5: the Available bit rate definition according to SA2 is not expected to change often and remain the same, thus questionable if UP signaling is the right solution. It is not clear why we cannot re-use AQP and signaling over CP.

Observation 6: The introduction of “available data rate” has many impacts in NGAP signaling, F1AP, E1AP, XnAP, RAN capability sharing, configuration steps, some aspects are very similar to ECN. RAN3 cannot proceed with these impacts without solid motivation from the proponents and added value wrt AQP and ECN features.

Observation 7: the unsolid stage 2 and stage 3 design will come with several mistakes.

Furthermore, we note that we are more than halfway in Rel-19, and SA2 has already a long “basket” of items being agreed that will have RAN3 impacts: DL PDU Set Marking, MMSID, PDU Set AQP, AL-FEC., etc. Considering the time and controversies XR generates in RAN3, it is proposed that RAN3 does not proceed with supporting available data rate and reply to SA2 that this feature adds a lot of work considering other items SA2 has specified.

Observation 8: SA2 has already a long “basket” of items being agreed that will have RAN3 impacts: DL PDU Set Marking, MMSID, PDU Set AQP, AL-FEC. Supporting another feature with unclear definition and benefits is not the right decision considering RAN3 load and time left in Rel-19 XR.

Proposal 1: Reply to SA2 that RAN3 will not proceed with supporting available data rate in Rel-19

A reply LS is available in [3].

# LS from SA on MMSID

SA has sent an LS indicating final support of MMSID from CN to RAN [2]

SA has discussed the RAN2(R2-2409272), RAN3(R3-245682) and SA4(S4-242223) LSs on Multimodality awareness. Based on the understanding that in R2-2409272, RAN2 agreed that multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping and that RAN3 agreed, as reported in R3-245682, that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node, SA has reached consensus that the multimodality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) to the NG-RAN by N2 SM information. SA expects SA2 to handle the above LSs and provide related specification updates based on the principle described above.

The MMSID can be included in the QoS flow information as discussed in previous meetings where RAN3 has agreed that it is technically feasible to signal such information to RAN.

According to SA4 answer in [4]:

Answer: From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established. The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS.

Based on the Answer, it is understood that in general not all multimodal applications will require all media flows to operate, but some may be optional/complementary, but “good” for the RAN to know. Therefore, RAN3 shall not specify how the MMSID is used at RAN; it suffices for RAN to just be aware of its existence in the QoS flow information that the QoS flows are related. For instance, when receiving the MMSID for  particular QoS flows over NGAP during the PDU Session management procedures, the NG-RAN shall, if supported, consider that the indicated QoS flows are related to a multi-modal service, as described in TS 23.501. How it is used is up to implementation.

Observation 9: Based on SA and SA4 answer,  the RAN can receive the MMSID from CN. How RAN uses such information and what “treatment” is performed are up to implementation.

Proposal 2: RAN3 shall not specify how the MMSID is used by NG-RAN when received by SMF for joint admission control. Its usage is up to implementation

# Conclusion

Observations and proposals are listed below:

Observation 1: SA2 has a misconception that “data rate” can be regarded to be “proportional” to “RAN resources” provided for a QoS flow and represents information about the “cost” of a service in terms of necessary RAN resources.

Observation 2:  The CN/AF shouldn’t consider “available data rate/resources” when it comes to admission control, e.g. by prioritising one QoS flow of the other etc., there is a clear worksplit between RAN and CN and introducing such feature will be a destabilisation of the 5G system.

Observation 3: there were contradicting views in SA2 about definition of available data rate (see S2-2410487 and S2-2410190)

Observation 4: For a GBR QoS flow, the RAN is committing to meet a bitrate requirement, thus the available data rate is supposed to remain the same. if the AF assumes the bitrate is too stringent, it can signal AQP in the QoS profile.

Observation 5: the Available bit rate definition according to SA2 is not expected to change often and remain the same, thus questionable if UP signaling is the right solution. It is not clear why we cannot re-use AQP and signaling over CP.

Observation 6: The introduction of “available data rate” has many impacts in NGAP signaling, F1AP, E1AP, XnAP, RAN capability sharing, configuration steps, some aspects are very similar to ECN. RAN3 cannot proceed with these impacts without solid motivation from the proponents and added value wrt AQP and ECN features.

Observation 7: the unsolid stage 2 and stage 3 design will come with several mistakes.

Observation 8: SA2 has already a long “basket” of items being agreed that will have RAN3 impacts: DL PDU Set Marking, MMSID, PDU Set AQP, AL-FEC. Supporting another feature with unclear definition and benefits is not the right decision considering RAN3 load and time left in Rel-19 XR.

Proposal 1: Reply to SA2 that RAN3 will not proceed with supporting available data rate in Rel-19

Observation 9: Based on SA and SA4 answer,  the RAN can receive the MMSID from CN. How RAN uses such information and what “treatment” is performed are up to implementation.

Proposal 2: RAN3 shall not specify how the MMSID is used by NG-RAN when received by SMF for joint admission control. Its usage is up to implementation

# References

[1] R3-250018, LS Reply on FS_XRM Ph2, SA2(Huawei)

[2] R3-250032, LS  on multi-modality awareness, TSG SA(China mobile)

[3] R3-250477, Draft Reply LS to SA2 on FS_XRM Ph2

[4] R3-250027, LS Reply on multi-modality awareness, SA4(Huawei)