3GPP TSG RAN WG3 Meeting #127	R3-250487

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	18.2

Source:	Ericsson

Title:	Support of Low power radio WUS

Document for:	Discussions

# 1	Introduction

This paper discusses further progress to consider in RAN3 to support low power radio WUS.

Below are previous RAN3 agreements:

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups. - The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups. - The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups. - Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping. - The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring. - WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID. - It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE. - NG-RAN node can acquire the total number of CN assigned subgrouping via OAM. - WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

# 2	Discussion on RAN3 impacts

SA2 replied to the RAN2 LS about LP-WUS subgrouping [1] and SA2 captured for LP-WUS similar changes as for CN-based subgrouping with PEI in CRs to 23.501 and 23.502 [2,3].

Observation 1: SA2 has agreed that for LP-WUS the same signalling procedure and signalling messages as with PEI will be used, the introduction of new LPW-WUSPS IEs should be straightforward

## 2.1	LP-WUS terminology

SA2 commented that the Wake-Up Signal is not a low power signal, and therefore the abbreviation LP-WUS could be misleading. SA2 is correct in their understanding that WUS is not a low power signal. But LP refers to the low power receiver used with LP-WUS.

In case the reader of the 3GPP specs is familiar with the LP-WUS feature, there is not a big risk of misunderstanding. Furthermore, the consequences of a misunderstanding are also not clear. But if RAN2 agrees that a correction is needed, it is proposed to update the terminology also in RAN3 from LP-WUS to LPR-WUS (Low Power Receiver - Wake Up Signal).

Proposal 1: If RAN2 agrees that a correction is justified, then RAN3 should rename the feature as LPR-WUS (Low Power Receiver - Wake Up Signal).

## 2.2	LP-WUS usage during emergency PDU session

How to handle UE-ID based subgrouping during an emergency PDU session is FFS in RAN2:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 the use of CN based LP-WUS during an emergency PDU session is also FFS (see section 5.4.12a.2 in 23.501):

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

The background issue is that with LP-WUS, the paging delay is increased due to the large time offset between LP-WUS and the associated PO. There is probability that the paging message is delayed by one DRX cycle.

In legacy, when the UE has an emergency PDU session, then eDRX is not used because eDRX may delay an (emergency) callback. Furthermore CN-assigned PEI is also not allowed when the UE has an emergency PDU session (see 24.501 section 5.3.25) for latency reasons. For LP-WUS the same principle should be kept.

Observation 2: When the UE has an emergency PDU session LP-WUS should not be used.

We note that in the context of eDRX in RRC_INACTIVE, RAN2 asked SA2 [4] whether there can be cases where the UE has an emergency PDU session, but the gNB is not aware of this, i.e. the gNB may release the UE to RRC_INACTIVE with eDRX configured. This topic is not directly related to LP-WUS (it is a rel-17 RAN2 LS), but whether the RAN will be aware or not (e.g., via ARP value), and decide to keep the UE in RRC_CONNECTED during an Emergency PDU session can help solve the issue in Observation 2. In fact, if the UE is kept connected, LP-WUS, and other paging optimization features, will not be used because paging will not be triggered when UE has emergency PDU session. For now RAN3 can wait for SA2 to liaise RAN2, RAN3 and be aware of their final solution.

Observation 3:  When UE has emergency PDU session, if the UE is kept in RRC_CONNECTED, then paging is not triggered and LP-WUS (together with paging optimization features that may cause delay to callback) are not used.

Proposal 2: RAN3 to wait for SA2 final confirmation to RAN2 on LS on Emergency Services and eDRX.

## 2.3	Network verification of LP-WUS capability

In the SA2 LS, SA2 asked RAN2 and RAN3 to investigate the following:

Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

If a pre-R19 gNB is updated with latest ASN.1, even if paging capability is not supported, it can still update the AMF with the updated UE Radio Capability for Paging Information (URCP) container. The AMF will then forward during NG Paging message the UE Radio Paging Capability to NG-RAN with latest ASN.1 who will perform the paging accordingly when it checks the paging capabilities supported. Also during RAN Paging, if the gNB is updated with latest ASN.1, it can inform new gNB during the Xn Paging message.

Observation 4: If a pre-Rel-19 gNB is updated with latest ASN.1, even if paging capability is not supported, it can still update the AMF with the updated UE Radio Capability for Paging Information container, which can send it to a (supporting) gNB for paging. The gNB will then check the container and apply paging as per supported capability.

Therefore, to align on the usage all gNBs, pre or current Release, should be updated with latest ASN.1. This is so that capabilities are not lost. In case of multi-vendor network, this I part of verification process by OAM.

Proposal 3: RAN3 to reply to SA2 that if all gNBs are updated with latest ASN.1, then the capabilities are not lost in multi-vendor network deployment. This would be part of OAM pre-configuration check.

Another aspect is that rationale of the UE Radio Capability for Paging Information (URCP) sent to the RAN at Initial Context Setup, is to act as a sort of “refresher”, i.e., to help build up the IE in the AMF to include information for different RATs (i.e., the RAN can check what information is available at the AMF and overwrite it if needed, and e.g., add redcap support, LP-WUS, etc.).  Then the RAN can modify it if something is missing and build up a fresher URCP to send to AMF. What is missing in NGAP text is the RAN behaviour when it receives the URCP and can therefore check and refresh the AMF including updated URCP capabilities. The CRs submitted in [5,6,7] as corrections CRs to previous releases help clarify this gNB behaviour.

Proposal 4: Agree the NGAP CRs that clarify that when the UE Radio Capability for Paging IE is contained in the INITIAL CONTEXT SETUP REQUEST message (either within the Core Network Assistance Information for RRC INACTIVE IE, or as an independent IE), the NG-RAN node shall, if supported, use it to verify the included information as specified in TS 23.501.

## 2.4	Last used cell info and band mismatch issue

RAN3 can wait for RAN2 agreement on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection).

In PEI, when the UE is released, the last serving gNB informs the CN of the last visited cell. During IDLE paging, the gNB uses this information relayed back from CN to page the UE only in the last visited cell in the first paging attempt. When the UE has not moved and the first paging attempt is limited to the last used cell, then paging in the complete registration/RNA area is avoided

One aspect to note is that the LR part of the UE not only starts and stops monitoring LP-WUS based on RRCReconfiguration, but also stops monitoring LP-WUS when the exit condition for LP-WUS monitoring is met in RRC_CONNECTED and UE switches to MR. Hence, there can be a case where the UE in MR is released from a cell, but LP-WUS is not supported on the PCell frequency in RRC_CONNECTED (i.e. LP-WUS is not supported in the cell/frequency when UE was released). The LR part of the UE will have to try to select a cell on a different frequency that supports LP-WUS, when the current frequency the UE was released from does not support LP-WUS.

This can lead to the first paging attempt to LR part of UE to be missed and fail, because the gNB will only page in the cell in which the MR was released from and indicated so by the CN during the NGAP PAGING message. When the first paging attempt is missed, the network may escalate the paging to the complete registration/RNA area in the second paging attempt to find the UE.

Observation 5: There can be a mismatch problem when an LP-WUS UE MR is released from a cell/carrier that does not support LP-WUS, then the UE LR will try to reselect to a cell/frequency where LP-WUS is supported leading to first paging attempt missed and the network escalating the paging.

Therefore, we think that the gNB when informing the CN about the Information on Recommended Cells and RAN Nodes for Paging IE in the UE CONTEXT RELEASE COMPLETE message,  it must additionally provide the cell/frequency associated to the last used cell by the LP-WUS supporting UE to which the UE LR was redirected to when released. This way, paging can be performed accordingly to ensure that paging is only done in cell where UE has been released and is LP-WUS supporting from a frequency/cell pov.

Proposal 5: RAN3 to discuss that network should be aware of which LP-WUS cell/frequency the LP-WUS supporting UE was released from to avoid first paging attempt being missed, when the RAN pages the LP-WUS in the last visited cell.

# 3	Conclusion

Observation 1: SA2 has agreed that for LP-WUS the same signalling procedure and signalling messages as with PEI will be used, the introduction of new LPW-WUSPS IEs should be straightforward

Proposal 1: If RAN2 agrees that a correction is justified, then RAN3 should rename the feature as LPR-WUS (Low Power Receiver - Wake Up Signal).

Observation 2: When the UE has an emergency PDU session LP-WUS should not be used.

Observation 3:  When UE has emergency PDU session, if the UE is kept in RRC_CONNECTED, then paging is not triggered and LP-WUS (together with paging optimization features that may cause delay to callback) are not used.

Proposal 2: RAN3 to wait for SA2 final confirmation to RAN2 on LS on Emergency Services and eDRX.

Observation 4: If a pre-Rel-19 gNB is updated with latest ASN.1, even if paging capability is not supported, it can still update the AMF with the updated UE Radio Capability for Paging Information container, which can send it to a (supporting) gNB for paging. The gNB will then check the container and apply paging as per supported capability.

Proposal 3: RAN3 to reply to SA2 that if all gNBs are updated with latest ASN.1, then the capabilities are not lost in multi-vendor network deployment. This would be part of OAM pre-configuration check.

Proposal 4: Agree the NGAP CRs that clarify that when the UE Radio Capability for Paging IE is contained in the INITIAL CONTEXT SETUP REQUEST message (either within the Core Network Assistance Information for RRC INACTIVE IE, or as an independent IE), the NG-RAN node shall, if supported, use it to verify the included information as specified in TS 23.501.

Observation 5: There can be a mismatch problem when an LP-WUS UE MR is released from a cell/carrier that does not support LP-WUS, then the UE LR will try to reselect to a cell/frequency where LP-WUS is supported leading to first paging attempt missed and the network escalating the paging.

Proposal 5: RAN3 to discuss that network should be aware of which LP-WUS cell/frequency the LP-WUS supporting UE was released from to avoid first paging attempt being missed, when the RAN pages the LP-WUS in the last visited cell.

# 4	References

- R3-250021, LS Reply on LP-WUS subgrouping, SA2 (Huawei)

- CR5824 (Rel-19 TS 23.501)

- CR5131 (Rel-19 TS 23.502)

- R3-250006, LS on emergency services and eDRX, RAN2 (ZTE)

- R3-250478, Correction on UE Radio Capability for Paging IE, Ericsson, QC, VDF, NGAP (Rel-16)

- R3-250479, Correction on UE Radio Capability for Paging IE, Ericsson, QC, VDF, NGAP (Rel-17)

- R3-250480, Correction on UE Radio Capability for Paging IE, Ericsson, QC, VDF, NGAP (Rel-18)