3GPP TSG RAN WG3 Meeting #127	R3-250492

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	31.2

Source:	Ericsson, NTT DOCOMO, Nokia, Polaris Wireless, ZTE, CATT

Title:	Discussion on improving E-CID positioning accuracy

Document for:	Discussion, Decision

- Introduction

In this contribution, we detail how the E-CID procedures in NRPPa and F1AP can be extended by including a new measurement report granularity to improve the accuracy of E-CID-based positioning.

# 2	Discussion

Currently, E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”. However, within a single cell several TRPs can be associated to the same NR CGI and can be deployed with different beam information. It should be possible then to report the gNB’s measurements from several measuring TRPs which could help improving the quality of the E-CID measurement report. For instance, several angles per TRP could be reported in the E-CID measurement report, each with a measurement quality indicator. This would help LMF to decide how to weight-factor the different reported measurements for calculating UE's position.

Observation 1: Current E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”, however one cell can comprise several TRPs which can report different measurements, if available.

To improve E-CID, it should be possible to add TRP level measurements by adding a new LMF request for multiple measurements, e.g., with a list of measurement quantities to report, and the gNB deciding what to report based on what is available from any measuring TRPs. This will keep the principle of the E-CID’s best effort reporting. A signalling flow is provided below:

Fig. 1: Addition of TRP measurement list in the E-CID Measurement Initiation procedure

Fig.1 shows the signalling in case of on-demand E-CID measurement request. In case of periodical reporting, the E-CID MEASUREMENT REPORT message can also be extended to provide the list of E-CID TRP Measurement Result List.

Proposal 1: To allow for multiple reporting of measurements, it is proposed to add TRP level of measurement in the E-CID Measurement procedures, so that more measurements can be collected from the RAN side, on a best effort basis, and reported to the LMF. This will improve the accuracy of E-CID positioning method.

Regarding the impacted measurements, UL-AoA can differ from one TRP to another. So it is proposed to start with this measurement and discuss if others (e.g., NR TA) could also be affected.

An example of encoding is proposed in the CR to NRPPa provided in [1] and the F1AP provided in [2].

# 3	Conclusions

Observation 1: Current E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”, however one cell can comprise several TRPs which can report different measurements, if available.

Proposal 1: To allow for multiple reporting of measurements, it is proposed to add TRP level of measurement in the E-CID Measurement procedures, so that more measurements can be collected from the RAN side, on a best effort basis, and reported to the LMF. This will improve the accuracy of E-CID positioning method.

# 4	References

- R3-250493, E-CID measurement enhancement, NRPPa CR, Ericsson, DCM, Nokia, Polaris W

- R3-250494, E-CID measurement enhancement, F1AP CR, Ericsson, DCM, Nokia, Polaris W