3GPP TSG RAN WG3 Meeting #127	R3-250493

Athens, Greece, 17 - 21 February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0186 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E-CID measurement enhancement [E-CID-enh1] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT Docomo, Nokia, Polaris Wireless, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current E-CID framework in NRPPa is designed on the concept of “one cell level measurement”, however one cell can comprise several measuring TRPs which can report different measurements. To allow for multiple reporting of measurements, it is proposed to add a request for multiple measurement quantities the E-CID Measurement procedures, so that more measurements can be collected from the RAN side. Based on the available transmitting TRPs in the NG-RAN, the NG-RAN reports the list of measurement result to the LMF. This will improve the accuracy of E-CID positioning method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce an E-CID TRP Measurement Request Info IE to allow LMF to request a list of measurements - Introduce an E-CID TRP Measuremen Result IE in the E-CID MEASUREMENT INITIATION RESPONSE and E-CID MEASUREMENT INITIATION REPORT messages, containing a list of available measuring TRPs with a list of results per TRP indicating the measurement (currently only UL-AoA), Time stamp and measurement quality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to exploit the existence of several angles per TRP when MIMO transmission is active, which could be reported in the E-CID measurement report, each with a measurement quality indicator. This would improve positioning accuracy with UL-AoA measurements for calculating the UE position using E-CID method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.3.2, 9.1.1.1, 9.1.1.2, 9.1.1.5, 9.2.x1 (new), 9.2.x2 (new), 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1543 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 8.2.1	E-CID Measurement Initiation

#### 8.2.1.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the LMF to request the NG-RAN node to report E-CID measurements used by LMF to compute the location of the UE.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-CID Measurement Initiation procedure, successful operation

The LMF initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the NG-RAN node is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement results other than the "Cell-ID" have been requested.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the NG-RAN Access Point Position IE or the Geographical Coordinates IE in the E-CID Measurement Result IE, and the LMF shall consider that the E-CID measurements for the UE has been terminated by the NG-RAN node. If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT INITIATION RESPONSE message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement. If the Report Characteristics IE is set to "OnDemand" and the Inter-RAT Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, in the Inter-RAT Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message. If the Report Characteristics IE is set to "OnDemand" and the WLAN Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, in the WLAN Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

If the Report Characteristics IE is set to "OnDemand" and the E-CID TRP Measurement Request Info IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node shall, if supported, provide the corresponding TRP measurements, if available in the NG-RAN node, in the E-CID TRP Measurement Result IE in the E-CID MEASUREMENT INITIATION RESPONSE message.

If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including the E-CID Measurement Result IE, Cell Portion ID IE, or E-CID TRP Measurement Result IE in this message. The NG-RAN node shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

//Next change//

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

If available, the NG-RAN node shall include the Mobile Access Point Location Information IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Mobile Access Point Location Information IE, the LMF may use the value as the location information of the mobile NG-RAN access point that is associated to the mobile TRP.

If the E-CID TRP Measurement Result IE is included in the E-CID MEASUREMENT REPORT message, the LMF shall, if supported, consider it as the measurement results from the indicated TRPs.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

#### 8.2.3.4	Abnormal Conditions

Void.

//Next change//

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15 ,…, 16..256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 60min applies only for ng-eNB. The codepoint “extended” is not applicable. This IE is not applicable to NR Angle of Arrival. | YES | reject |
| Measurement Quantities |  | 1 |  |  | YES | reject |
| >Measurement Quantities Item |  | 1..<maxnoMeas> |  |  | EACH | reject |
| >>Measurement Quantities Value | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival, NR Timing Advance, UE Rx-Tx Time Difference) |  | - | - |
| Other-RAT Measurement Quantities |  | 0..1 |  |  | YES | ignore |
| >Other-RAT Measurement Quantities Item |  | 0 .. <maxnoMeas> |  |  | EACH | reject |
| >>Other-RAT Measurement Quantities Value | M |  | ENUMERATED (GERAN, UTRAN,…, NR, EUTRA) |  | - |  |
| WLAN Measurement Quantities |  | 0..1 |  |  | YES | ignore |
| >WLAN Measurement Quantities Item |  | 0 .. <maxnoMeas> |  |  | EACH | reject |
| >>WLAN Measurement Quantities Value | M |  | ENUMERATED (WLAN, ...) |  | - |  |
| Measurement Periodicity NR-AoA | C- ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA |  | ENUMERATED (160ms, 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| E-CID TRP Measurement Request Info | O |  | 9.2.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA | This IE shall be present if the Report Characteristics IE is set to the value "Periodic" and the Measurement Quantities Value IE in the Measurement Quantities IE is set to the value "NR Angle of Arrival". |

#### 9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement is successfully initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Other-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.14 |  | YES | ignore |
| E-CID TRP Measurement Result | O |  | 9.2.x2 |  | YES | ignore |

//Next change//

#### 9.1.1.5	E-CID MEASUREMENT REPORT

This message is sent by NG-RAN node to report the results of the requested E-CID measurement.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.5 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| E-CID TRP Measurement Result | O |  | 9.2.x2 |  | YES | ignore |

//Next change//

### 9.2.x1	E-CID TRP Measurement Request Info

This information element contains the requested E-CID TRP measurement information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-CID TRP Measurement Quantities |  | 1 |  |  |
| >E-CID TRP Measurement Quantities Item |  | 1 .. <maxnoMeas> |  |  |
| >>E-CID Measurement Quantities Value | M |  | ENUMERATED (NR Angle of Arrival, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

### 9.2.x2	E-CID TRP Measurement Result

This information element contains the E-CID TRP measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-CID TRP Measurement Response List |  | 1 |  |  |
| >E-CID TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>E-CID Measured Result Item | M | 1 ..<maxnoMeas> |  |  |
| >>>CHOICE E-CID Measured Results Value | M |  |  |  |
| >>>>UL Angle of Arrival |  |  | 9.2.38 |  |
| >>>Time Stamp | M |  | 9.2.42 | Time Stamp of the corresponding measured result. |
| >>>Measurement Quality | O |  | 9.2.43 | Measurement Quality of the corresponding measured result. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

//Next change//

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Omitted text unchanged

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

PosValidityAreaCellList,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

PosSRSResourceSet-Aggregation-List,

Remote-UE-Indication-Request,

Remote-UE-Indication,

Remote-UE-Status,

E-CID-TRP-Measurement-Request-Info,

E-CID-TRP-Measurement-Result,

Omitted text unchanged

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-PosValidityAreaCellList,

id-SRSReservationType,

id-NewCellIdentity,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-PosSRSResourceSet-Aggregation-List,

id-PreconfiguredSRSInformation,

id-Remote-UE-Indication-Request,

id-Remote-UE-Indication,

id-Remote-UE-Status,

id-E-CID-TRP-Measurement-Request-Info,

id-E-CID-TRP-Measurement-Result

Omitted text unchanged

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION REQUEST

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID			CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics				PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity				PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to the value “periodic” --

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities				PRESENCE mandatory}|

{ ID id-OtherRATMeasurementQuantities	CRITICALITY ignore	TYPE OtherRATMeasurementQuantities		PRESENCE optional}|

{ ID id-WLANMeasurementQuantities		CRITICALITY ignore	TYPE WLANMeasurementQuantities			PRESENCE optional}|

{ ID id-MeasurementPeriodicityNR-AoA	CRITICALITY reject	TYPE MeasurementPeriodicityNR-AoA		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to the value “periodic” and the Measurement Quantities Value IE in the Measurement Quantities IE is set to the value "NR Angle of Arrival" –

{ ID id-E-CID-TRP-Measurement-Request-Info			CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Request-Info		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION RESPONSE

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID					PRESENCE optional}|

{ ID id-OtherRATMeasurementResult	CRITICALITY ignore	TYPE OtherRATMeasurementResult		PRESENCE optional}|

{ ID id-WLANMeasurementResult		CRITICALITY ignore	TYPE WLANMeasurementResult			PRESENCE optional}|

{ ID id-E-CID-TRP-Measurement-Result	CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Result		PRESENCE optional},

...

}

Omitted text unchanged

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-E-CID-TRP-Measurement-Result	CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Result		PRESENCE optional},

...

}

//Next change//

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

Omitted text unchanged

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

E-CID-MeasurementResult-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-GeographicalCoordinates	CRITICALITY ignore	EXTENSION GeographicalCoordinates PRESENCE optional}|

{ ID id-MobileAccessPointLocation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-MeasuredResultsAssociatedInfoList		CRITICALITY ignore	EXTENSION MeasuredResultsAssociatedInfoList		PRESENCE optional},

...

}

E-CID-TRP-Measurement-Request-Info ::= SEQUENCE {

eCID-TRPMeasQuantity			E-CID-TRP-Measurement-Quantities,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-Measurement-Request-Info-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-Measurement-Request-Info-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TRP-Measurement-Quantities ::= SEQUENCE (SIZE (1.. maxnoMeas)) OF E-CID-TRP-Measurement-Quantities-Item

E-CID-TRP-Measurement-Quantities-Item ::= SEQUENCE {

eCID-MeasurementQuantityValue	E-CID-MeasurementQuantityValue,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-Measurement-Quantities-Item-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-Measurement-Quantities-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantityValue ::= ENUMERATED {

angleOfArrivalNR,

...

}

E-CID-TRP-Measurement-Result ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF E-CID-TRP-MeasurementResponseItem

E-CID-TRP-MeasurementResponseItem ::= SEQUENCE {

tRP-ID							TRP-ID,

e-CID-measurementResult			E-CID-TrpMeasurementResult,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-MeasurementResponseItem-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-MeasurementResponseItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF E-CID-TrpMeasurementResultItem

E-CID-TrpMeasurementResultItem ::= SEQUENCE {

e-CID-measuredResultsValue			E-CID-TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer {{ E-CID-TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

E-CID-TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

choice-extension	ProtocolIE-Single-Container { { E-CID-TrpMeasuredResultsValue-ExtIEs } }

}

E-CID-TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

EUTRACellIdentifier ::= BIT STRING (SIZE (28))

EARFCN ::= INTEGER (0..262143, ...)

Expected-Value-AoA ::= INTEGER (0..3599)

Expected-Value-ZoA ::= INTEGER (0..1799)

//Next change//

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

Omitted text unchanged

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-Remote-UE-Indication-Request									ProtocolIE-ID ::= 159

id-Remote-UE-Indication											ProtocolIE-ID ::= 160

id-Remote-UE-Status												ProtocolIE-ID ::= 161

id-LocalOrigin													ProtocolIE-ID ::= 162

id-PreconfiguredSRSInformation									ProtocolIE-ID ::= 163

id-DL-reference-signal-UERxTx-TD								ProtocolIE-ID ::= 164

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-E-CID-TRP-Measurement-Request-Info							ProtocolIE-ID ::= xxx

id-E-CID-TRP-Measurement-Result									ProtocolIE-ID ::= xxx

//End of change//