3GPP TSG-RAN WG3 Meeting #127			                                         R3-250500

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	10.4

Source:	ZTE Corporation

Title: 		(TPs to BL CR for 38.300, 38.413) Further discussion on Rel-18 leftovers

Document for:	Discussion and Approval

1	Introduction

This paper discusses the Rel-18 leftovers of SON/MDT.

- Discussion

2.1 RACH optimization for SDT

During the RAN2#128 meeting, the following agreements were reached.

Agreements - The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. - The following failure causes can be logged for failed SDT in RA report: ⁻ upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; ⁻ upon random access problem indication is received from MCG MAC while SDT procedure is ongoing; ⁻ upon T319a expires; ⁻ upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; ⁻ FFS: unsuccessfully completed upon cell re-selection.

Based on the SDT failure cases above, in most cases, it is difficult for the network to determine whether the failure occurred during the CG-SDT or RA-SDT procedure, and optimize the corresponding parameters. We see benefit in including an indication in the RA report, to indicate whether the failure occurred during the CG-SDT or RA-SDT procedure.

Proposal 1:   RAN3 agrees to include an indication in the RA report, to indicate whether the failure occurred during the CG-SDT or RA-SDT procedure.

2.2 MHI Enhancement for SCG Deactivation/Activation

RAN3 agreed the following at previous meeting.

RAN3 agree to support a network-based solution for enhancing PSCell activation/deactivation.

SCG (de)activation is to tune data traffic transmission to optimize DC configurations to balance the UE battery usage and data traffic efficiency. Although the percentage of SCG activation time can provide insights into UE traffic volume, there is no guarantee that this percentage is appropriately configured by the RAN. Therefore, it would be beneficial to include UE traffic characteristics (e.g., average throughput (bit/s) during cell activation) over SCG together with activation time information. This allows the RAN to better tune UE traffic over SCG resources based on UE history information.  We suggest RAN3 to discuss additional assistance information, such as UE traffic characteristics (e.g., average throughput).

Proposal 2:  RAN3  to discuss additional assistance information, such as UE traffic characteristics (e.g., average throughput (bit/s) during cell activation). (The corresponding TPs for network-based solution for TS38.413 and TS38.300 are provided in annex).

- Reference

[1] RAN2#128 Chairman Notes

- Conclusion

Based on the above discussion, we provide the following proposals.

RACH optimization for SDT:

Proposal 1:   RAN3 agrees to include an indication in the RA report, to indicate whether the failure occurred during the CG-SDT or RA-SDT procedure.

MHI Enhancement for SCG Deactivation/Activation:

Proposal 2:  RAN3  to discuss additional assistance information, such as UE traffic characteristics (e.g., average throughput (bit/s) during cell activation). (The corresponding TPs for network-based solution for TS38.413 and TS38.300 are provided in annex).

Annex: Text Proposal

- TP to BL CR for TS 38.413 on MHI enhancement on SCG (de)activaon

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |
| Percentage of SCG Activation Time | O |  | INTEGER (0..100) |  |
| Average UE Throughput DL | O |  | INTEGER (0..4,000,000,000,000,…) | This IE indicates the number of bits delivered by NG-RAN in UL over the corresponding PScell. The unit is: bit/s. |
| Average UE Throughput UL | O |  | INTEGER (0..4,000,000,000,000,…) | This IE indicates the number of bits delivered by NG-RAN in DL over the corresponding PScell. The unit is: bit/s. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>

- TP to BL CR for TS 38.300 on MHI enhancement on SCG (de)activaon

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

### 15.5.4	UE History Information from the UE

The source NG-RAN node collects and stores the UE History Information for as long as the UE stays in one of its cells.

The UE may report the UE history information when connecting to a cell of the NG-RAN node, consisting of PCell and PSCell mobility history information, as specified in TS 38.331 [12].

When information needs to be discarded because the list is full, such information will be discarded in order of its position in the list, starting with the oldest cell record. If the list is full, and the UE history information from the UE is available, the UE history information from the UE should also be discarded.

The resulting information is then used in subsequent handover preparations by means of the Handover Preparation procedures over the NG and XN interfaces, which provide the target NG-RAN node with a list of previously visited cells and associated (per-cell) information elements. The Handover Preparation procedures also trigger the target NG-RAN node to start collection and storage of UE history Information and thus to propagate the collected information.

In case of CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PCell entry (i.e. the source cell) when the UE successfully accesses a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source cell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving the Handover Request message from the source NG-RAN node to receiving the RRC Reconfiguration Complete message from the UE. When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via the Handover Request message, the target NG-RAN node also updates the time UE stayed in cell, percentage of cell activation time, and average UE throughput during cell activation of the latest PSCell entry (i.e. the source PSCell) as specified in TS 37.340 [21].

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>