3GPP TSG-RAN WG3 Meeting #127	R3-250501

Athens, Greece, 17th-21st Feb 2025

Agenda Item:	14.2

Source:	ZTE Corporation

Title:	(TPs to BL CR 38.300 and 38.413) Discussion on NTN broadcast service supporting

Document for:	Discussion & Approval

- Introduction

In previous meetings, the support of NTN broadcast service was discussed, and the following agreements were reached.

RAN3 starts work on MBS broadcast service. WA: The mapped cell ID can be used to define MBS broadcast service in NTN. Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN. RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu. The current MBS broadcast procedures can be reused for NR NTN without any enhancement.

In this contribution, we provide our further views on the support of broadcast service for NTN in Rel-19.

- Discussion

In last meeting, it was agreed that the current MBS broadcast procedures can be reused for NR NTN without any enhancement. Regarding the name and encoding of the geographical service area , RAN2 has replied an LS in [2] as below.

1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceAreaInfo is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

It is clear that RAN2 has considered the IE name of the geographical area as “IntendedServiceAreaInfo”. In addition, SA2 has also replied the LS in [2], where the “NR NTN Intended Service Area” is derived from geographical area information. Based on the above two statements, the intended service area of broadcast service should only represent the geographical area information. Currently, in RAN3, the corresponding IE name to represent the geographical area is “MBS Service Area Small Area List”, to align with RAN2 and SA2’s understanding, RAN3 shall rename the IE as “MBS Service Intended Area List”.

In addition, the current IE name “MBS Service Area Small Area List” may lead to ambiguity. The small area can be interpreted as part of the NTN cell, and it could move with the satellite’s movement in case of gNB as regenerative payload. While, our intention of introducing this intended service area is to provide the MBS service in the specific geographical area.

For the encoding of this IE, since the IE design of “IntendedServiceAreaInfo” is still under discussion in RAN2, and details are not fixed. In this case, RAN3 could wait for the further progress in RAN2.

Proposal 1: It is proposed to rename the IE “MBS Service Small Area List” as “MBS Service Intended Area List” in BL CR 38.413.

Based on the above analysis, the intended service area information only corresponds to the geographical area. In this case, the below highlighted description in current BL CR 38.300 is not correct.

To support the intended service area of a broadcast service (e.g. MBS broadcast) the AMF provides the gNB with the intended service area information. This information may be in the form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UE as defined in TS 38.331 [12].

The intended service area information should be in combination with cell ID list and TAI list rather than in form of them. And it should be the MBS service area information which may be in the form of cell ID list, TAI list, geographical area or any combination.

Proposal 2: It is proposed to correct the description of intended service area information transfer in BL CR 38.300.

- Conclusion

Proposal 1: It is proposed to rename the IE “MBS Service Small Area List” as “MBS Service Intended Area List” in BL CR 38.413.

Proposal 2: It is proposed to correct the description of intended service area information transfer in BL CR 38.300.

The corresponding TPs to BL CRs are given in the Annex

- 4 Reference

- R3-250011	Reply LS on supporting MBS broadcast service for NR NTN, RAN2

- R3-250072	Reply LS on Supporting MBS broadcast service for NR NTN, SA2

- Annex

- TP to BL CR TS38.300

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) the AMF provides the gNB with the intended service area information. This information may be in combination with cell ID list and TAI list as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UE as defined in TS 38.331 [12].

TP to BL CR 38.413

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Service Area Intended Area List |  | 0..<maxnoofIntendedAreasforMBS> |  |  |
| >Intended Area | M |  | 9.3.1.x |  |

Editor’s Note: The IE encoding is pending RAN2 discussion.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofIntendedAreasIforMBS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

9.3.1.x	Intended Area

This IE provides information on the Intended Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Area | M |  | OCTET STRING | Pending RAN2 discussion |

-----------------Next Change-------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-ServiceAreaIntendedAreaList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

UNCHANGED PART OMITTED

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofIntendedAreasforMBS

FROM NGAP-Constants

UNCHANGED PART OMITTED

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-ServiceAreaIntendedAreaList		CRITICALITY ignore	EXTENSION MBS-ServiceAreaSmallAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-ServiceAreaIntendedAreaList ::= SEQUENCE (SIZE(1..maxnoofSmallAreasforMBS)) OF SmallArea

IntendedArea ::= OCTET STRING

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UNCHANGED PART OMITTED

-----------------Next Change-------------------

9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

UNCHANGED PART OMITTED

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofIntendedAreasforMBS					INTEGER ::= 65536

UNCHANGED PART OMITTED

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-MBS-ServiceAreaIntendedAreaList						ProtocolIE-ID ::= x

-----------------End of the Change-------------------