3GPP TSG-RAN WG3 Meeting #127				                          R3-250507

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		(TP to BL CR for 38.423) Further discussion on on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

- This paper provides further discussion on on-demand SIB1.

2	Discussion

During the RAN3#125 bis meeting, the following open issue was discussed.

Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast? Further check on first activation of UL WUS configuration broadcast in Cell A?

Three options are listed in the SOD[R3-245737], see below:

Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.  (XnAP: Cell A gNB “confirm” means that it will broadcast received UL WUS configuration via its SIB right away ( i.e. not wait for 2 days) ; Cell A gNB “reject” means it will not perform the request.)

Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.(The reason is, it may take up to 41 seconds to broadcast UL WUS even Cell A broadcast immediately, NES cell can stay as legacy until it knows that Cell A has broadcast its UL WUS.)

Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. Then, the second message is sent from NES Cell gNB to Cell A gNB to indicate it is time to broadcast UL WUS configuration. And NES cell goes to OD-SIB Mode.

During the RAN3#126 meeting, the following agreements were reached, and a corresponding XnAP BL CR was endorsed in R3-247897. However, the choice of which option to use is still unclear.

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

We prefer Option 3, as it is better suited to a dynamic energy-saving strategy, especially in cases where the NES cell may require assistance from multiple Cell(s) A .To avoid coverage holes, any change in the assistance from these assisting Cell(s) A could lead to a change in the NES cell state, such as switching in or out of NES state. Additionally, when CELL A first receives UL WUS, there is no need to broadcast immediately, as the NES cell may require feedback from other base stations to determine whether to enter the NES state. If the NES cell ultimately does not enter the NES state, CELL A's immediate broadcasting would waste both resources and energy.

Proposal 1: RAN3 selects option3 as a baseline for UL WUS provision, as it is better suited to a dynamic energy-saving strategy. (The corresponding TP for TS38.423 is provided in annex).

Additionally, we suggest waiting for the Xn interface solution to be finalized before discussing the impact on the F1 interface.

Proposal 2: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

3 Conclusion

Based on the above discussion, we provide the following proposals.

Proposal 1: RAN3 selects option3 as a baseline for UL WUS provision, as it is better suited to a dynamic energy-saving strategy. (The corresponding TP for TS38.423 is provided in annex).

Proposal 2: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

Annex

TP for TS 38.423 BL CR for OD-SIB1 (Option3)

Editor’s Note: The procedure, message and the IE names are FFS

8.x.x	UL WUS Configuration Provision (FFS)

8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide , update, or remove the UL WUS configuration information to NG-RAN node2 .

The procedure uses non UE-associated signaling.

8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending a UL WUS CONFIGURATION PROVISION REQUEST message to NG-RAN node2 to provide, update, or remove the UL WUS configuration for one or more cells of NG-RAN node2. Upon receipt, if supported, NG-RAN node2 shall store or update the UL WUS configuration for the indicated cells for subsequent UL WUS configuration broadcasting, or remove the UL WUS configuration for the indicated cells and stop subsequent UL WUS configuration broadcasting. NG-RAN node2 shall then respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot support the broadcasting of the UL WUS configuration in one or more cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message.

8.x.x.4	Abnormal Conditions

Void.

8.x.y	UL WUS Configuration Transmission (FFS)

8.x.y.1	General

The purpose of the UL WUS Configuration Transmission procedure is to enable an NG-RAN node1 to request an NG-RAN node2 to start or pause the UL WUS configuration broadcasting in one or more cells.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS TRANSMISSION REQUEST message to the NG-RAN node2 to start or pause UL WUS configuration broadcasting in one or more cells.

8.x.y.3	Abnormal Conditions

Void.

8.x.y	UL WUS Configuration Transmission Status Update (FFS)

8.x.y.1	General

The purpose of the UL WUS Configuration transmission Status Update procedure is to enable an NG-RAN node2 to inform NG-RAN node1, which initiated the UL WUS configuration transmission request, that the ongoing broadcasting of the UL WUS configuration has stopped in one or more cells.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 initiates the procedure by sending a UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message to an NG-RAN node1 to inform that the ongoing broadcasting of the UL WUS configuration has stopped in one or more cells.

8.x.y.3	Unsuccessful Operation

Not applicable.

8.x.y.4	Abnormal Conditions

Void.

9.x.x.a	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the NG-RAN node2 to write, update, or remove UL WUS Configuration for subsequent UL WUS configuration broadcasting in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| UL WUS Configuration | O |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. if this IE is not present, NG-RAN node2 remove the previous stored UL WUS configuration for assisting cells. | YES | reject |
| Assisting Cells List |  | 1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI at the NG-RAN node2 | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

9.x.x.b	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the UL WUS configuration will be stored, updated, or removed for the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

9.x.x.c	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the UL WUS configuration provision cannot be supported in the one or more requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

9.x.x.d	UL WUS TRANSMISSION REQUEST

This message is sent by NG-RAN node1 to request NG-RAN node2 to start or pause the UL WUS configuration broadcasting in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Request type |  |  | ENUMERATED (start, pause, …) |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

9.x.x.e	UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

This message is sent by an NG-RAN node2 to an NG-RAN node1 to indicate that the ongoing broadcasting of the UL WUS configuration has stopped in one or more cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |