3GPP TSG-RAN WG3 Meeting #127	R3-250512

Athens, Greece, February 17th – 21st 2025

Agenda Item:	12.2

Source:	Ericsson

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreements

# Introduction

This TP reflects the proposals from R3-250511 and R3-250513.

Proposal: Agree the TP for WAB BL CR for TS 38.401, provided below.

# TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

BH-5GC: The 5GC serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-RAN-node: The NG-RAN node serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

WAB-gNB: The gNB functionality, as defined in TS 38.300 [2], that provides NR access interface towards the UE.

WAB-MT: The WAB-node’s function that terminates the Uu interface to the BH-RAN-node using the procedures and behaviours specified for UEs. Corresponds to the MWAB-UE function defined in TS 23.501 [3].

WAB-node: An NG-RAN node comprising the WAB-MT and the WAB-gNB functionality.

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

OAM	Operation, Administration and Maintenance

PLMN	Public Land Mobile Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

UL	Uplink

ULI	User Location Information

WAB	Wireless Access Backhaul

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.3 	NG connection management

The UE’s AMF can become aware that the gNB is a WAB-gNB during the setup or the update of the NG connection. The NG connection(s) of the WAB-gNB can be removed, either due to inter-AMF mobility of a WAB-gNB or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”.

## X.4 	Xn connection management

A WAB-gNB can establish Xn connectivity with the BH-gNB serving the co-located WAB-MT, and with the neighbouring gNBs. The BH-gNB can become aware about the co-location of the WAB-MT and the WAB-gNB during the setup or the update of the Xn connection between the WAB-gNB and the BH-gNB.

Xn connection can also be established between WAB-gNBs.

## X.5 	WAB-node authorization

WAB-node authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB.

The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB via BH PDU session(s). The WAB-MT is authorized based on one or more S-NSSAIs and DNN dedicated for WAB.

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

WAB-node authorization can be steered by policies provided to the node by the OAM or by the SeGW (location-, time- or event-based policies). Alternatively, the OAM can enforce the policies by configuring the WAB-gNB only when the WAB-gNB is authorized.

The WAB-gNB’s service authorization status may change. The OAM ensures that the de-authorization of a WAB-gNB is not conflicting with the de-authorization of WAB-MT, meaning that the allowed S-NSSAI(s) for the BH PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to hand over or release the UEs. The coordination of the WAB-gNB service authorization/configuration from OAM and the update of subscription data (e.g. slice/DNN) of the WAB-MT are handled by the OAM/management system.

In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to hand over, or releases, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- Some or all PDU sessions of the WAB-MT may be released, and the WAB-MT may be de-registered from the network.

## X.6 	Avoidance of multihop WAB topology

Avoidance of multihop topology WAB topology refers to the mechanisms to prevent the WAB-MTs from accessing WAB-gNB cells and being handed over to WAB-gNB(s).

Upon Xn-based WAB-MT handover, the target WAB-gNB rejects the PDU session(s) mapped to the WAB-dedicated S-NSSAI(s) and admits the PDU sessions whose S-NSSAIs it supports, if any.

Upon NG-based WAB-MT handover, the target WAB-gNB or the (target) UE’s AMF rejects the PDU session(s) mapped to the WAB-dedicated S-NSSAI(s), and admits the PDU sessions whose S-NSSAIs it supports, if any.

## X.7 	User Location Information for UEs served by a WAB-gNB

For the UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the WAB ULI, which includes the TAI and the NR CGI of the WAB-gNB’s PLMN/SNPN.

If the WAB-MT and the WAB-gNB are served by the same PLMN/SNPN, the WAB ULI includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s PLMN/SNPN, the WAB-gNB determines the WAB ULI based on the WAB-node’s geo-location.

-------------------------------------------End of changes-------------------------------------------