3GPP TSG-RAN WG3 Meeting #127	             R3-250515

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	20.2

Source:	CMCC

Title:	Support Assisted AI/ML Positioning

Document for:	Discussions & Approval

# Introduction

In the RAN 3 126 meeting, the following agreement for Case 3a has been made [1]:

Agreement (RAN3#126) Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated.

# Discussions on Assisted AI/ML Positioning

AI/ML-assisted positioning utilizes AI/ML models to produce intermediate measurement statistics that play a critical role in determining location. These statistics can include metrics such as the probability of a signal being Line-of-Sight (LOS) or Non-Line-of-Sight (NLOS), the angle of arrival (AoA) or departure (AoD) of signals, and time-of-arrival data (ToA). The NG-RAN Node requires data for AI/ML assisted positioning model training, testing, validation and monitoring. Such data samples includes input and output pairs, where the input data is the raw measurements or channel information between UE and NG-RAN Node (also called as Part A), and the output data is the intermediate measurement statistics for positioning (also called as Part B).

### Model Training

As agreed from the last RAN3 meeting, the NG-RAN Node obtains measurements from TRP and LMF provides ground truth label and related data to the NG-RAN Node, with the latter part having RAN3 impact.

﻿Quoted from TS 38.305 [1], an NRPPa transaction is currently only initiated by the LMF, which is realized by the execution of one procedure defined as a request and a response (class 1), followed by one or several procedures initiated by the NG-RAN Node, each of which is defined as a single message (class 2).

Quoted from TS 38.305 For possible extensibility, the protocol is considered to operate between a generic "access node" (e.g. gNB, ng-eNB) and a "server" (e.g. LMF). An NRPPa transaction is only initiated by the server. In the NAPPa protocol, ﻿the described transaction may be realized by the execution of one procedure defined as a request and a response, followed by one or several procedures initiated by the NG-RAN node (each procedure defined as a single message) to realize the additional responses.

Although the NRPPa protocol was designed that an NRPPa transaction is only initiated by the server (e.g., LMF), it did not take the AI/ML for positioning scenarios into consideration. In order for a NG-RAN Node to be able to initiate a transaction to the LMF, the NG-RAN Node only needs to remember the routing ID of the LMF, which could be attached by the LMF during any past NI-LR, MT-LR or MO-LR positioning requests that involve the NG-RAN Node.

- A Network Induced Location Request (NI-LR) is initiated by a serving AMF to locate a UE. This request is typically made for regulatory purposes, such as verifying the UE's location during an emergency call, confirming the UE's location within a specific country or international area especially in NR satellite access scenarios.

- A Mobile Terminated Location Request (MT-LR) occurs when a LCS client or an Application Function (AF), either external or internal to the serving Public Land Mobile Network (PLMN), sends a request for the location of a target UE. It facilitates the applications involving tracking, location-based services, or regulatory purposes, which needs the UE’s location as external services.

- A Mobile Originated Location Request (MO-LR) occurs when a UE sends a request to its serving PLMN for location-related information. This request is initiated by the UE itself, typically for purposes such as providing its location to a service or application, or for regulatory compliance.

As part of these interactions, the NG-RAN Node identifies and records the routing IDs of all participating LMFs. By capturing and storing these routing IDs, the NG-RAN Node establishes a reference, enabling it to initiate data collection processes with those specific LMFs.

During any NI-LR, MT-LR, MO-LR, a LMF could attach its routing ID in either DL or UL positioning message. An involved NG-RAN Node could record the routing ID of the LMF.

NRPPa protocol could be designed to be initiated by NG-RAN Node, by utilizing the recorded routing ID of the LMF to reach the LMF.

A NG-RAN Node could proactively gather the necessary data samples to support AI/ML-assisted positioning model training, testing, validation, and monitoring. This approach involves the following key components, which are discussed below and illustrated in Figure 1.

Figure 1 Data Collection Procedures in Case 3a

Data Collection Intent Communication: This includes mechanisms such as the NG-RAN Node subscribing to the data sample status in the LMF, by including various criteria. Some examples are:

- Availability of Data Samples Over a Specific Time Period: The LMF provides the NG-RAN Node with information regarding the number of new data samples stored during a defined time frame

- Number of UEs Permitting Positioning: The LMF monitors the number of neighbouring UEs that allow positioning, which could generate data samples for the subscribed NG-RAN Node. The NG-RAN Node get notified of this information in a periodic basis or threshold basis. The UEs’ identifiers might be revealed to the NG-RAN Node, which could decide whether to request the LMF to launch network-triggered UE positioning for those UEs selected by itself.

The subscription mechanism is particularly well-suited for scenarios where the NG-RAN Node requires a consistent and ongoing supply of new data samples. This method enables the NG-RAN Node to convey its long-term, recurring data collection needs to the LMF, creating an efficient and structured process for data acquisition.

Alternatively, data collection can be initiated without using the subscription method. In this approach, the NG-RAN Node can send a direct request to the LMF, offering greater flexibility and immediacy. This allows the NG-RAN Node to initiate the data collection process as needed, based on real-time requirements or specific events.

The NG-RAN Node can indicate its data collection intent to one or multiple LMFs with recorded routing ID of those LMFs.

The NG-RAN Node could subscribe to the data sample availability to one or multiple LMFs with specified criteria.

The NG-RAN Node could be informed with the UEs in the neighborhood that permits positioning from the LMF, and selects the list of UEs for positioning, thus obtaining their data samples.

The NG-RAN Node could proceed with data collection request to the LMF on the on-demand basis.

Data Availability Notification: The LMF can either notify the NG-RAN Node about the availability of data samples based on the subscription criteria or directly transmit the requested data samples as specified in the subscription message.

In the first scenario, where the LMF notifies the NG-RAN Node about available data samples, the NG-RAN Node gains the flexibility to decide when to collect the data. For example, delaying data collection until closer to the next training or monitoring cycle can guarantee the relevance and timeliness of the data, which can enhance the accuracy and overall effectiveness of AI/ML positioning models.

The LMF notifies the data sample availability and information to the subscribed NG-RAN Node, dependent on its subscription criteria, e.g., the number of satisfied data samples, UEs permitting positioning, etc.

The LMF could send the available data samples to the NG-RAN Node directly.

The NG-RAN Node sends Data Collection Request to the LMF according to its need and context, the LMF shall reply with a Data Collection Response, containing the data samples.

When the LMF’s collected samples are insufficient to meet the NG-RAN Node's requirements within a specified period, the LMF can take proactive measures. This involves initiating network-triggered UE positioning after verifying permissions from nearby UEs. Multiple UEs can be positioned simultaneously to generate the required data samples for the NG-RAN Node. This new type of positioning request, termed Proactive LMF Triggered Location Request (PLT-LR). Unlike existing positioning types (e.g., NI-LR, MT-LR, MO-LR), where raw measurements or channel information are not always reported to the LMF, PLT-LR explicitly aims to collect and deliver comprehensive data samples, including both part A and part B samples, to the NG-RAN Node.

To initiate PLT-LR, the LMF retrieves information from the AMF regarding UEs served in the "Area of Interest." For each UE, the AMF consults its associated UDM to determine if the UE has privacy restrictions on location positioning. The AMF returns a list of UEs that permit positioning to the LMF.  After selecting a sufficient number of UEs that allow positioning, the LMF initiates PLT-LR for those UEs.

- Downlink Positioning Methods (e.g., DL-TDOA, DL-AoD): Multiple NG-RAN Nodes transmit Positioning Reference Signals (PRS) to the selected UEs. The UEs then send raw measurements, channel information, and calculated positioning-related statistics (e.g., TDoA, AoD) to the LMF. These data are considered a complete sample for AI/ML-assisted positioning models.

- Uplink Positioning Methods (e.g., UL-TDOA, UL-AoA): The selected UEs transmit Sounding Reference Signals (SRS) to multiple NG-RAN Nodes. Each NG-RAN Node sends the corresponding raw measurements, channel information, and calculated positioning-related statistics (e.g., TDoA, AoD) to the LMF. These data similarly form a complete sample for AI/ML-assisted positioning models.

The LMF could initiate UE positioning to provide enough data samples to the subscribing or requesting NG-RAN Nodes.

The LMF retrieves a list of UEs that permit positioning from the AMF and selects the UEs for positioning.

For downlink positioning methods, the UEs send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

For uplink positioning methods, the NG-RAN Node sends raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

In NI-LR, MT-LR, MO-LR, for downlink positioning methods, the UEs could be indicated by the LMF to send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

In NI-LR, MT-LR, MO-LR, for downlink positioning methods, the NG-RAN Node could be indicated by the LMF to send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

In the CU-DU split architecture, the gNB-CU is responsible for higher-layer functions, such as RRC and PDCP. These layers are involved in signaling procedures, including positioning-related messages.  When UE positioning is required, the gNB-CU handles communication with the LMF via control plane signaling. This typically includes exchanging messages to initiate positioning procedures or forward measurement results. The gNB-DU is typically responsible for lower-layer functions like MAC, RLC, and PHY. It performs tasks such as sending PRS or receiving uplink signals like SRS for positioning. The gNB-DU forwards measurements to the gNB-CU, which in turn communicates with the LMF.

In Section 6.3.1 of [11], it says: “﻿In case of a split gNB architecture, the NRPPa protocol is terminated at the gNB-CU.” It is our view, that AI/ML model training can be hosted within the gNB-CU, aligning with the principles outlined in the AI/ML for NG-RAN framework. It also avoids frequent F1 communication for data sample forwarding, such as from LMF to gNB-CU to gNB-DU if the model training is hosted at gNB-DU.

For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. gNB-CU involves in the above proposed procedures.

gNB-DU collects raw measurements, channel information (Part A), and calculates positioning-related statistics (e.g., TDoA, AoD) (Part B). gNB-DU sends both Part A and Part B to gNB-CU via F1 interface.

### Model Inference

It was agreed in RAN1 118 meeting [5], that the existing procedures can be reused in terms of SRS configuration for model training, monitoring and inference. Regarding the model inference output, RAN1 116 meeting [4] agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting from the gNB to the LMF. If reporting the LOS/NLOS indicator, it can be provided as a soft or hard indicator, as specified in TS 38.214 [7]. If reporting timing information, it includes at minimum the UL RTOA or the gNB Rx-Tx time difference, as specified in TS 38.215 [8]. However, the details of the reporting are pending further discussions in RAN1.

Agreement (RAN1#116) Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

Agreement (RAN1#118 Bis) [6] Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

It has agreed from the RAN3 126 meeting, the model inference at the NG-RAN side in Case 3a can reuse the current NRPPa measurement procedures [1][10]. Legacy measurement IEs can still be used as part of traditional positioning methods, while additional AI/ML-based information elements may be introduced. The LMF can then calculate the UE's position using either legacy measurements or AI/ML-based measurements, incorporating both traditional and new IEs as needed.

To track model accuracy in the model monitoring function, in our view, the LMF could provide some assistance information to the NG-RAN Node, such as the precision level or accuracy level of the final positioning based on the intermediate measurement statistics inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. Thus, it is important for the LMF be aware of that those intermediate measurement statistics actually inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. In other words, AI/ML inferred measurements are differentiated from the measurements generated in a traditional manner.

AI/ML inferred measurements are differentiated from the measurements generated in the traditional manner. They need to be marked by the NG-RAN Node as different types of measurements.

For CU/DU Split architecture, since the model training is performed at the gNB-CU, it is reasonable to think that the gNB-CU also performs the model inference. The input data for the model inference comes from TRPs, which are located in the gNB-DU. According to TS 38.470 [9], the positioning function of F1 interface enables the gNB-CU to request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

### Model Monitoring

As agreed in the RAN3 125 126 meeting [1], we turned the WA to agreement: RAN3 will proceed with Option A: NG-RAN Node performs monitoring metric calculation for its own model. However, the LMF is the entity that gathers data from multiple NG-RAN nodes and calculates the UE’s position in the final step. Therefore, it may be advantageous for the LMF to provide assistance to the NG-RAN Node in calculating the monitoring metric or making the monitoring decision. What assistance information could be provided from the LMF to the NG-RAN node is for further discussion.

For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following proposals:

- During any NI-LR, MT-LR, MO-LR, a LMF could attach its routing ID in either DL or UL positioning message. An involved NG-RAN Node could record the routing ID of the LMF.

NRPPa protocol could be designed to be initiated by NG-RAN Node, by utilizing the recorded routing ID of the LMF to reach the LMF.

The NG-RAN Node can indicate its data collection intent to one or multiple LMFs with recorded routing ID of those LMFs.

The NG-RAN Node could subscribe to the data sample availability to one or multiple LMFs with specified criteria.

The NG-RAN Node could be informed with the UEs in the neighborhood that permits positioning from the LMF, and selects the list of UEs for positioning, thus obtaining their data samples.

The NG-RAN Node could proceed with data collection request to the LMF on the on-demand basis.

The LMF notifies the data sample availability and information to the subscribed NG-RAN Node, dependent on its subscription criteria, e.g., the number of satisfied data samples, UEs permitting positioning, etc.

The LMF could send the available data samples to the NG-RAN Node directly.

The NG-RAN Node sends Data Collection Request to the LMF according to its need and context, the LMF shall reply with a Data Collection Response, containing the data samples.

The LMF could initiate UE positioning to provide enough data samples to the subscribing or requesting NG-RAN Nodes.

The LMF retrieves a list of UEs that permit positioning from the AMF and selects the UEs for positioning.

For downlink positioning methods, the UEs send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

For uplink positioning methods, the NG-RAN Node sends raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

In NI-LR, MT-LR, MO-LR, for downlink positioning methods, the UEs could be indicated by the LMF to send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

In NI-LR, MT-LR, MO-LR, for downlink positioning methods, the NG-RAN Node could be indicated by the LMF to send raw measurements, channel information (Part A), and calculated positioning-related statistics (e.g., TDoA, AoD) (Part B) to the LMF.

For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. gNB-CU involves in the above proposed procedures.

gNB-DU collects raw measurements, channel information (Part A), and calculates positioning-related statistics (e.g., TDoA, AoD) (Part B). gNB-DU sends both Part A and Part B to gNB-CU via F1 interface.

AI/ML inferred measurements are differentiated from the measurements generated in the traditional manner. They need to be marked by the NG-RAN Node as different types of measurements.

For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# References

draft_MeetingReport_RAN3#126_vDec05

3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 18)

3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2 (version 18.7.0 Release 18)

Final_Minutes_report_RAN1#116_v100

Final_Minutes_report_RAN1#118_v100

Draft_Minutes_report_RAN1#118b_v010

﻿3GPP TS 38.214, “Physical layer procedures for data,” (version 18.4.0 Release 18)

﻿3GPP TS 38.215, “Physical layer measurements,” (version 18.3.0 Release 18)

﻿3GPP TS 38.470, “F1 general aspects and principles,” (version 18.3.0 Release 18)

3GPP TS 38.455, “NR Positioning Protocol A (NRPPa),” V18.3.0 (2024-09)

- ﻿“Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN ,” 3GPP, Tech. Rep. 3GPP TS 38.305 version 18.3.0 Release 18, 2024.