3GPP TSG-RAN WG3 Meeting #127	             R3-250519

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on SA2 LS Reply on FS_XRM Ph2 Regarding Available Data Rate Reporting from RAN

Document for:	Discussions & Approval

# Introduction

SA2 provides the feedback on RAN3’s question regarding: RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting [1]:

Feedback from SA2: From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN. Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR). Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

# Discussions

According to the feedback from SA2, “available data rate” refers to the actual data rate that the RAN can currently offer for a particular QoS flow. This available data rate can vary within a defined range depending on specific conditions:

- For a QoS Flow with GFBR: The available data rate can range from the GFBR up to the MFBR if the MFBR is greater than the GFBR. This means the NG-RAN can provide a data rate that is higher than the GFBR, but it cannot exceed the MFBR, as the system is not required to allocate more resources than what the MFBR stipulates.

- For a QoS Flow with QoS Notification Control enabled and no Admission Control (AQP): The available data rate can fall below the GFBR. This is a situation where the NG-RAN may not be able to provide the full GFBR due to network conditions, but the system will still communicate these adjustments through appropriate mechanisms like QoS Notifications.

While the NG-RAN is capable of adjusting the available data rate between GFBR and MFBR, it will never exceed the MFBR, as NG-RAN is not required to provide more resources than necessary. Therefore, the available data rate is constrained by these values and will always stay within the limits defined by the system’s configuration. Periodic reporting of the available data rate is generally not useful for the application layer because, in many cases, the available data rate remains constant at the GFBR. The application is typically concerned with changes in the data rate, so frequent reporting would not add significant value in these scenarios. Threshold-based reporting is seen as more valuable for applications. In this approach, one or more threshold values are pre-configured by the application. When the available data rate crosses one of these thresholds, the NG-RAN could report this change in the uplink (UL) user plane packets. The application can then adjust its data rate or other parameters accordingly based on the reported information. To prevent excessive notifications, the NG-RAN is expected to follow a reporting logic that includes hysteresis, particularly when the available data rate crosses the thresholds. When the network condition improves (crosses to a higher threshold), there is typically a delay or some form of stabilization before a notification is sent. This is done to avoid frequent changes in resource allocation or to prevent “flapping” (frequent, minor fluctuations in available data rates causing multiple notifications). For example, when the available data rate increases beyond a threshold, NG-RAN Node may wait for the data rate to stabilize before sending a notification, reducing unnecessary signaling and overhead. However, when the available data rate crosses a threshold in one direction (e.g., going lower), it should immediately trigger an uplink report to the application.

The first few packets sent after a change in the available data rate are expected to be sufficient to notify the UPF about the new available data rate.

NG-RAN could enable user-plane notifications of the available data rate to the UPF, triggered by predefined lower and upper threshold boundaries of the available data rate.

NG-RAN shall wait for the data rate to stabilize before sending a notification when the available data rate increases beyond a higher threshold.

NG-RAN shall immediately trigger an uplink report of the available data rate, when it decreases and crosses a lower threshold.

# Conclusions

In this submission, we discussed reporting the available data rate at NG-RAN via user plane. The following proposals are made:

- NG-RAN could enable user-plane notifications of the available data rate to the UPF, triggered by predefined lower and upper threshold boundaries of the available data rate.

NG-RAN shall wait for the data rate to stabilize before sending a notification when the available data rate increases beyond a higher threshold.

NG-RAN shall immediately trigger an uplink report of the available data rate, when it decreases and crosses a lower threshold.

# References

R3-250018, LS Reply on FS_XRM Ph2