3GPP TSG-RAN WG3 Meeting #127	R3-250527

Athens, Greece, 17-21 Feb 2025

Agenda Item:	22.2

Source:	Ericsson

Title:	Authorization of Intermediate Relay UEs in RRC_INACTIVE

Document for:	Discussion, Agreement

# Introduction

In Rel-19, a Work Item for NR Sidelink Multi-hop has been established to enable multi-hop Layer-2 UE-to-Network relay through a single indirect path.

RAN2 has started work on this topic and has reached initial agreements on baseline procedures. This document discusses the potential challenges and provides an analysis of key considerations related to intermediate relay UEs.

# Background

Regarding the requirement for each intermediate relay UE to be in RRC_CONNECTED when the remote UE transitions to RRC_CONNECTED, RAN2 reached the following agreements during RAN2#128.

- Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

  - 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

  - 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

- All agreements in this WI apply to both these cases unless otherwise specified.

The current baseline requires all relay UEs to be in RRC_CONNECTED state when a remote UE initiates a connection, leading to increased signaling traffic, as well as reduced flexibility for multi-cell relay scenarios due to the restriction of relay UEs within the same serving cell.

An alternative was discussed where intermediate relay UEs could remain in RRC_INACTIVE state. However, concerns remain regarding implementation complexity and the need for explicit network authorization.

# Discussion

For RAN3, the primary consideration is how to authorize a UE in RRC_INACTIVE state to function as an intermediate relay UE. One possible solution is to authorize an intermediate relay UE in RRC_INACTIVE using pre-authorization when the UE was in RRC_CONNECTED. Additionally, the ways of authorization should enable smooth multi-hop operations by ensuring efficient relay-related data transfer between gNB-DU, gNB-CU, and neighboring gNBs.

- Authorization of intermediate relay UEs in RRC_INACTIVE should be taken into accout for multi-hop operation.

In general, two potential solutions for authorizing an intermediate relay UE in RRC_INACTIVE are outlined below:

- UE-Based Authorization: The intermediate relay UE can check its own authorization information upon receiving a discovery message from a remote UE and proceeds with relaying if authorized.

- Network-Assisted Authorization: The gNB verifies the relay UE’s authorization through stored or retrieved network data and configures the UE accordingly to enable multi-hop operation.

RAN3 should continue discussing the appropriate mechanisms for ensuring secure and efficient authorization of UEs in RRC_INACTIVE for relay operations.

- RAN3 to discuss a solution to support authorization of intermediate relay UEs in RRC_INACTIVE.

# Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1	Authorization of intermediate relay UEs in RRC_INACTIVE should be taken into accout for multi-hop operation.

In this paper we propose:

Proposal 1	RAN3 to discuss a solution to support authorization of intermediate relay UEs in RRC_INACTIVE.