3GPP TSG-RAN WG3 Meeting #127	R3-250530

# Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on Enhancements in SON/MDT Data Collection and Mobility Robustness Optimization (MRO)

Source: 	Jio Platforms

Agenda item:	10.2

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we propose enhancements in Self-Organizing Networks (SON), Minimization of Drive Tests (MDT), and Mobility Robustness Optimization (MRO) for 5G NR Standalone and NR-DC (Dual Connectivity). The proposed changes focus on improving measurement reporting, refining mobility failure differentiation, and introducing ML-based proactive optimizations.

The contribution identifies issues in the current specifications (TS 38.300, TS 38.331, TS 38.401, TS 38.423) and proposes modifications to existing procedures.

The paper presents:

- Enhancements for SON/MDT data collection and measurement reporting.

- Improvements in Mobility Robustness Optimization (MRO) for LTM and CHO/CPAC in NR-DC.

- Introduction of new Information Elements (IEs) to improve handover decisions.

- Enhance data collection for Candidate Cell Scoring for handover prediction and failure mitigation.

- Proposed changes to 3GPP procedures, message structures, and Information Elements (IEs).

# 2. Discussion

The current 3GPP specifications lack sufficient granularity in measurement reporting, failure classification, and inter-node coordination for mobility events. The following issues were identified:

## 2.1 Near-Failure LTM Scenarios and Differentiation of LTM Failure Types

Existing Issue:

The current 3GPP specifications do not provide sufficient granularity in the classification of failures occurring during Lower Layer Triggered Mobility (LTM). This limitation significantly affects network optimization, making it difficult to apply the most effective recovery and corrective actions when mobility failures occur. Specifically, there is no clear differentiation between three major failure types:

- Too Early LTM: In this scenario, the LTM process is triggered before the ideal handover conditions are met. As a result, the handover may be premature, leading to an unnecessary switch to a target cell that may not yet offer stable coverage. This can result in frequent handover failures, increased signaling load, and a degradation in user experience.

- Too Late LTM: This failure occurs when the LTM process is delayed beyond the optimal point, leading to Radio Link Failure (RLF) before the handover can be successfully completed. This type of failure can cause a sudden drop in connectivity, leading to packet loss, session interruptions, and an overall deterioration in network reliability.

- LTM to Wrong Cell: In some cases, the UE is handed over to an unintended or non-optimal target cell due to incorrect mobility decisions. This can trigger ping-pong handovers, increased mobility failures, and inefficient resource utilization within the network. The selection of a wrong target cell also increases the probability of the UE re-initiating handover attempts shortly after the first failed handover, contributing to excessive network overhead.

The lack of explicit failure classification in LTM-based mobility mechanisms prevents the network from making targeted optimizations based on failure type, thereby increasing the overall handover failure rate. This issue becomes even more critical in high-mobility scenarios, such as moving vehicles or dense urban deployments, where precise mobility execution is essential for maintaining seamless connectivity.

To improve failure detection, root cause analysis, and optimization strategies, we propose extending the existing RLF reporting framework by introducing new Information Elements (IEs) that will explicitly define different failure types and their underlying causes. These enhancements will provide the necessary granularity in failure classification, enabling the network to make proactive and intelligent handover optimizations.

Enhancements in RLF Reporting with New IEs

One of the most crucial enhancements involves modifying the RLF report structure by introducing the Candidate Cell Quality Report (CCQR) IE and the LTM Failure Type IE. These IEs will improve the visibility and traceability of LTM failures and will help the network perform a more accurate root cause analysis.

The Candidate Cell Quality Report (CCQR) IE will provide detailed information about the quality of the candidate cells during the handover process. This includes key radio metrics such as Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ), Signal-to-Interference-plus-Noise Ratio (SINR), Channel Quality Indicator (CQI). Additionally, the CCQR IE will introduce historical measurement trends, capturing variations over time rather than relying solely on instantaneous values. This will help identify whether a poor candidate cell selection was responsible for the failure, allowing future handover decisions to be optimized accordingly.

The LTM Failure Type IE will explicitly categorize failures into Too Early LTM, Too Late LTM, and LTM to Wrong Cell. By clearly identifying which type of failure has occurred, the network can apply targeted optimizations based on the root cause rather than using a one-size-fits-all recovery strategy. For example, if multiple Too Early LTM failures are detected within a short period, the network can delay the LTM trigger timing for future attempts in similar scenarios. Likewise, if a Too Late LTM failure is observed, the network can adjust the failure detection thresholds to trigger handovers earlier.

RLF-Report-r16 ::= SEQUENCE {

...

ltm-FailureType ENUMERATED {tooEarly, tooLate, wrongCell} OPTIONAL

}

Centralized vs. Distributed Failure Handling for LTM Optimization

To efficiently process LTM failures, we propose a dual-layer failure handling strategy, where the Central Unit (CU) and the Distributed Unit (DU) play distinct roles in analyzing and mitigating failures.

At the Central Unit (CU) level, the main responsibility is to perform root cause analysis for failures caused by incorrect LTM candidate cell selection. The CU will leverage Machine Learning-based Dynamic Candidate Cell Scoring (DCCS-ML) to optimize target cell selection and reduce future failures. This scoring mechanism will evaluate historical handover success rates, real-time measurement reports, and network load conditions to assign a predictive success probability score to each candidate cell. The CU will then use this Candidate Cell Score to dynamically adjust handover thresholds, ensuring that handovers occur at the most optimal time and to the most reliable target cell.

On the other hand, the Distributed Unit (DU) will focus on timing and execution-related failures, optimizing handover trigger conditions at the local level. The DU will:

- Delay the LTM trigger if multiple Too Early LTM failures are detected.

- Adjust the RLF detection threshold if excessive Too Late LTM failures occur.

- Refine target cell selection filters if multiple LTM to Wrong Cell failures are reported.

To ensure smooth coordination between CU and DU, we propose enhancing the Access and Mobility Indication (TS 38.401) to include additional failure classification IEs, which will allow the CU to forward enriched LTM failure reports to the DU for further optimization.

Proposed Enhancement:

- Reuse existing handover failure definitions from TS 38.300.

- Extend RLF Report to include:

  - Candidate Cell Quality Report (CCQR) IE

  - LTM Failure Type IE

- CU to handle root cause analysis for failures due to wrong LTM cell selection.

- Source DU to optimize failures due to wrong cell switch timing.

- Modify TS 38.300, TS 38.331, and TS 38.401 to incorporate LTM-specific failure definitions.

## 2.2 MRO for CHO with Candidate SCG Failure and Near-Failure Cases

Existing Issues and Challenges in Current 3GPP Standards

The current 3GPP specifications lack sufficient granularity in failure classification and optimization for Conditional Handover (CHO) with Candidate Secondary Cell Groups (SCG) in NR Dual Connectivity (NR-DC) scenarios. This limitation results in ineffective failure handling mechanisms, leading to increased handover failures, suboptimal cell selection, and inefficient resource allocation.

One of the key deficiencies is the absence of a structured mechanism to determine which Candidate SCG(s) met the execution conditions before failure occurred. Without this information, the network cannot distinguish between successful candidate evaluations and those that contributed to the failure. This hinders MRO functionalities, as the network lacks insights into which candidate cells should be prioritized, re-evaluated, or excluded in future handover decisions.

Additionally, CHO failures are not categorized effectively in current standards. The lack of failure-specific RLF reporting impacts root cause analysis and subsequent optimization, making it difficult for the network to determine whether a failure was caused by:

- An SCG candidate failing to meet the required CHO execution conditions.

- A late execution of CHO, causing the UE to experience Radio Link Failure (RLF).

- An incorrect selection of a Candidate PCell or PSCell, leading to a handover to a non-optimal target.

Without explicit failure categorization, network optimization strategies remain reactive rather than proactive, increasing handover failure rates, unnecessary signaling overhead, and poor user experience in CHO scenarios.

To improve failure detection, CHO failure classification, and optimization mechanisms, we propose enhancing the existing RLF reporting framework by introducing new Information Elements (IEs) to explicitly track CHO candidate evaluation status and execution conditions before failure occurred. These enhancements will enable the network to differentiate between different CHO failure types, leading to more precise and targeted mobility optimizations.

Enhancements in RLF Reporting with New IEs

One of the critical improvements involves modifying the RLF report structure to incorporate the following new Information Elements (IEs):

- Candidate PCell(s) and PSCell(s) that met CHO/CPAC conditions before failure

  - This information will help the network understand which cells had successfully met the CHO/CPAC execution criteria before failure occurred.

  - If multiple CHO candidate cells had been evaluated but none successfully met the conditions, the network can analyze and refine the CHO trigger thresholds to prevent premature failures.

- Tracking of fulfilled execution conditions before failure occurred

  - This will allow the CU/DU to trace back the sequence of CHO execution attempts and determine whether the failure was caused by late execution, an incorrect target selection, or other factors.

  - The network can then use this information to adjust CHO execution timings, improve target selection mechanisms, or refine CHO triggering conditions.

Introduction of MRO Failure Categories for CHO Candidate SCG Failures

To improve CHO failure categorization, we propose defining explicit MRO failure categories that will help in differentiating failure types based on execution conditions. These categories will be used to trigger specific recovery and optimization actions, allowing the network to adapt to various CHO failure scenarios dynamically. Enhance RLF reports to include conditionalHandoverExecutionList.

RLF-Report-r16 ::= SEQUENCE {

...

conditionalHandoverExecutionList SEQUENCE (SIZE(1..maxCHO)) OF CHO-ExecutionInfo OPTIONAL

}

CHO Failure Categorization Based on Candidate SCG Failures

| CHO Failure Case | Failure Condition | Optimization Action |
| --- | --- | --- |
| Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 | Candidate SCG(s) failed to meet execution conditions | Adjust CHO execution parameters and refine SCG selection |
| Cases 4, 5, 6 | General CHO failures (late execution, incorrect selection, etc.) | Perform network-wide CHO optimization and refine decision-making logic |

By categorizing failures explicitly, the network can dynamically adjust CHO-related thresholds, modify execution timing, and improve target cell selection accuracy, leading to better handover success rates.

Enhancements to CHO Failure Handling and Inter-Node Coordination

To ensure seamless coordination between network nodes, we propose enhancing the inter-node failure handling procedures using the F1AP and XnAP signaling mechanisms. Specifically, the SCG Failure Information (TS 38.423) should be extended to include explicit forwarding rules for CHO failure handling.

- The serving MN performs the initial failure analysis

  - The serving Master Node (MN) will be responsible for analyzing CHO failure causes and determining the root cause before forwarding the failure report to other nodes.

  - This prevents unnecessary signaling overhead by filtering irrelevant failure data at the initial stage.

- The serving MN forwards SCG Failure Information to the respective MN

  - If an SCG failure is detected, the serving MN will forward the SCG Failure Information to the MN responsible for the failed execution condition.

  - The responsible MN will then perform additional failure analysis and execute corrective optimizations to prevent future failures.

By implementing these enhancements, we can ensure better failure traceability, optimize inter-node coordination, and reduce the overall failure rate for CHO-based mobility events.

## 2.3 Inadequate Proactive Failure Prevention

Existing Issue

The current 3GPP standards lack predictive mechanisms to detect and mitigate failures before a Radio Link Failure (RLF) occurs. The existing measurement reporting framework primarily focuses on instantaneous radio conditions, such as Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ), and Signal-to-Interference-plus-Noise Ratio (SINR). However, these metrics alone are insufficient to predict a failure before it occurs.

Due to this limitation, network optimization primarily occurs after an RLF is detected, leading to:

- Higher Mobility Failures – Networks react to failures instead of proactively mitigating them.

- Suboptimal Handover Decisions – Since no preemptive thresholds exist, the network lacks insight into early degradation signs that could help trigger preventive mobility actions.

- Poor Quality of Experience (QoE) for Users – The system fails to adjust mobility parameters dynamically, leading to unnecessary handovers, service disruptions, and data session drops.

- Inefficient Resource Utilization – The lack of failure anticipation results in excessive signaling overhead, wasted bandwidth.

While Self-Organizing Networks (SON) and Minimization of Drive Tests (MDT) provide basic failure detection mechanisms, they do not include real-time predictive analytics to detect near-failure conditions. As a result, handover decisions remain static and reactive, leading to suboptimal network performance.

Proposed Enhancement:

- Introduce Near-Failure Threshold IE → Detects critical conditions and triggers network optimization before failure occurs.

To address these limitations, we propose the introduction of a new Information Element (IE) called "Near-Failure Threshold IE". This IE will allow proactive failure detection by enabling the CU and DU to identify early warning signs of potential mobility failures based on historical and real-time data.

Key Features of Near-Failure Threshold IE:

- Monitors Multiple Failure Metrics:

- RSRP, RSRQ, SINR degradation trends.

- Increased HARQ retransmissions.

- RACH access failures and reattempts.

- High interference fluctuations or link quality degradation.

Detects Near-Failure Conditions:

- Establishes threshold values for each failure metric.

- Triggers network optimizations before an RLF occurs.

- Enables Predictive Mobility Optimization:

- Allows preemptive handovers to be initiated.

- Reduces signaling overhead by limiting unnecessary handovers.

## 2.4 Enhancements for RACH Information in LTM

Existing Issue

The current 3GPP specifications do not differentiate between RACH-based and RACH-less Lower Layer Triggered Mobility (LTM) handovers. This limitation impacts mobility robustness optimization (MRO) in LTM, as failed handovers due to RACH-related issues are not explicitly reported.

Key challenges include:

- No Explicit Differentiation Between RACH-Based and RACH-Less LTM Handovers

  - Existing measurement reports do not indicate whether the Random Access Channel (RACH) procedure was triggered during LTM.

  - Networks cannot optimize LTM execution based on RACH success/failure trends.

- Absence of RACH-Specific Failure Information

  - When an LTM failure occurs, the network does not receive details on whether:

    - The RACH attempt was made.

    - The RACH procedure failed due to contention resolution timeout.

    - The UE performed a RACH-less handover but failed due to synchronization loss.

- Limited Optimization Capability in RACH-Related Failures

  - The gNB-CU cannot determine if the failure was due to:

    - RACH failure at the target cell (contention-based or contention-free).

    - Incorrect LTM trigger timing, causing unsuccessful RACH.

  - Without these insights, Mobility Robustness Optimization (MRO) cannot adaptively tune RACH parameters.

Proposed Enhancement:

To address these issues, we propose enhancements in measurement reporting and failure logging for RACH-based LTM failures.

- Introduce RACH Access Type IE

A new RACH Access Type IE will be introduced to capture:

- Whether RACH was used for LTM.

- RACH attempt count and success/failure status.

- RACH access type (Contention-based vs. Contention-free).

- Failure reason classification (e.g., Random Access Response timeout, contention resolution failure).

This new IE will allow the CU to distinguish between RACH and RACH-less LTM failures and optimize handover procedures accordingly.

- Modify RLF Reports to Include RACH Information

Enhance Radio Link Failure (RLF) Reports to contain:

- LTM attempt details (Was RACH used? If so, how many attempts?).

- RACH success/failure reasons.

- Time elapsed between LTM trigger and RACH failure.

This will allow the gNB-CU to:

- Identify patterns in RACH failures for LTM handovers.

- Fine-tune RACH-related parameters for future handovers.

- Update Measurement Reporting for RACH-LTM Analysis

Modify Measurement Reports (TS 38.331) to log:

- RACH-specific success/failure information.

- RACH preamble format and configuration used during LTM.

- Sync loss detection if RACH-less LTM was attempted but failed.

These enhancements will provide better visibility into RACH failures, enabling adaptive LTM handover strategies.

## 2.5 UHI and Ping-Pong Mitigation

Existing Issue

The current 3GPP mobility management framework does not explicitly define a responsibility model for handling LTM ping-pong handovers. This lack of clarity results in the absence of a standardized detection and mitigation strategy for excessive and unnecessary handovers between cells.

- Uncontrolled LTM Cell Switching (Ping-Pong Effect)

  - In scenarios where a UE rapidly oscillates between two or more cells, there is no mechanism to detect and flag these repetitive LTM handovers.

  - Current RLF reports do not capture excessive handovers in a given timeframe, making it difficult to optimize mobility decisions.

- No Explicit Decision Authority Between CU and DU for Handling LTM Ping-Pong Events

  - CU and DU both participate in mobility decision-making, but the standard does not clearly define:

    - Which node should detect frequent handovers and take corrective action.

    - Whether CU should override DU-based mobility triggers in case of excessive LTM events.

- Inefficient Network Resource Utilization

  - Frequent, unnecessary handovers increase signaling overhead and resource consumption, leading to degraded network efficiency.

  - The lack of an explicit control mechanism prevents the network from preventing excessive LTM handovers.

Proposed Enhancement: Introduction of Ping-Pong Detection IE and Unnecessary Handover Indication (UHI)

To address these issues, we propose the following enhancements to detect, report, and mitigate LTM ping-pong handovers.

1. Introduce Ping-Pong Detection IE

A new Ping-Pong Detection IE will be introduced to:

- Monitor UE handover history and identify excessive LTM handovers within a short timeframe.

- Log timestamped events to track repetitive cell switches between the same set of cells.

- Provide statistical trends on handover attempts within a specific time window, allowing the CU to make smarter mobility decisions.

2. CU to Send Unnecessary Handover Indication (UHI) to DU

- If the CU detects excessive LTM handovers based on Ping-Pong Detection IE, it will:

  - Generate a Unnecessary Handover Indication (UHI) message.

  - Send the UHI to the DU, instructing it to pause or deprioritize handover triggers for the affected UE.

- This will prevent the DU from initiating back-to-back handovers when the ping-pong threshold is exceeded.

3. Modify TS 38.401 and TS 38.423 to Handle Ping-Pong Mitigation

- TS 38.401 (F1AP Signaling):

  - Extend Access and Mobility Indication Procedure to include Ping-Pong Detection IE.

  - Add new UHI signaling for mobility coordination between CU and DU. UHI as a new F1AP message in TS 38.401

UEContextModificationRequired (UHI) ::= SEQUENCE {

...

unnecessaryHO-Indication ENUMERATED {true}

}

- TS 38.423 (XnAP Signaling):

  - Modify Handover Request Acknowledgment message to include Ping-Pong Detection IE.

  - Improve Inter-gNB handover procedures for proactive ping-pong mitigation.

# 3. Proposal for Specification Changes

| Technical specification | Proposed Change |
| --- | --- |
| TS 38.300 | Add LTM-specific failure definitions. |
| TS 38.331 | Introduce CCQR IE, LTM Failure Type IE, RACH Access Type IE. |
| TS 38.401 | Enhance Access and Mobility Indication messages. |
| TS 38.423 | Extend SCG Failure Information reporting. |
| TS 37.340 | Introduce CPAC execution failure definitions. |

- Conclusion

Proposal 1: The current 3GPP specifications lack detailed classification of Lower Layer Triggered Mobility (LTM) failures, leading to inefficient handover recovery. This proposal introduces Candidate Cell Quality Report (CCQR) IE and LTM Failure Type IE to explicitly categorize failures into Too Early LTM, Too Late LTM, and LTM to Wrong Cell. The CU will analyze failures due to incorrect LTM candidate selection, while the DU will optimize handover execution timing to minimize failures.

Proposal 2: Existing specifications do not differentiate between successful and failed Conditional Handover (CHO) candidate Secondary Cell Groups (SCG). This proposal enhances the RLF Report to track which Candidate PCell(s) and PSCell(s) met CHO/CPAC conditions before failure. New failure categories will help in refining Mobility Robustness Optimization (MRO) for CHO failure cases, ensuring better handover decisions.

Proposal 3: There is no predictive mechanism in 3GPP standards to detect near-failure conditions before a Radio Link Failure (RLF) occurs. This proposal introduces a Near-Failure Threshold IE, which will allow networks to detect deteriorating radio conditions in advance and trigger proactive mobility optimizations. The system will analyze RSRP, RSRQ, SINR trends, and RACH failures to optimize handover decisions before failures occur.

Proposal 4: The current LTM handover process does not differentiate between RACH-based and RACH-less handovers, leading to inefficient failure handling. This proposal introduces a RACH Access Type IE that captures whether LTM used RACH, the number of attempts, and failure reasons. Enhancements in TS 38.300 and TS 38.331 will improve the network’s ability to adaptively optimize RACH-related parameters in LTM.

Proposal 5: There is no clear responsibility assignment between CU and DU for handling excessive LTM-based handovers, leading to frequent ping-pong effects. This proposal introduces a Ping-Pong Detection IE, which will track repeated handovers within a short timeframe. When excessive LTM events are detected, the CU will send a Unnecessary Handover Indication (UHI) to the DU to deprioritize unnecessary handovers. Modifications are proposed for TS 38.401 and TS 38.423 to enhance ping-pong mitigation.