3GPP TSG-RAN WG3 Meeting #127	R3-250534

Athens, Greece,  17 – 21 February, 2025

Agenda item:	17.3

Source:	Nokia

Title:	Xn and F1 signalling for On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

First baseline CRs for XnAP and F1AP were endorsed at RAN3#126, however both CRs are entirely marked as FFS. We here discuss how to progress and stabilize these CRs.

# 2	Discussion

## 2.1 XnAP

We here list RAN3’s agreements relevant for the XnAP BLCR (numbering added by us for the purpose of this discussion):

- [1] UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- [2] Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

- [3] The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- [4] Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [5] The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

- [6] The procedure uses non UE-associated signaling.

- [7] Cell A stores the UL WUS configuration information after it has received it.

- [8] A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [9] The procedure uses non UE-associated signalling.

The current name of the XnAP class 1 procedure is UL WUS Configuration Provision (FFS), which could be slightly renamed into UL WUS Configuration Provisioning.

Proposal 1: Rename the XnAP class 1 procedure into UL WUS Configuration Provisioning.

One agreed purpose of the procedure is to transfer the UL WUS configuration information (agreement 5). Two open points correspond to the additional requests ‘discontinue’ and ‘stop’:

- “Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

- “Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)”

The functionality discussed at last meeting was to enable NG-RAN node 1 to request broadcast, pause, resume and cancellation of the UL WUS configuration. This functionality may be obtained using the following three codepoints:

- broadcast: a previously stored UL WUS configuration is requested to be broadcast by NG-RAN node 2, or an UL WUS configuration provided in the message is requested to be stored and broadcast by NG-RAN node 2;

- no broadcast: the UL WUS configuration is requested not to be broadcast, but kept stored in NG-RAN node 2;

- cancel: a previously stored UL WUS configuration is requested to be discarded in the NG-RAN node 2, and corresponding broadcast shall stop.

Proposal 2: Include the following codepoints in the UL WUS CONFIGURATION PROVISION REQUEST message: broadcast, no broadcast, cancel.

Furthermore the UL WUS CONFIGURATION PROVISION RESPONSE message should include confirm and reject codepoints as given as example in agreement 2.

Proposal 3: Include the following codepoints in the UL WUS CONFIGURATION PROVISION RESPONSE message: confirm, reject.

Because both involved NG-RAN nodes may serve multiple cells, while each cell will only use a single UL WUS configuration, we suggest to use the NCGI of the NES cell in order to identify the UL WUS Configuration within XnAP and F1AP signalling procedures.

Proposal 4: Use the NCGI of the NES cell in order to identify the UL WUS Configuration within XnAP and F1AP signalling procedures.

The current name of the XnAP class 2 procedure is UL WUS Configuration Transmission Status Update (FFS), which we propose to confirm.

Proposal 5: Confirm the name of the XnAP class 2 procedure to be UL WUS Configuration Transmission Status Update.

Agreements 4, 8 and 9 apply to this procedure, which means that at current stage the codepoint “no broadcast” should be included in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message, meaning that the NG-RAN node 2 doesn’t broadcast the UL WUS Configuration.

Proposal 6: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Furthermore, RAN3 has not yet taken any agreement on whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 7: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

## 2.2 F1AP

The following agreements are relevant for the XnAP BLCR:

(10) Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

This agreement is covered by the F1AP BLCR that was agreed at last meeting. By the agreed signalling, the CU-CP may trigger the UL WUS Configuration Provisioning procedure over Xn. When the CU-CP can confirm successful response from the required  cell As (both intra-gNB scenario and inter-gNB scenario should be supported), it should inform the gNB-DU so that it can activate the OD-SIB1 mode of operation. A simple solution for this could be to include an indicator in the Cells to be Activated List Item IE. However, in order to also support ‘discontinue’ and ‘stop’ scenarios, as discussed in section 2.1, we suggest that this indicator contains two codepoints e.g. as follows:

| >>SIB1 Operation Mode | O |  | ENUMERATED (on demand, periodic, …) | SIB1 operation mode as described in TS 38.300 [4]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

This indicator should be included within the Cells to be Activated List Item IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 8: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 9: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic

# 3	Conclusion

We have made the following proposals:

Proposal 1: Rename the XnAP class 1 procedure into UL WUS Configuration Provisioning.

Proposal 2: Include the following codepoints in the UL WUS CONFIGURATION PROVISION REQUEST message: broadcast, no broadcast, cancel.

Proposal 3: Include the following codepoints in the UL WUS CONFIGURATION PROVISION RESPONSE message: confirm, reject.

Proposal 4: Use the NCGI of the NES cell in order to identify the UL WUS Configuration within XnAP and F1AP signalling procedures.

Proposal 5: Confirm the name of the XnAP class 2 procedure to be UL WUS Configuration Transmission Status Update.

Proposal 6: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Proposal 7: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 8: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 9: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic