3GPP TSG-RAN WG3 Meeting #127	R3-250535

Athens, Greece,  17 – 21 February, 2025

Agenda item:	17.4

Source:	Nokia

Title:	Energy saving indicator from AMF and adaptation of common signals/channels

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss SA2’s request relative to transfer of a subscription-based energy saving indicator from the AMF to the NG-RAN [1].

We furthermore provide initial discussion for the RAN3 work on the following parts of the NES WID [2]:

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

# 2	Discussion

## 2.1	NGAP Energy Saving Indicator

In their LS [1], SA2 request RAN3 to provide feedback on the Energy Saving Indicator that may be provided from AMF to NG-RAN. SA2 has attached an approved CR implemented in TS 23.501 v. 19.2.1 as copied below including Editor’s notes (EN). It is further clear that SA2 will need to update this specification text by CR at least to remove the ENs, taking into account RAN3’s reply to their LS.

It is understood that Subscription data for a UE has been enhanced with a new Energy Saving Indicator which is forwarded by the AMF to the PCF. Whether the Energy Saving Indicator is additionally forwarded to the NG-RAN is dependent on the feedback to the LS.

One possible alternative solution to convey energy saving policy from the CN to the NG-RAN without using an explicit Energy Saving Indicator could be to use the RFSP index. However, this approach would come with the risk of interfering with the basic RRM policy. We believe that energy saving implementations in the NG-RAN node may or may not require coordination between the applied energy saving policy and the applied RRM policy, and use of the RFSP index will therefore not provide sufficient implementation freedom for energy saving techniques in the NG-RAN.

Observation 1: Use of RFSP index as alternative to an explicit Energy Saving Indicator will not provide sufficient implementation freedom for energy saving techniques in the NG-RAN.

A further alternative for energy saving policy per UE could be based on slicing. E.g. for an eMBB SST (Slice Service Type) we can add an SD (Slice Differentiator) field for one (or more) for energy saving behaviours. This can be implemented based on today’s specification.

With such solution, from slice configuration point of view, the Area of Service of the equivalent slices with SD(s) for energy saving and with other SDs that do not give energy saving (or no SD) should be the same. This means that support upon initial access, as well as mobility, will be the same for these two categories of slices (normal slice, energy saving slice(s)).  So e.g. the handover would succeed (or fail) equally for each type of slice identified by the different SD value if target gNB (or cell) does (or does not) support this set of slices. This approach also gives full flexibility in terms of configuration of the behaviour of the NG-RAN node, because the configuration of supported slices in an NG-RAN node can point to different behaviours in different nodes.

From UE subscription management point of view, we expect that subscription changes for the Energy Saving Indicator parameter proposed by SA2 will not be too frequent, which means that slice subscription update for the UE can be performed when changes are needed at the same frequency.

Based on this, we propose to reply back to SA2 that RAN3 sees that the use of S-NSSAI is sufficient and no new indicators are needed over NGAP, i.e. the subscribed slices behaviour will determine the RAN behaviour also for energy. In addition, this allows per slice behaviour granularity (if only a subset of the subscribed slices S-NSSAIs is linked to energy saving behaviour in the RAN) or UE level granularity (i.e. when all slices use SD values that are associated to energy saving behaviour).

Proposal 1: RAN3 sees that the use of S-NSSAI is sufficient and no new indicator is needed over NGAP, i.e. the subscribed slices behaviour will determine the RAN behaviour also for energy.

## 2.2	Adaptation of SSB in time domain

The adaptation of SSB in time domain for NES has been in discussions in RAN1.

The following is one of the agreements from RAN1#118.

For adaptation mechanism(s) of SSB in time-domain,

•	For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

•	FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

•	FFS: Adaptation for SSB not on sync raster is supported (A3)

•	For Rel-19 NES-capable UE’s SCell

•	Adaptation of SSB configured for the SCell is supported for the following cases

•	FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

•	Adaptation for SSB that is not CD-SSB on sync raster (B2’)

•	Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Among these, the highlighted scenario (B1) could have some potential RAN3 impacts. RAN3 should monitor the progress of this particular use case and also any agreements from RAN2 (where the SSB adaptation related discussions are just starting).

Proposal 2: RAN3 continue to monitor progress in RAN1 and RAN2 to identify any requirements in SSB adaptation in time domain.

## 2.3	Adaptation of PRACH in time domain

In previous RAN1 meetings, a list of candidate mechanisms for PRACH adaptation in time domain was discussed. The agreed mechanism is based on additional PRACH resources configured and activated for NES-capable UEs only when needed, hence allowing the network to enter longer sleeping periods in low load periods.

Agreement from RAN1#116 (Feb. 2024):

For adaptation of PRACH in time-domain, support at least the following:

-	Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

Observation 2:

For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

In subsequent RAN1 meetings, the mechanism (with the associated details) to support the additional PRACH resources for NES-capable UEs has been discussed. We assume that from configuration perspective the OAM will configure the new additional PRACH resources in the gNB-DU.

Proposal 3: OAM configures the new additional PRACH resources to the gNB-DU

We also observe that RAN1 has designed the additional PRACH resources to be activated or deactivated in a dynamic manner. RAN3 needs to determine which entity that decides this activation and deactivation. From our side we believe this can be done autonomously by the gNB-DU.

Proposal 4: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

RACH optimization was introduced in Rel-16, based on Rel-15 PRACH resource definition. RAN3 is tasked by the Rel-19 NES WID [1] to enable NG-RAN support of the new Rel-19 PRACH resources. Support of RACH optimization is part of the NG-RAN functionality, and it therefore seems that RAN3 within the present work item should evaluate whether there is impact on the RACH optimization feature and determine enhancements needed, if any.

At the present stage, we observe that in the legacy case the gNB-CU has the full view of the PRACH resources configured in the gNB-DU, and these resources are considered to be statically allocated and activated. Information about these resources may today also be transferred over F1 to gNB-DUs serving neighbour cells, and transferred over Xn to neighbour gNBs. But, due to dynamic activation and deactivation of the new additional PRACH resources defined in Rel-19, the network entities in charge of RACH optimization (gNB-CU, gNB-DU) may, based on the existing information exchange, have incomplete information available about the PRACH resources actually used in different cells.

We therefore propose that RAN3 further works on the analysis of impact of Rel-19 additional PRACH resources on the legacy RACH optimization feature.

Proposal 5: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

## 2.4	Adaptation of paging occasions

RAN1 and RAN2 have made the following key agreements on adaptation of paging occasions:

- The solution option b) – where the paging frame interval is extended – has been agreed as the baseline solution to be worked upon

- RAN2 to aim for signaling overhead minimization

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration

- Legacy UE is not barred

The agreed option b) can be illustrated as in Figure 1:

Figure 1: Illustration of adaptation of paging occasions based on extension of the paging frame interval as per RAN1’s agreement.

As can be observed, this solution targets to increase energy savings by extending the paging time interval, thereby allowing for paging resource configuration over a longer period of time. While this is desirable from the energy savings perspective, it also has the effect to reduce the resources allocated for paging. In a legacy network, paging resources will be dimensioned in order to reasonably support periods of high load. If for some reason the paging load becomes particularly high in a given cell or beam for a given Paging Occasion (PO), some of the paging messages will not be transmitted in that PO and in case the UE is located in that cell or beam the network will have to escalate the paging. However, although paging escalation is based on implementation dependent strategies, at one point the mechanism requires extension of the paging to wider areas (additional cells/nodes) so that the full registration area of the UE can eventually be paged. Paging escalation is therefore resource consuming and should be avoided as far as possible, and this aspect is therefore given much attention in deployed networks.

However, the need for network energy saving is equally important, and the reduction of POs based on  extension of the paging frame interval as per RAN1’s agreement allows to increase sleep opportunities for the transmitter (due to reduced number of POs), as well as for the receiver due to reduced number of PRACH resources as illustrated in Figure 1. It can be noted that as per RAN2’s agreement “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration”, and an energy saving action based on paging adaptation will therefore unavoidably create similar risk for paging overload (and hence skipped paging messages) in periods of low paging load as the risk currently existing in high-load periods in legacy networks.

RAN3 will therefore first need to determine the entity to be in charge of the decision of activation and deactivation of adaptation of the POs for the purpose of NES (paging energy saving). In non-split architecture this could be the OAM or the gNB. We here assume that the gNB will need to be able to at least take decision of deactivation of paging energy saving in order to cope with situation of paging overload as fast as possible. And in order to optimize opportunities for micro-sleep modes, the gNB then also needs to take the decision of activation of paging energy saving. Cell level granularity seems to us to be the good granularity for this decision, hence enabling same paging capacity in all beams within the cell which is needed e.g. for escalated paging. We here derive the following principles for paging energy saving:

Proposal 6: Paging energy saving is done on a per cell granularity

Proposal 7: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

In split architecture, the gNB-DU will be the entity that can determine potential energy saving gains from paging energy saving. Both the gNB-CU and the gNB-DU know the current paging load. The gNB-CU has a better knowledge to determine upcoming paging load based on information about UEs in RRC_Inactive for which it is anchor gNB, and RAN paging represents a significant part of the paging load in deployed networks. Of course, still the gNB-CU will just transparently forward Paging requests for CN paging and RAN paging requests received over Xn. Also, entering/leaving paging energy saving mode will not be instantaneous but rely on the delay for update of SIB1. Based on this it seems that the gNB-DU could take the decision to enter/exit paging energy saving mode, but the gNB-CU should be aware of this mode in order to take the energy saving mode into account in its paging strategy and paging escalation strategy for RAN paging. It therefore seems the following principles can be taken as base-line for paging energy saving in split architecture:

Proposal 8: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

On top of this, in order to enable more precise handling of gNB-CU decided paging escalation for RAN paging, the gNB-DU could inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

Proposal 9: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: Use of RFSP index as alternative to an explicit Energy Saving Indicator will not provide sufficient implementation freedom for energy saving techniques in the NG-RAN.

Proposal 1: RAN3 sees that the use of S-NSSAI is sufficient and no new indicator is needed over NGAP, i.e. the subscribed slices behaviour will determine the RAN behaviour also for energy.

Proposal 2: RAN3 continue to monitor progress in RAN1 and RAN2 to identify any requirements in SSB adaptation in time domain.

Observation-2: For PRACH adaptation, two types of PRACH resources will be defined in a cell:

•	Legacy (Rel-15) PRACH resources (minimum set of PRACH resources) – always activated

•	Additional PRACH resources (new in Rel-19 for NES-capable UEs) – activated dynamically

Proposal 3: OAM configures the new additional PRACH resources to the gNB-DU

Proposal 4: Activation and deactivation of additional PRACH resources to be decided autonomously by the gNB-DU.

Proposal 5: RAN3 to further work on the analysis of impact on Rel-19 additional PRACH resources on the legacy RACH optimization feature

Proposal 6: Paging energy saving is done on a per cell granularity

Proposal 7: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB takes the decision to enter paging energy saving mode

- The gNB takes the decision to resume regular paging operation (exit paging energy saving mode)

Proposal 8: The following principles are taken as base line for paging energy saving in non-split architecture:

- The gNB-DU takes the decision to enter paging energy saving mode and informs the gNB-CU.

- The gNB-DU takes the decision to resume regular paging operation (exit paging energy saving mode) and informs the gNB-CU.

Proposal 9: RAN3 to discuss whether the gNB-DU should inform the gNB-CU about Paging messages that could not be transmitted over the air due to shortage of paging resources.

# References

[1]		R3-250024 – S2-2413034, LS on energy saving indication from CN to RAN, SA2

[2]		RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105