3GPP TSG-RAN WG3 #127	R3-250546

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.1

Source:	Ericsson

Title: 	Support UAV UE Flight Information Reporting to CN

Document for:	Discussion and Approval

# 1	Background

In the incoming LS [1],  SA2 stated that:

SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19. Availability of such information is applicable to any UAV UE(s) that support the relevant procedures.

ACTION: 	SA2 kindly asks RAN3 to take above information into consideration.

This paper discusses how to support the SA2 request in RAN3, and provide our feedback to SA2.

# 2	Discussion

## 2.1	Aerial information reporting from NG-RAN to CN

In TS 23.256, SA2 specified in the “Pre-flight Planning and In-flight Monitoring for UAVs” how to Reporting UE's altitude information:

Option B: Node-level signalling. 1a-2b. The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported. NOTE 2: Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23]. The NG-RAN node includes the received UE's altitude information in an N2 message to the AMF. 3b. The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request. 4. The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

Our understanding of the feature is that NG-RAN node based on the existing aerial UE measurement reporting to build the aerial information to be reported from NG-RAN to CN. In TS 38.300, it is specified that:

An Aerial UE can be configured with altitude-dependent, event-based measurement reporting (i.e., eventH1 and eventH2 as defined in TS 38.331 [12]). An Aerial UE sends a measurement report when its altitude becomes higher or lower than configured threshold. The UE includes its altitude and location information in the measurement report if configured to do so by NG-RAN. RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured. The Aerial UE can also be configured to trigger measurement reporting only when both an altitude-dependent condition and an RSRP/RSRQ/SINR-based condition are met (i.e., eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1 and eventA5H2 in TS 38.331 [12], commonly denoted as eventAxHy). For the content of eventAxHy measurement report, the same rules as described above for eventH1 and eventH2 apply.

We can conclude that NG-RAN node receives the Aerial UE Altitude Information in the measurement, Thus it can be a Mandatory IE to report to CN when it is requested by the CN.

SA2 mented Time Stamps. The UE does not include any Time Stamps in the measurement. However NG-RAN node could mark a time stamp when the measurement report is received over RRC. Therefore we could provide the Time Stamps as well as Mandatory IE to CN.

Related to the NGAP procedure to use, we propose to reuse the “Location Reporting Procedure”.

- In Location Reporting Control, CN will provide the thresholds together with the request to the Aerial UE. CN may at any time Stop the report request.

- In Location Report, NG-RAN node will include the Aerial UE Altitude Information and Time Stamps when the condition is met to send the report. NG-RAN node stops reporting to CN when the “stop” request is received.

- In Location Reporting Failure Indication, NG-RAN node will inform CN that the aerial UE report request has failed or stopped. To be explicit, we propose an “UAV UE flight information reporting failed” indicator with Code Point “Stopped, …”, to differ from the existing other location reporting.

## 2.2	Related to UAV Velocity

## UAV Velocity: this information is not a part of UAV measurement reporting. Therefor we propose to reply to SA2 that the UAV Velocity will not be part of the reporting.

## MDT could report velocity through it need UE consent. Other positioning procedure can also contain such information.

## We could reply to SA2 that NG-RAN node could not provide UAV Velocity info as it is not in the Aerial UE measurement. RAN understanding that this information can be obtained from the position procedure. Please SA2 consider that the UAV Velocity is not needed to be include in the N2 reporting.

Proposal 1: RAN3 to discuss and agree that the UAV Altitude and Time Stamps information can be provided to CN for the Aerial UE.

Proposal 2: RAN3 to discuss and agree to use the existing Location Reporting procedure to start the Aerial UE flight information reporting, from CN, and to stop the reporting from both CN and NG-RAN (failure).

Proposal 3: RAN3 to reply to SA2 that the UAV UE altitude can be provided, the Time Stamp is when gNB receives the UAV UE measurement reporting and UAV Velocity is not included.

The CR for NGAP/XnAP and the Draft Reply LS are submitted in [2], [3], ]4]

# 3	Proposals

Proposal 1: RAN3 to discuss and agree that the UAV Altitude and Time Stamps information can be provided to CN for the Aerial UE.

Proposal 2: RAN3 to discuss and agree to use the existing Location Reporting procedure to start the Aerial UE flight information reporting, from CN, and to stop the reporting from both CN and NG-RAN (failure).

Proposal 3: RAN3 to reply to SA2 that the UAV UE altitude can be provided, the Time Stamp is when gNB receives the UAV UE measurement reporting and UAV Velocity is not included.

The CR for NGAP/XnAP and the Draft Reply LS are submitted in [2], [3], ]4],

# 4	Reference

[1]	R3-250022	Reply LS on UAV regulation				SA2(Samsung)

[2]	R3-250541	Support UAV UE Flight Information Reporting to CN	Ericsson

[3]	R3-25xxxx	Support UAV UE Flight Information Reporting to CN	Ericsson

[4]	R3-250545	[Draft] Reply LS on UAV regulation			Ericsson