3GPP TSG-RAN WG3 Meeting #127	R3-250562

Athens, Greece, Feb. 17th – Feb. 21st, 2025

Agenda item:	20.2

Source: 	Jio

Title: 	AI/ML-Assisted Positioning

Document for:     Discussion and Decision

# 1.	Introduction

The objectives for AI/ML positioning within the Work Item on artificial intelligence and machine learning for the NR air interface are as follows [1]:

Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

- Direct AI/ML positioning:

(1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

(2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

- Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

- Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Consider aggregated gNB case first, then split architecture.

The existing LCS framework is re-used for AI/ML positioning.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

RAN3#126:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

Check progress in RAN1 and solution discussion…

RAN3 to discuss Cases 3a/3b are NG-RAN node assisted/network-based positioning methods.

# 2.	Discussion

# 2.1 Unified Data Collection & Measurement Reporting

Existing contributions have established that for AI/ML-based positioning:

- Case 3a: The NG-RAN node (gNB) hosts the AI/ML model, collecting measurement data (Part A) from TRPs, while ground truth labels (Part B) are provided by the LMF.

- Case 3b: The LMF hosts the AI/ML model and directly infers UE position using measurement data reported by NG-RAN nodes.

Building on these concepts, we propose a unified data collection procedure that supports both cases. In this framework, the measurement reporting is made adaptive:

- Adaptive Reporting Mechanism: NG-RAN nodes dynamically adjust parameters (such as the sampling rate, quality thresholds, and reporting periodicity) in response to changing network conditions (e.g., traffic load, interference levels, or UE mobility).

- Enhanced Negotiation via NRPPa: The negotiation process between NG-RAN nodes and the LMF is extended to include additional Information Elements (IEs), which can cover parameters like Nt, Nt′, k, as well as optional fields for phase and power quality.

- Unified Data Sample Structure: A standardized sample is proposed that combines:

  - Part A: Channel measurements (timing, phase, power, and quality indicators) with precise timestamps.

  - Part B: Ground truth labels or approximations with associated quality indicators and timestamps.

This unified approach not only streamlines data collection for both model training and inference but also ensures backward compatibility with legacy measurement reports (e.g., “Time Stamp”, “Measurement Quality”, LOS/NLOS indicators).

Proposal 1:

- Unified Adaptive Data Collection Procedure:
Develop and standardize a procedure that supports adaptive measurement reporting across both Case 3a and 3b. This includes:

  - A unified data sample structure (covering both Parts A and B).

  - An enhanced NRPPa interface that allows dynamic negotiation and adjustment of measurement parameters.

AI-ML-MeasurementReport ::= SEQUENCE {

timestamp                     INTEGER (0..MAX),

TRP-ID                          PhysCellId,  -- Reference to gNB

UE-ID                           INTEGER (0..MAX), -- Anonymized

channelMeasurements             SEQUENCE SIZE (1..maxMeasurements) OF ChannelMeasurement,

groundTruthLabel                GroundTruth OPTIONAL,

measurementQuality              ENUMERATED {excellent, good, fair, poor},

...

}

ChannelMeasurement ::= SEQUENCE {

signalType                      ENUMERATED {SSB, CSI-RS},

referenceSignalID               INTEGER (0..MAX),

timingInfo                      INTEGER (0..MAX),  -- e.g., TDoA

phaseInfo                       INTEGER (0..MAX),

powerInfo                       INTEGER (-150..50), -- dBm

qualityIndicator                INTEGER (0..100),  -- SINR

LOS-NLOS-Indicator              BOOLEAN,

...

}

GroundTruth ::= CHOICE {

positionInformation           GeographicalCoordinates,

...

}

GeographicalCoordinates ::= SEQUENCE {

latitude                        INTEGER (-9000000..9000000),  -- Microdegrees

longitude                       INTEGER (-18000000..18000000), -- Microdegrees

altitude                        INTEGER OPTIONAL -- Meters

}

2.2 Hybrid Model Performance Monitoring

Past proposals have segmented performance monitoring responsibilities:

- In Case 3a, the NG-RAN node is expected to monitor its own model performance.

- In Case 3b, the LMF is responsible for model inference and monitoring.

To further improve responsiveness and ensure accurate positioning, we propose a hybrid monitoring approach that leverages both local and centralized assessments:

- Local (On-Node) Monitoring: The gNB continuously monitors performance using locally collected measurement data. This includes calculating deviations in timing or LOS/NLOS indicators and triggering local re-training actions if necessary.

- Centralized (LMF) Monitoring: The LMF evaluates overall positioning accuracy by comparing inferred positions against ground truth from dedicated sources. This centralized perspective can then provide feedback to NG-RAN nodes when model performance degrades.

An integrated feedback loop is suggested, where the LMF sends periodic performance reports via an enhanced signaling channel. NG-RAN nodes combine these reports with their local metrics to decide on potential model updates or re-training.

Proposal 2:

- Implement a dual-level monitoring system in which:

- Dual-Level Monitoring:

  - Local (gNB): Monitors deviations in metrics (e.g., timing, LOS/NLOS) to trigger retraining.

  - Centralized (LMF): Validates accuracy using ground truth and provides feedback to gNBs.

- Feedback Loop: LMF sends periodic performance reports to gNBs to improve model reliability.

3.3 Enhnranced Security & Privacy Mechanisms

The transmission of measurement data and ground truth labels involves sensitive positioning information that must be securely managed. To address security and privacy:

- Data Encryption & Integrity: All measurement and label data exchanged between NG-RAN nodes and the LMF must be encrypted (using standardized methods like AES) and include integrity verification (such as message authentication codes).

- Anonymization & Access Control: User identifiers within measurement reports should be anonymized. Only authorized entities (e.g., the LMF and designated monitoring components) should have access to detailed positioning data.

- Secure Signaling Channels: Enhanced security associations (via IPsec or TLS) should be established for NRPPa and any new signaling procedures to ensure confidentiality and authenticity.

In addition, integrating these services with network slicing is essential to provide the Quality of Service (QoS) required for high-accuracy positioning. A dedicated network slice for AI/ML positioning can guarantee:

- Dedicated Resources: Allocation of specific bandwidth and processing capabilities for positioning data.

- High-Priority Traffic Handling: Prioritization of measurement data and model feedback messages to meet stringent latency and reliability requirements.

Proposal 3:

- Enhanced Security and Privacy Framework:
Integrate end-to-end encryption, integrity checks, anonymization, and strict access controls into the data exchange procedures for AI/ML positioning.

- Proposal 4:
Leverage network slicing to establish a dedicated QoS profile for AI/ML positioning traffic, ensuring that both measurement reporting and model feedback are transmitted with high priority.

#### 3.4 Integration with Network Slicing and QoS

The detailed signaling design for AI/ML positioning is still under development in RAN1. However, to support adaptive data collection and hybrid monitoring, an enhanced signaling mechanism is needed:

- Dynamic Configuration Updates: The mechanism should support real-time negotiation and updating of measurement parameters between the NG-RAN nodes and the LMF.

- Flexible Message Structures: New or enhanced messages should be defined (or existing ones extended) to include dynamic parameters (such as adaptive sampling rates, quality thresholds, and additional measurement fields like phase and power quality).

Proposal 5:

- Enhanced Signaling Mechanism for Dynamic Measurement Configuration:
Develop an enhanced signaling framework that enables dynamic negotiation and real-time updates of measurement parameters. This mechanism should support both sample-based and path-based measurement reporting and integrate seamlessly with the existing NRPPa procedures.

MeasurementParameterNegotiation ::= SEQUENCE {

samplingRate                      ENUMERATED {ms10, ms20, ms50, ms100},

qualityThreshold                  INTEGER (0..100),  -- SINR threshold

reportingPeriodicity            ENUMERATED {ms100, ms200, ms500, ms1000},

phaseQualityReporting             BOOLEAN OPTIONAL,

powerQualityReporting             BOOLEAN OPTIONAL,

...

}

# 3.	Conclusion

Proposal 1: Develop a standardized data sample structure (Parts A and B) that supports adaptive measurement reporting for both Case 3a and 3b. Extend the NRPPa interface to dynamically negotiate measurement parameters such as sampling rates and quality thresholds.

Proposal 2: Implement a dual-level monitoring system with NG-RAN nodes performing continuous local monitoring and the LMF conducting centralized evaluation. Establish a feedback loop from the LMF to trigger local re-training or parameter adjustments when positioning accuracy degrades.

Proposal 3: Integrate end-to-end encryption, integrity protection, and message authentication for all exchanges between NG-RAN nodes and the LMF. Enforce anonymization and strict access controls to safeguard sensitive measurement and label data.

Proposal 4: Leverage network slicing to allocate dedicated resources and prioritize AI/ML positioning traffic in the network. Define and standardize QoS parameters to ensure high-priority, low-latency delivery of both measurement reports and model feedback.

Proposal 5: Develop an enhanced signaling framework to enable real-time negotiation and dynamic configuration of measurement parameters between NG-RAN nodes and the LMF. Support flexible message structures that accommodate both sample-based and path-based measurement reporting with adaptive capabilities.