3GPP RAN WG3 Meeting #127	R3-250594

# Athens, Greece, 17th – 21st February 2025

Source:	CATT, JIO, Qualcomm, CBN

Title:	Enhancement of UE performance metrics

Agenda Item:	11.2

Document for:	Discussion and decision

- Introduction

Granularity of UE performance was marked as an open issue last meeting. This document shows the benefit of introducing per-DRB UE performance metrics.

- Discussion

  - Background

The UE performance metrics introduced in Rel-18 are all per-UE ones, whereas in Rel-19 there are some proposals of adding per-slice or per-session ones, especially for the Rel-19 use case for slice optimisation.

And for details, the exact formulas for calculating the results are unclear as well. Take average delay as an example. All the measurements introduced in Rel-18 are natively per-DRB ones:

DelayperDRB=eachPacketForTheUEDelayperPacketeachPacketInTheDRB1

So RAN3 have to define a way to average the per-DRB results into a per-UE/per-slice/per-session result.

One opinion is to define them as simple average, i.e. sum of each result per-DRB average delay divided by number of radio bearers:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachDRB_ForTheUE1

Whereas another opinion is to define them as average weighted by number of packets, i.e. sum of delay of each packet divided by the total number of packets, regardless of the DRB used to deliver:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachPacketInTheDRB1eachDRB_ForTheUEeachPacketInTheDRB1=eachPacketForTheUEDelayperPacketeachPacketForTheUE1

Observation 1: There are two major opinions for the definition of per-UE (and potentially per-slice or per-session): simple average, or average weighted by number of packets.

Regardless of the two flavours of definition, we find such per-UE/per-slice/per-session metric prone to disruptions caused by change in actual data rates, and may not be sufficient enough to indicate the service status for flows with relatively stricter QoS requirements. Two typical cases are shown in the following section.

  - Cases that per-UE/slice/session metric is not sufficient

    - Change of actual data rate

It is a common case that packets of a flow come in separate bursts, either periodic or aperiodic. Some example includes:

- Browsing web pages—it often costs many seconds to open a web page with contents coming up in bursts;

- Viewing stored videos—some sites/apps push packets every e.g. 10 seconds rather than constantly; and

- Holding e-meetings—the data rate when the slide changes is quite different form the one when the slice does not change.

As a result, it is a common case that the actual (not maximum!) data rate of a flow changes upon time.

Observation 2: It is a common case that the actual data rate of a flow changes.

The average window of UE performance measurement is configurable (as of Rel-18), with possible value ranging from 0.5s to 10s. It is possible that there is temporarily no data for a DRB within the average window.

For the “simple average” flavour, there are two possible sub-flavours for calculation: excluding the DRBs with no data from calculating the per-UE/slice/session average, or regarding it as 0. Both sub-attitudes are troublesome for handover:

If there are two DRBs for the UE/slice/session with different QoS requirements, and there some data before handover but temporarily no data after handover for the DRB with the stricter QoS requirement, the average value reported to the source node will be higher than the value at source if adopting the “excluding” sub-flavour (i.e. value equals to the one with the looser DRB), and will be lower than the value at source if adopting the “regarding as 0” sub-flavour (i.e. value equals to half of the one with the looser DRB), even if the service levels are kept the same before and after the handover.

For the “average weighted by number of packets” flavour, the case is similar to the “excluding” sub-flavour described above.

| Definition flavour and sub-flavour | Before handover | After handover (no data on DRB1) |
| --- | --- | --- |
| Simple average, excluding DRBs with no data when averaging | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = N/A DRB2: average delay = 100ms overall average delay = 100ms |
| Simple average, regarding the value as 0 for DRBs with no data | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = 0ms DRB2: average delay = 100ms overall average delay = 50ms |
| Average weighted by number of packets (assume the number of packets of the two DRBs are the same before handover) | DRB1: average delay = 40ms DRB2: average delay = 100ms overall average delay = 70ms | DRB1: average delay = N/A DRB2: average delay = 100ms overall average delay = 100ms |

Even if the data is not reduced to 0, the case is also troublesome for the “average weighted by number of packets” flavour, provided the actual data rate changes.

Figure 1: Change in per-UE average packet delay (weighted by number of packets) due to change in data rate. (35.2ms → 81.9ms)

Observation 3: Change of data rate often causes change in per-UE/slice/session average, even if the service level for every DRB is kept unchanged and thus should not be interpreted to better or worse performance for AI/ML model training or monitoring.

    - Fluctuation of measurement result caused by DRB with loose QoS requirement

This problem occurs when the UE is configured with DRBs with vastly different QoS requirements. Any metric in UE performance is statistical, and thus normally fluctuates.

Assume that there are two DRBs for a UE/slice/session before handover (with entirely the same actual data rates):

- DRB1 is configured with strict QoS requirement, with measured average delay fluctuates between 16ms and 24ms.

- DRB2 is configured with loose QoS requirement, with measured average delay fluctuates between 160ms and 240ms.

As the result, the per-UE/slice/session average fluctuates between 88ms to 132ms.

But after handover, the service level for DRB1 deteriorates significantly, doubling the delay. The service level for DRB2 does not change anyway. As the result the per-UE/slice/session average now fluctuates between 96ms to 144ms, virtually the same range as before handover. If the source node receives a value “115ms”, for example, it will have no idea whether the service level deteriorates.

| Before handover | After handover |
| --- | --- |
| DRB1: average delay fluctuates at 16–24ms DRB2: average delay fluctuates at 160–240ms overall average delay fluctuates at 88–130ms | DRB1: average delay fluctuates at 32–48ms DRB2: average delay fluctuates at 160–240ms overall average delay fluctuates at 96–144ms |

Observation 4: Due to the statistical fluctuation of any measurement, any per-UE/slice/session measurement result can hardly reflect how well a DRB with strict QoS requirement is served, provided the UE/slice/session is also configured with a DRB with significantly looser QoS requirement.

  - Handling of flow-to-DRB mapping

In most cases, the flow-to-DRB mapping does not change at handover, since it is a common case that the concerned flows are GBR ones and follows 1-to-1 mapping, or neighbouring gNBs are manufactured by the same vendor and configured with the same policy. For such cases the measured per-DRB UE performance values can be directly used for comparison.

Observation 5: In most cases (e.g. GBR flows, neighbouring gNBs configured with the same mapping policy) the flow-to-DRB mapping does not change at handover, and the measured per-DRB UE performance values can be directly used for comparison.

Nevertheless, there are also some cases that the mapping does change due to various reasons. For these cases, the measured per-DRB values (except throughput) can still be used for comparison, but in an indirect means:

- Step 1: Let the performance value of each QoS flow be the performance value of the DRB to which the QoS flow is mapped.

- Step 2: For each flow, compare the per-QoS-flow values before handover and after handover.

Such indirect means works. Take the delay metric as an example:

Statistically, the delay of each packet is a random variable following a given (random) distribution, in a given radio circumstance and load level. Generally speaking, the goal of improving UE performance is mostly improving the mathematical expectation of the random variable.

A mathematical expectation can never be measured—it can only be estimated. The most common way to estimate an expectation is repeating it many times and estimating the expectation as the average of all measured values. The more times are repeated, the more accurate the estimation is. For delay, the “repeating” is performed as delivering many packets for which the delay values share the same random distribution. The more packets are involved in averaging, the more accurate the estimation is.

So the next question is: for what packets do the delay values share virtually the same random distribution?

The answer is clear: all of the packets which are mapped to the same DRB, regardless of the QoS flow which each packet belongs to:

- All of the packets which are mapped to the same DRB come from the same PDU session, so the processing delay from receiving the packet from the upper layer to generating the SDAP PDU follows the same distribution.

- For PDCP layer and below, all scheduling is based on DRB (these layers even ordinarily do not know what flow a packet belongs to), so the delay values of each SDAP PDU share the same distribution as well.

Observation 6: For QoS flows which are mapped to a given DRB, the packet delays share the same distribution.

Therefore, per-DRB measurement results are the most accurate ones for estimating the mathematical expectation of performance value for each QoS flow, and thus the most accurate ones for comparison for AI/ML purpose.

Observation 7: Based on Observation 6, the following method works for AI/ML purpose:
-	Step 1: Let the performance value of each QoS flow be the performance value of the DRB to which the QoS flow is mapped.
-	Step 2: For each flow, compare the estimated per-QoS-flow values before handover and after handover.

  - Shared message of UE performance feedback for handover and DC

For the handover case, the UE performance is delivered in the XnAP DATA COLLECTION UPDATE message as defined in Rel-18. For the DC case, the majority view (and de facto working assumption) is also reusing the XnAP DATA COLLECTION UPDATE message to deliver the UE performance.

The case in DC, on the other hand, shared many commons with the split-gNB scenario, where a popular opinion is defining UE performance as per-DRB metrics. Therefore introducing per-DRB metrics on XnAP for DC may also be a popular opinion.

Observation 8: Introducing per-DRB UE performance into XnAP for the DC case may be a popular opinion.

Considering that for the handover case it is assumed to use the same XnAP message to deliver UE performance as for the DC case, once we introduce per-DRB UE performance metrics for DC, reusing them in the handover case hardly introduce any additional complexity.

Observation 9: If per-DRB UE performance metrics are introduced for the DC case, reusing them in the handover case hardly introduce any additional complexity.

  - Signalling design

Considering the abovementioned observations, we propose introducing essentially per-DRB UE performance metrics for the handover case.

Proposal 1: Introducing essentially per-DRB UE performance metrics for the handover case.

Nevertheless, the DRB ID may change upon handover (or even remapped). Therefore we propose using group of QFIs as the reference ID.

Proposal 2: Using a group of QFIs (within a session) to indicate the flows with which the per-DRB UE performance result is associated. That is to say, “essentially per-DRB” should be interpreted as “per-QoS-flow-group”.

- Conclusion

Observation 1: There are two major opinions for the definition of per-UE (and potentially per-slice or per-session): simple average, or average weighted by number of packets.

Observation 2: It is a common case that the actual data rate of a flow changes.

Observation 3: Change of data rate often causes change in per-UE/slice/session average, even if the service level for every DRB is kept unchanged and thus should not be interpreted to better or worse performance for AI/ML model training or monitoring.

Observation 4: Due to the statistical fluctuation of any measurement, any per-UE/slice/session measurement result can hardly reflect how well a DRB with strict QoS requirement is served, provided the UE/slice/session is also configured with a DRB with significantly looser QoS requirement.

Observation 5: In most cases the flow-to-DRB mapping does not change at handover, and the measured per-DRB UE performance values can be directly used for comparison.

Observation 6: For QoS flows which are mapped to a given DRB, the packet delays share the same distribution.

Observation 7: Based on Observation 6, the following method works for AI/ML purpose:
-	Step 1: Let the performance value of each QoS flow be the performance value of the DRB to which the QoS flow is mapped.
-	Step 2: For each flow, compare the estimated per-QoS-flow values before handover and after handover.

Observation 8: Introducing per-DRB UE performance into XnAP for the DC case may be a popular opinion.

Observation 9: If per-DRB UE performance metrics are introduced for the DC case, reusing them in the handover case hardly introduce any additional complexity.

Proposal 1: Introducing essentially per-DRB UE performance metrics for the handover case.

Proposal 2: Using a group of QFIs (within a session) to indicate the flows with which the per-DRB UE performance result is associated. That is to say, “essentially per-DRB” should be interpreted as “per-QoS-flow-group”.

- Reference