3GPP TSG-RAN WG3 #125	R3-250599

Athens, Greece, 17th – 21st February 2025

Title:	[Draft] Reply to LS on LP-WUS subgrouping

Response to:	LS Reply on LP-WUS subgrouping (R3-250021/S2-2412876)

Release:	Rel-19

Work Item:	N/A

Source:	CATT [To be RAN3]

To:	RAN2, SA2, CT1, SA3

Cc:

Contact Person:

#### Name:	LIU Aijuan

E-mail Address:	liuaijuan (at) catt (dot) cn

1. Overall Description:

RAN3 thanks RAN2’s and SA2’s LS for providing information. RAN3 notices that one issue may need further coordination following RAN2’s conclusion in the LS and one working assumption achieved in RAN1:

1) The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. 5) For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

According to the current TS 38.304 where LP-WUS is not supported yet, the subgroup for PEI is determined by the (truncated) 5G-S-TMSI: up to 12 Least Significant Bits (LSBs) of 5G-S-TMSI are used for selecting the PO, and the next up to 3 bits are used for determining the PEI subgroup. Therefore, up to 15 LSBs of 5G-S-TMSI in total are used.

Now for LP-WUS, up to 5 bits (as in RAN1 working assumption) will be needed for determining the LP-WUS subgroup, and they will probably be truncated from 5G-S-TMSI too (as RAN2 conclusion). PEI and LP-WUS may be used together, so up to 20 LSBs of 5G-S-TMSI in total may be needed.

Therefore, if all of PEI, LP-WUS and RRC_INACTIVE state are supported by the UE and the NG-RAN, the 20 LSBs of 5G-S-TMSI of a given UE will be needed to be provided from the AMF to any gNB which serves the UE, and stored as a part of the UE context in the gNB, so that the NG-RAN can perform paging correctly after releasing the UE into RRC_INACTIVE state.

RAN3, however, is not sure whether it will be allowed to do so. RAN3 would like to invite SA2/SA3 for further check.

## 2. Actions:

To SA2 and SA3:

ACTION: RAN3 respectfully asks SA2/SA3 to check whether it is allowed that the AMF provides the 20 LSBs of the 5G-S-TMSI of a given UE toward every gNB which serves the UE, and that the receiving gNB stores it as a part of the UE context.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127-bis	07th – 11th April 2025	TBD, China

TSG-RAN3 Meeting #128	19th – 23rd May 2025	Malta