3GPP TSG-RAN WG3 Meeting #127	R3-250612

Athens, Greece, 17 – 21 Feb, 2025

Agenda item:	 10.4

Source:		Samsung

Title:	(TP for SON BLCR for 38.423, 36.423 and TS37.340) Rel-18 SON and MDT leftovers

Document for:		Discussion and Decision

# 1	Introduction

The following leftovers in Rel-18 SON/MDT have been agreed in the scope of Rel-19 SON/MDT WI.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

The contribution discussed RACH optimization for SDT, MHI Enhancement for SCG Deactivation/Activation and MRO for MR-DC SCG failure.

# 2	Discussion

## 2.1 RACH optimization for SDT

RA-SDT has been designed to send the small data over RACH resources, which enables UE to transmit/send data in RRC inactive state. It is an efficient solution to avoid UE coming back to RRC connected state for the small data. gNB sets the following configuration parameters for RA-SDT and sends to UE via SIB1:

- RSRP threshold for UE to determine whether to perform SDT procedure

- Data volume threshold used to determine whether SDT can be initiated

- The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT

- A timer for SDT (T319a)

The SDT performance may be not good due to poor configuration. For example, with the low RSRP threshold, the data transmission may be interrupted due to poor channel condition. In addition, if data volume threshold is high but the timer is small, the data can not be sent within the timer limited period, so that the UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission. Thus, to do self-optimization, the information of RA-SDT is needed to transfer to the node who sets the configuration. The configuration information is broadcasted in system information block. For split architecture, the configuration parameters are set by DU, so F1AP impact should be considered to transfer the related information from CU to DU for DU self-optimization.

With above analysis, the following information should be provided by the UE to give the reference for node to do self-optimization.

- RSRP when SDT fails: this information gives reference information to the node to decide whether the RSRP threshold is set too low. For example, the RSRP threshold is set as a low value, while the channel condition as such threshold is not good to support the SDT transmission. If SDT fails, the nodes need to set the RSRP threshold as a higher value.

- Remaining data volume after SDT transmission: this information tells the node whether the data volume threshold is set properly or not. When the remaining data volume is small, it means that UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission for such a small data volume. The more desired way is to set a higher data volume threshold for the SDT triggering or set a larger timer for transmission.

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

## 2.2 MR-DC SCG failure scenario

TPs for TS38.423 and TS36.423 have been agreed in RAN3#125bis meeting.

In the procedure text on IE “Time SCG Failure” in the SCG FAILURE INFORMATION REPORT message, TS38.300 is referred in the BLCR for TS38.423, and TS36.300 is referred in the BLCR for TS36.423. On the MRO for MR-DC SCG failure scenarios, TS37.340 should be referred. Corrections are provided in Annex.

Proposal 2: Agree the TPs for TS38.423 and TS36.423 in Annex A and Annex B.

## 2.3 MHI Enhancement for SCG Deactivation/Activation

RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. For how to collect the time spent information, RAN2 identified two options as listed in R3-245009:

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

In RAN3 126 meeting, it has been agreed to support a network-based solution for enhancing PSCell activation/deactivation. There are two remaining issues:

- Do we prefer an absolute time in active state or a relative time?

- Is additional information needed, e.g. Throughput threshold used for activating, Average throughput, Number of activations?

The information of PSCell status is used to optimize the configuration of dual connectivity, such as the necessity to configure the UE as DC, or choose which cell as the PSCell. So the time duration (relative time) is reasonable, similar as the UE history information stored in the network side. Based on the relative time, the network can accurately get how long the PSCell is activated or deactivated.

Proposal 3: The network can collect the time duration spent in PSCell activation or deactivation status as a relative time.

For the second issue, once the nodes (i.e. MN and/or SN) have the PSCell information of activation/deactivation status and time duration spent in each status, it know whether the PSCell is in deactivation status for a long time without actual usage, the configuration of dual connectivity can be optimised, such as the necessity to configure the UE as DC, or choose which cell as the PSCell. Hence, the PSCell activation/deactivation status and time duration spent in each status respectively are enough without introducing additional information.

Observation: PSCell activation/deactivation status and time duration spent in the status respectively are enough for the optimization of DC configuration.

Proposal 4: There is no need to introduce additional information, e.g. Throughput threshold used for activating, Average throughput, Number of activations, etc.

Regarding to the standard impact, SN knows the PSCell status change and corresponding time, so SN can store the status and time duration by itself. In terms of MN, the activation/deactivation decision is transferred via MN. Thus, MN also knows whether SCG is deactivated or not, MN can collect the time duration information. Based on the time duration information, the MN could understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity. Based on the understand so far, it seems only stage 2 impact is needed. If any scenario is identified that the PSCell status information should be transmitted between nodes, such as from MN to target SN, the stage 3 impact can be discussed accordingly.

Proposal 5: Agree the stage 2 description of network-based solution in Annex C.

# 3	Conclusion

This contribution discussed MRO for MR-DC SCG failure and RACH optimization for SDT. We have the following proposals.

RACH optimization for SDT

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

MR-DC SCG failure scenario

Proposal 2: Agree the TPs for TS38.423 and TS36.423 in Annex A and Annex B.

MHI Enhancement for SCG Deactivation/Activation:

Proposal 3: The network can collect the time duration spent in PSCell activation or deactivation status as a relative time.

Observation: PSCell activation/deactivation status and time duration spent in the status respectively are enough for the optimization of DC configuration.

Proposal 4: There is no need to introduce additional information, e.g. Throughput threshold used for activating, Average throughput, Number of activations, etc.

Proposal 5: Agree the stage 2 description of network-based solution in Annex C.

# Reference

[1] TS38.331

[2] TS36.331

[2] TS38.423

# Annex A: TP for TS38.423 on MR-DC SCG failure

#### 8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

# Annex B: TP for TS36.423 on MR-DC SCG failure

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS 37.340 [32].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the en-gNB shall, if supported, store the information and act as specified in TS 37.340 [32].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

# Annex C: TP for TS37.340 on MHI enhancement

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

When the PSCell status changes, MN and/or SN store(s) the PSCell activation or deactivation status and corresponding time duration spent in the status to do the further optimization on PSCell addition or change.