3GPP TSG-RAN WG3 #127	          R3-250631

# Athens, Greece, 17th – 21th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0188 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SRS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE, CATT, Qualcomm, Nokia, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, Validity Area Specific SRS Information IE was introduced in the Requested SRS Transmission Characteristics IE. The NG-RAN may take this information into account for configuring SRS transmissions for the UE in the indicated validity area during Positioning Information Exchange procedure, and/or may reserve the indicated SRS information during SRS Information Reservation Notification procedure. However, the current information contained in the Validity Area Specific SRS Information IE corresponds to only one SRS resource, which leads to the following situations, - During Positioning Information Exchange procedure, the LMF can recommend only one SRS resource for NG-RAN to configure SRS configuration containing multiple SRS resource sets and multiple SRS resources in each SRS resource set. Resulting in more chances for different NG-RAN node to allocate the same area-specific SRS resources for different UEs, which should be avoided. - During SRS Information Reservation Notification procedure, the LMF can notify the NG-RAN to reserve only one SRS resource in each SRS INFORMATION RESERVATION NOTIFICATION message, which is inefficient if the LMF needs to notify NG-RAN to reserve multiple SRS resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the current Validity Area Specific SRS Information IE to a list. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR is an NBC change. This CR has an impact from protocol and functional point of view. The impact can be considered isolated because it only impacts the procedure to support NR positioning function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionality of Area-specific SRS configuration is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.96, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR1545 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

9.2.96	Validity Area Specific SRS Information

This IE contains the validity area specific SRS Information when area-specific SRS is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Validity Area Specific SRS Resource Set List |  | 1 |  |  | - |  |
| >Validity Area Specific SRS Resource Set Item |  | 1..<maxnoSRS-PosResourceSets> |  |  | - |  |
| >>Validity Area Specific SRS Resource List | M |  | 9.2.XX |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

-------------------------------------------Next change-------------------------------------------

9.2.XX	Validity Area Specific SRS Resource List

This IE contains the validity area specific SRS resource list.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Validity Area Specific SRS Resource Item |  | 1..<maxnoSRS-PosResources> |  |  | - |  |
| >CHOICE Transmission Comb | O |  |  |  | - |  |
| >>Comb Two |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >>Comb Four |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >>Comb Eight |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..7) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..5) |  | - |  |
| >Resource Mapping |  | 0..1 |  |  | - |  |
| >>Start Position | M |  | INTEGER(0..13) |  | - |  |
| >>Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12) |  | - |  |
| >Frequency Domain Shift | O |  | INTEGER(0..268) |  | - |  |
| >C-SRS | O |  | INTEGER(0..63) |  | - |  |
| >CHOICE Resource Type Positioning | O |  |  |  | - |  |
| >>periodic |  |  |  |  |  |  |
| >>>SRSPeriodicity | M |  | 9.2.99 |  | - |  |
| >>>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms | YES | ignore |
| >>semi-persistent |  |  |  |  |  |  |
| >>>SRSPeriodicity | M |  | 9.2.99 |  | - |  |
| >>>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms | YES | ignore |
| >>aperiodic |  |  |  | Not applicable if the Positioning Validity Area Cell List IE is included | - |  |
| >>>slot offset | M |  | INTEGER(0..32) |  | - |  |
| >Sequence ID | O |  | INTEGER(0..65535) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |

-------------------------------------------Next change-------------------------------------------

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

<<<Omit unchanged>>>

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-LocalOrigin,

id-DL-reference-signal-UERxTx-TD,

id-SRSPosPeriodicConfigHyperSFNIndex

id-ValidityAreaSpecificSRSResource-List

<<<Omit unchanged>>>

#### -- V

ValueRSRP-EUTRA ::= INTEGER (0..97, ...)

ValueRSRQ-EUTRA ::= INTEGER (0..34, ...)

ValueRSRP-NR ::= INTEGER (0..127)

ValueRSRQ-NR ::= INTEGER (0..127)

ValidityAreaSpecificSRSInformation ::= SEQUENCE {

validityAreaSpecificSRSResourceSet-List	SEQUENCE (SIZE(1.. maxnoSRS-PosResourceSets)) OF ValidityAreaSpecificSRSResourceSet-Item,

iE-extensions		ProtocolExtensionContainer { { ValidityAreaSpecificSRSInformation-ExtIEs } }	OPTIONAL,

...

}

ValidityAreaSpecificSRSInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ValidityAreaSpecificSRSResourceSet-Item ::= SEQUENCE {

validityAreaSpecificSRSResource-List			ValidityAreaSpecificSRSResource-List,

iE-Extensions		ProtocolExtensionContainer { {ValidityAreaSpecificSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

ValidityAreaSpecificSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ValidityAreaSpecificSRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF ValidityAreaSpecificSRSResource-Item

ValidityAreaSpecificSRSResource-Item ::= SEQUENCE {

transmissionCombPos				TransmissionCombPos 	OPTIONAL,

resourceMapping 				ResourceMapping			OPTIONAL,

freqDomainShift					INTEGER (0..268)		OPTIONAL,

c-SRS							INTEGER (0..63)			OPTIONAL,

resourceTypePos					ResourceTypePos			OPTIONAL,

sequenceIDPos					INTEGER (0..65535)		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ValidityAreaSpecificSRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

ValidityAreaSpecificSRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------Next change-------------------------------------------

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

<<<Omit unchanged>>>

id-PreconfiguredSRSInformation									ProtocolIE-ID ::= 163

id-DL-reference-signal-UERxTx-TD								ProtocolIE-ID ::= 164

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-ValidityAreaSpecificSRSResource-List							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------