3GPP TSG-RAN WG3 #127	          R3-250636

# Athens, Greece, 17th – 21th Feb 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	Discussion on RAN architecture and procedures for AIoT

Document for:	Discussion and decision

Introduction

The Ambient IoT SI has been transferred to a WI, with the following RAN3 scope in terms of RAN architecture and procedures,

Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

- Inventory and command operations

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we discuss the fundamental aspects to be specified.

Discussion

The Ambient IoT WI has been approved in the last RANP meeting. According to the WID, in general only D1T1 with Device 1 is considered in this WI.

The first aspect is about the architecture, and TR 38.769 has provided a concrete analysis on the system architecture as well as the protocol stack for D1T1 as follows,

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface is the NG-C interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1:

Figure 6.4.1-2. Protocol Stack for Topology 1

…

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

NOTE 2:	Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision.

NOTE 3: The A-IoT CN may include AMF and A-IoTF which is up to SA2 decision.

From the above text, we can observe that the first thing to determine during WI is whether XXAP is realized by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP.

Although according to SA2 conclusion, AIoT RAN node can communicate with AIoTF directly or indirectly via AMF, we see little difference on carrying AIoTF information directly in NGAP or by a new protocol layer over NGAP. Considering that the current NGAP has already captured SMF related information (i.e. RAN communicates with SMF via AMF), and the AIoTF related information is not expected to be too much, we prefer to directly use NGAP to include AIoTF information without creating a new protocol layer over NGAP.

Proposal 1: XXAP is realized by including AIoTF information in NGAP.

Another architectural issue that should be determined is on the reader selection. In previous RAN3 discussion, the reader selection is still left open with the following text captured in TR 38.769,

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

NOTE 5:	How to perform reader selection needs further discussions.

NOTE 6: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

In addition, according to the latest LS from SA2 in R3-250075, the following agreement has been achieved,

SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network.

SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters.

Firstly, as can be observed from SA2 LS, OAM solution is always by default if there’s no signalling solution that can be agreed. Secondly, if a signalling solution is considered, the basic question is at what granularity the serving area information should be provided from AIoT RAN to AIoTF, TAI, Cell ID, geographical coordinates or other approach (e.g. a reference point + a radius, etc.). We believe such question can be discussed in another Agenda Item driven by incoming LS.

Another aspect is to discuss the procedures. Note that in TR 38.769 we have captured the primary message flow for AIoT inventory and command operations in T1 as follows,

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

So basically the following questions should be discussed,

- What information is included in Inventory/Command Request/Response?

- Whether the Command Request/Response is Device-associated or non-Device-associated?

For the first question, in RAN3’s previous discussion, the AIoT Device ID and the Scope of Inventory Request have been agreed to be included in Inventory Request message. The detail of AIoT Device ID should still wait for more SA2 progress. And the detail of Scope (a list of BS reader IDs, area scope, or other options) is partially dependent on the outcome of the reader selection mechanism.

Proposal 2: Include the AIoT Device ID and the Scope in the Inventory Request message. FFS on details.

For other potential parameters that can be included in the Inventory Request message, based on LS from SA2 to RAN2 (CC RAN3), SA2 has confirmed that the following information can be provided from CN to the reader if available,

- the service type of A-IoT (e.g. inventory, command)

- whether the service is targeted for a single or multiple devices;

- the approximate number of target devices (if available).

To align with the discussion in RAN2 and SA2, we believe the above three parameters are also needed to be included in the Inventory Request message. For the service type of AIoT, we think it can be distinguished by the procedure code so no need to additionally include such information in the Inventory Request message.

Moreover, we think the Inventory Session ID is also needed. Recall that RFID supports multiple sessions for inventory. In order to provide a comparable capability for Ambient IoT to RFID. We see no obstacle to support multiple inventory sessions. In our understanding, the inventory session is determined by AF/CN, and a session related identifier should be provided in the Inventory Request message.

Proposal 3: Include the following parameters in the Inventory Request message,

- Whether the service is targeted for a single or multiple devices;

- The approximate number of target devices (if available).

Proposal 4: Consider the Inventory Session/Transaction ID as the content of Inventory Request message.

From the second question, RAN3 is better to wait for more progress from SA2, SA3 and RAN2, since these WGs are discussing the similar issue.

Conclusion

In this paper, we provide primary discussion on RAN-CN interface impact for Ambient IoT. The following proposals are provided,

Proposal 1: XXAP is realized by including AIoTF information in NGAP.

Proposal 2: Include the AIoT Device ID and the Scope in the Inventory Request message. FFS on details.

Proposal 3: Include the following parameters in the Inventory Request message,

- Whether the service is targeted for a single or multiple devices;

- The approximate number of target devices (if available).

Proposal 4: Consider the Inventory Session/Transaction ID as the content of Inventory Request message.

Reference