3GPP TSG-RAN WG3 #127								R3-250643

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	16.2

Source:	China Telecom

Title: 	Discussion on Signalling Procedure for Inventory and Command

Document for:	Discussion

Introduction

In SI phase, the signalling and procedures for Topology 1 were agreed and captured into TR 38.769[1]. The work item on solutions for Ambient IoT (Internet of Things) in NR was agreed in RAN#106 meeting [2]. RAN3 scope is included in the following objectives:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

The inventory and command procedures need to be specified in WI phase. This contribution aims to discuss the procedure and content information for inventory and command.

Discussion

Procedures for inventory and command

Per clause 6.5.2 of TR38.769, as shown in Figure 1, the candidate inventory procedure comprises one class 1 inventory request/response procedure and multiple class 2 inventory report procedures. Upon receiving the inventory request message, the A-IoT RAN node may attempt to allocate and coordinate resources and send a response or failure message to the A-IoT CN node. Subsequently, the A-IoT RAN node sends multiple inventory report messages upon receiving inventory results from A-IoT device(s). Therefore, a class 1 inventory request procedure and a class 2 inventory report procedure need to be specified in the xx interface. Additionally, the inventory request procedure also supports a failure message as a response to indicate that the inventory procedure could not be initiated for the A-IoT device(s).

Proposal 1: A class 1 inventory request procedure and a class 2 inventory report procedure need to be specified in the xx interface.

Proposal 2: the inventory request procedure also supports a failure message as response to indicate that the inventory procedure could not be initiated towards the A-IoT device(s).

Figure 1 Message flow for A-IoT Inventory

As shown in Figure 2, the Class 1 Command Procedure comprises of Command Request message and Command Response/Failure message. Therefore, a new Class 1 Command Procedure comprises of Command Request message and Command Response/Failure message need to be introduced in xx interface.

Figure 2 Message flow for A-IoT Command

Proposal 3: a new Class 1 Command Procedure comprises of Command Request message and Command Response/Failure message need to be introduced in xx interface.

Content in Inventory Procedure

According to conclusions of TR23.700-13, the following information can be included in inventory request message:

- correlation identifier: As a single A-IoT RAN may connect to multiple A-IoT CNs, the correlation identifier allows the AIOTF to match inventory responses with the corresponding request. Therefore, the correlation identifier is required in inventory request, response, and failure message.

- AIOTF identifier: To route the request back to the originating AIOTF, the AMF additionally provides an AIOTF identifier when forwarding the request from the AIOTF. Therefore, the AIOTF identifier is necessary in inventory request, response, and failure messages.

- part of or full AIoT Device Identifier: Inventory requests can be sent by the A-IoT CN for a single device, a group of devices, or all devices. According to TR23.700-13, AIoT devices compare the AIoT Device Identification information with their own AIoT Device Identifier (partial or full) to determine whether they should respond to the paging message. Additionally, a partial AIoT Device Identifier can be used as filtering information to associate multiple AIoT devices.

Additionally, we propose introducing a session ID, as the specifications must allow multiple inventory procedure instances to run in parallel over the RAN-CN interface. Given that the A-IoT RAN system may cover multiple regions in practical deployments, the inventory area list should be included in the inventory message. Therefore, the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Proposal 4: the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Content in Command Procedure

The Command Request message is performed after inventory message. According to conclusions of TR23.700-13, the following information can be included in Command Request message:

- correlation identifier

- CN UE ID and RAN UE ID: in legacy mechanism, both RAN and CN need to allocate a separate ID for UE. This mechanism can reused in A-IoT.

- Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- IoT NAS PDU

- Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Proposal 5: the following information can be included in A-IoT Command Request message:

- [M]correlation identifier

- [M] CN UE ID

- [M] RAN UE ID

- [O]Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- [M]IoT NAS PDU

- [O]Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: a class 1 procedure inventory request and class 2 inventory report procedure need to be specified in xx interface.

Proposal 2: the inventory request procedure also supports a failure message as response to indicate that the inventory procedure could not be initiated towards the A-IoT device(s).

Proposal 3: a new Class 1 Command Procedure comprises of Command Request message and Command Response/Failure message need to be introduced in xx interface.

Proposal 4: the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Proposal 5: the following information can be included in A-IoT Command Request message:

- [M]correlation identifier

- [M] CN UE ID

- [M] RAN UE ID

- [O]Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- [M]IoT NAS PDU

- [O]Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR