3GPP TSG-RAN WG3 Meeting #127	R3-250646

Athens, Greece, February 17th – 21st, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	(TP for NR_Mob_Ph4 TS 38.401 and TS 38.473) Conditional LTM

Document for:	Other

# Introduction

This contribution proposes text proposal for TS 38.401 [1] (rapporteur: China Telecom) and TS 38.473 [2] (rapporteur: Huawei), based on the discussions proposed in [3].

# Reference

[1] TS 38.401, NG-RAN; Architecture description (Release 18)

[2] TS 38.473, NG-RAN; 	NG-RAN; F1 Application Protocol (F1AP) (Release 18)

[3] R3-250645, “Discussions on RAN3 impacts from Conditional Intra-CU LTM”, LG Electronics Inc.

# TP for TS 38.401

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.X	Inter-gNB-DU Conditional LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for Conditional LTM. Figure 8.2.1.X-1 shows the inter-gNB-DU Conditional LTM procedure for intra-NR.

Figure 8.2.1.X-1: Inter-gNB-DU Conditional LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate Conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID . The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to generate the L1-based execution conditions for subsequent Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration. The gNB-CU may provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of Conditional LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 1:	In case that the candidate cells chosen by the CU are under the source gNB-DU, the CU-initiated UE Context Modification procedure is initiated to the source gNB-DU for each candidate cell. The process described in the above Step 3-4 applies similarly, with the difference being the use of the UE Context Modification procedure, and the use of the source gNB-DU instead of the candidate gNB-DU.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may request the source gNB-DU to generate the L1-based execution conditions for initial Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration of the source cell. If the L1-based execution conditions are requested, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent Conditional LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration. The gNB-CU may request the candidate gNB-DU(s) to generate the L1-based execution conditions for subsequent Conditional LTM.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration of the requested candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells

NOTE 2: Step 7 may also be triggered after step 20 by implementation for subsequent Conditional LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the Conditional LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. The gNB-CU may request the source gNB-DU to trigger early TA acquisition for a specific candidate cell.

14.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

15.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

16.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

17.	The source gNB-DU configures the TA value to the UE .

18.	An execution condition to trigger initiation of LTM cell switch is fulfilled.

19.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

20.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID. The target gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU.

21.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

22.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

23.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU to inform that the UE executed an inter-DU Conditional LTM and indicate to stop the data transmission for the UE. The source gNB-DU revert the previous serving cell to a prepared state, if it remains one of the configured LTM candidate cells, and sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 2:	The step 23 may happen before step 22, as soon as the gNB-CU knows which cell the UE has successfully accessed.

24.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# TP for TS 38.473

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.1.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM ConfigurationIE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM for the included SpCell ID IE. If the CSI Resource Configuration is also contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT SETUP RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation) in the UE CONTEXT MODIFICATION RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the Early Sync Candidate Cell Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Candidate Cell Information List IE for some candidate cell, the gNB-DU shall, if supported, take them into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the Early Sync Serving Cell Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Serving Cell Information IE, the gNB-DU shall, if supported, take it into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM. If the CSI Resource Configuration IE is also contained in the LTM Information Modify IE and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message. If the CSI Resource Configuration IE is also contained in the LTM Information Modify IE while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the serving cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  | YES |  | reject |
| gNB-DU UE F1AP ID |  | O |  |  |  | 9.3.1.5 |  |  |  | YES |  | ignore |
| SpCell ID |  | M |  |  |  | NR CGI 9.3.1.12 |  | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. |  | YES |  | reject |
| ServCellIndex |  | M |  |  |  | INTEGER (0..31,...) |  |  |  | YES |  | reject |
| SpCell UL Configured |  | O |  |  |  | Cell UL Configured 9.3.1.33 |  |  |  | YES |  | ignore |
| CU to DU RRC Information |  | M |  |  |  | 9.3.1.25 |  |  |  | YES |  | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |
| LTM Information Setup |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >LTM Indicator | M |  |  |  | ENUMERATED (true, …, C-LTM) |  |  |  | - |  |  |  |
| >Reference Configuration | O |  |  |  | 9.3.1.292 |  |  |  | - |  |  |  |
| >CSI Resource Configuration | O |  |  |  | 9.3.1.330 |  |  |  | - |  |  |  |
| LTM Configuration ID Mapping List | O |  |  |  | 9.3.1.294 |  |  |  | YES |  | reject |  |
| Early Sync Information Request |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Request for RACH Configuration | M |  |  |  | ENUMERATED (true, …) |  |  |  | - |  |  |  |
| >LTM gNB-DUs List |  |  | 1 |  |  |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |  | YES |  | reject |  |
| >>LTM gNB-DUs Item IEs |  |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |  | - |  |  |  |
| >>>LTM gNB-DU ID | M |  |  |  | gNB-DU ID 9.3.1.9 |  |  |  | - |  |  |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >Conditional LTM Execution Condition List | O |  | 9.3.1.XXX | FFS | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  | YES |  | reject |  |
| gNB-DU UE F1AP ID |  | M |  |  |  | 9.3.1.5 |  |  |  | YES |  | reject |  |
| SpCell ID |  | O |  |  |  | NR CGI 9.3.1.12 |  | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. |  | YES |  | ignore |  |
| ServCellIndex |  | O |  |  |  | INTEGER (0..31, ...) |  |  |  | YES |  | reject |  |
| SpCell UL Configured |  | O |  |  |  | Cell UL Configured 9.3.1.33 |  |  |  | YES |  | ignore |  |
| DRX Cycle |  | O |  |  |  | 9.3.1.24 |  |  |  | YES |  | ignore |  |
| CU to DU RRC Information |  | O |  |  |  | 9.3.1.25 |  |  |  | YES |  | reject |  |
| Transmission Action Indicator |  | O |  |  |  | 9.3.1.11 |  |  |  | YES |  | ignore |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LTM Information Modify |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >LTM Indicator | M |  |  |  | ENUMERATED (true, …, C-LTM) |  |  |  | - |  |  |  |
| >Reference Configuration | O |  |  |  | 9.3.1.292 |  |  |  | - |  |  |  |
| >CSI Resource Configuration | O |  |  |  | 9.3.1.330 |  |  |  | - |  |  |  |
| LTM CFRA Resource Config List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >LTM CFRA Resource Config Item IEs |  |  | 1 .. <maxnoofLTMCells> |  |  |  |  |  | EACH |  | ignore |  |
| >>Cell ID | M |  |  |  | NR CGI 9.3.1.12 |  |  |  | - |  |  |  |
| >>LTM CFRA Resource Configuration | O |  |  |  | OCTET STRING |  | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. |  | - |  |  |  |
| >>LTM CFRA Resource Configuration for SUL | O |  |  |  | OCTET STRING |  | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. |  | - |  |  |  |
| LTM Configuration ID Mapping List | O |  |  |  | 9.3.1.294 |  |  |  | YES |  | reject |  |
| Early Sync Information Request |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Request for RACH Configuration | M |  |  |  | ENUMERATED (true, …) |  |  |  | - |  |  |  |
| >LTM gNB-DUs ID List |  |  | 1 |  |  |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |  | YES |  | reject |  |
| >>LTM gNB-DUs Item IEs |  |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |  | - |  |  |  |
| >>>LTM gNB-DU ID | M |  |  |  | gNB-DU ID 9.3.1.9 |  |  |  | - |  |  |  |
| Early Sync Candidate Cell Information List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Early Sync Candidate Cell Information Item IEs |  |  | 1 .. <maxnoofLTMCells> |  |  |  |  |  | EACH |  | ignore |  |
| >>Cell ID | M |  |  |  | NR CGI 9.3.1.12 |  |  |  | - |  |  |  |
| >>TCI States Configurations List | O |  |  |  | OCTET STRING |  | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. |  | - |  |  |  |
| >>Early UL Sync Configuration | O |  |  |  | 9.3.1.328 |  |  |  | - |  |  |  |
| >>Early UL Sync Configuration for SUL | O |  |  |  | Early UL Sync Configuration 9.3.1.328 |  | This IE applies for SUL carrier. |  | - |  |  |  |
| >>TA Assistance Information | O |  |  |  | ENUMERATED (zero, …) |  | The value "zero" corresponds to TA value of the cell being equal to zero. |  | - |  |  |  |
| >>UE Based TA Measurement Configuration | O |  |  |  | OCTET STRING |  | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. |  | - |  |  |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  |  |  | 9.3.1.138 |  | This IE applies to early TA acquisition. |  | YES |  | ignore |  |
| Early Sync Serving Cell Information |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >UE Based TA Measurement Configuration | O |  |  |  | OCTET STRING |  | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. |  | - |  |  |  |
| LTM Cells To Be Released List | O |  |  |  | 9.3.1.291 |  |  |  | YES |  | reject |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >Conditional LTM Execution Condition List | O |  | 9.3.1.XXX | FFS | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.XXX		Conditional LTM Execution Condition List

This IE indicates the list of conditional LTM execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Conditional LTM Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING (FFS) | Up to RAN2 | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////