3GPP TSG-RAN WG3 Meeting #127	R3-250657

Athens, Greece, 17-21 Feb, 2025

Title:	Consideration on A-IoT Conclusions in SA WG2

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion

# Introduction

In SA2#166 meeting, SA2 sent an LS to RAN3 in [1], informed us their A-IoT Conclusions. In this contribution, we discuss the impacts to RAN3 and provide information as necessary.

# Discussion

In the SA2 LS [1], the following can be found:

Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request? SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2. Q2: If such information (see Q1) can be provided by CN to the reader, can it be provided only in some cases or in all cases? SA2 answer: it can be provided in some cases. See answer to Q1.

From RAN3 point of view, the AIOTF will be able to provide the approximate D2R message size, but whether it can always provide such information is under SA2 discussion, therefore, RAN3 needs to include Approximate D2R Message Size IE in the Inventory Request Transfer IE within the INVENTORY REQUEST message, and the presence of this IE is FFS.

Conclusion 1: NGAP signaling will enable AIOTF providing the approximate D2R message size to the gNB (e.g., include Approximate D2R Message Size IE in the Inventory Request Transfer IE within the INVENTORY REQUEST message), and the presence of this IE is FFS.

SA2 also listed issues in coordination with RAN WG as follows:

- Issues in coordination with RAN WG: - Radio resources allocation (what information and how to provide it) to Readers for topology 2. - For RRC based option of topology2, whether the gNB performs the down selection of UE readers provided by AIOTF. - The layer to enable AIoT Reader control, whether using NGAP or a layer above NGAP. - Whether and how AIOTF selects BS readers or AIOT RAN nodes - UE reader authorization for topology 2 - For RRC option for topology 2, RRC support to forward UE Reader control information between UE and gNB. - Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1 - How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF

- About “The layer to enable AIoT Reader control, whether using NGAP or a layer above NGAP”

This is a very important aspect to be discussed in the WI, in [5], we propose to use NGAP containers rather than introduce a new protocol layer carried by NGAP.

RAN3 will inform the conclusion to SA2 based on WI progress.

Conclusion 2: Inform SA2 that RAN3 agree to use NGAP containers rather than introduce a new protocol layer carried by NGAP [subject to WI discussion].

- About “Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1”

Based on the latest progress information from SA2 in [3], RAN3 should make decision by taking their latest progress into accunt, the corresponding discussion can be found in [4]. In [4] and [5] we propose to Reply to SA2 that RAN3 concluded that the AIOTF is able to get the mentioned information, via NG signalling, and/or OAM configuration.

With that, it can be understood that the AIOTF will first select gNBs, and the AIOTF may also select BS readers or the Inventory Area(s), in the latter case the gNB further select readers within the Inventory Area(s).

Conclusion 3: The AIOTF will first select gNBs, and the AIOTF may also select BS readers or the Inventory Area(s), in the latter case the gNB further select readers within the Inventory Area(s).

- About “	How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF”

In case of indirect communication, the gNB communicates with the AIOTF via AMF.

Upon receiving the INVENTORY REQUEST message, the gNB will sends the the INVENTORY RESPONSE message and subsequent INVENTORY REPORT message(s) towards the AMF from which the INVENTORY REQUEST was received. Similar, the gNB will also sends the INVENTORY RESPONSE message towards the AMF from which the COMMAND REQUEST message was received.

Conclusion 4: The gNB is able to route the UL signalling towards the AMF from which the inventory/command request was received.

From AMF point of view, one AMF may connect to multiple AIOTFs, therefore, upon receiving the UL signalling from the gNB, the AMF needs to route the message toward correct AIOTF.

- There are following SA2 TR conclusions about indirect communication via AMF:

- SA2 TR 13.700-13: - 2. The AIoTF performs the following functionality. … f. When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF. … - The AMF shall be enhanced to support: - On the Nz reference point, the AMF supports Services which are used by an AIoTF for Ambient IoT Operations e.g. to send AIoT requests towards AIOT RAN and to receive AIoT responses from the AIOT RAN. - On the N2 reference point, the AMF supports sending AIOT information to the AIOT RAN (e.g. operation requests) and receiving responses from the AIOT RAN. - The AMF routes the AIoT messages between the AIOT RAN (over N2 reference point) and the AIOTF (over Nz reference point).

From RAN3 perspective, to enable the UL traffic routing by the AMF (avoid store mapping information at AMT), the AIOTF identifier will be included in the UL and DL NGAP inventory/command related messages. Details see [7].

- Conclusion 5: NGAP signalling will include AIOTF identifier, to enable the AMF to perform UL traffic routing towards the corresponding AIOTF.

# Conclusion

In this contribution, we discussed the SA2 conclusions by taking all previous discussion into account, and get the following Conclusions:

Conclusion 1: NGAP signaling will enable AIOTF providing the approximate D2R message size to the gNB (e.g., include Approximate D2R Message Size IE in the Inventory Request Transfer IE within the INVENTORY REQUEST message), and the presence of this IE is FFS.

Conclusion 2: Inform SA2 that RAN3 agree to use NGAP containers rather than introduce a new protocol layer carried by NGAP [subject to WI discussion].

Conclusion 3: The AIOTF will first select gNBs, and the AIOTF may also select BS readers or the Inventory Area(s), in the latter case the gNB further select readers within the Inventory Area(s).

Conclusion 4: The gNB is able to route the UL signalling towards the AMF from which the inventory/command request was received.

- Conclusion 5: NGAP signalling will include AIOTF identifier, to enable the AMF to perform UL traffic routing towards the corresponding AIOTF.

The draft reply LS is provided in [8].

# Reference

[1] S2-2413035 LS on A-IoT Conclusions in SA WG2, Source: SA2, To:	SA3, SA5, RAN2, RAN3, Cc:	RAN, SA

[2] TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3] S2-2501343 LS on SA2 Ambient IoT TR conclusion update, Source: SA2, To: RAN3

[4] R3-250195Consideration on SA2 Ambient IoT TR conclusion update, Huawei, China Unicom

[5] R3-250196 [Draft] Reply LS on SA2 Ambient IoT TR conclusion update, Source: Huawei [to be RAN3], To: SA2

[6] R3-250197 (TPs for TS 38.300, TS 38.410 BL CRs) Ambient IoT RAN Architecture, Huawei, China Telecom, Lenovo, China Unicom

[7] R3-250199 (TPs to TS 38.413 TS 38.410 BL CRs) A-IoT Inventory procedures, Huawei, China Telecom, China Unicom

[8] R3-250658 [Draft] Reply LS on A-IoT Conclusions in SA WG2, Source: Huawei [to be RAN3], To: SA2, SA3, SA5, RAN2, Cc:	RAN, SA