3GPP TSG-RAN WG3 Meeting #127														R3-250671

Athens, GR, Feb 17th - Feb 21th, 2025

Agenda item:			11.2

Source:					CMCC

Title:						Discussion on AI-based network slice

Document Type:		Discussion

Introduction

During RAN3#126, the following agreements have been captured for AI/ML enabled slicing use case.

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message.

Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node.

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

In this paper, we provide our consideration of AI/ML based Network Slicing.

Discussion

Xn interface enhancement

In RAN3#126, RAN3 has agreed to enhance the Data Collection Reporting Initiation and Data Collection Reporting procedure for slice use case, and discussed how to support the new introduced input/feedback information.

For the predicted radio resource status per slice, the first bit (Predicted Radio Resource Status) is reused with the list of slices in the Data Collection Request message to request slice level radio resource status. In the online discussion, it is clear that the cell-level predicted radio resource status will always be provided through Data Collection Response message. But the slice-level radio resource status is provided only upon request.

If the slice-level radio resource status is requested, and the requested node temporarily cannot provide the slice-level prediction information, the current procedure lacks a mechanism to indicate this limitation to the requesting node. It’s necessary to indicate which slice’s prediction information cannot be provided in the Data Collection Response message.

Proposal 1: Indicate which slice’s prediction information cannot be provided in Data Collection Response message. Same as the R18 procedure.

Currently, the Data Collection Response message including the ‘Node Measurement Initiation Result List’ and ‘Cell Measurement Initiation Result List’. For how to implement the slice level prediction information, two options can be discussed:

Opt 1: Introduce a separate IE for the failed slice level prediction information, different from the Node Measurement Initiation Result List IE and Cell Measurement Initiation Result List IE.

Opt 2: Introduce a new IE under the Cell Measurement Initiation Result List IE.

As Slice To Report List for Data Collection IE introduced in Data Collection Request message is under the Cell To Report List for Data Collection IE, it’s better to follow the same principle in Data Collection Response message that introduce a new IE under the Cell Measurement Initiation Result List IE. Thus, we slightly prefer Opt 2.

Proposal 2: Introduce a new IE under the Cell Measurement Initiation Result List IE in Data Collection Response message to indicate which slice’s prediction information cannot be provided.

Finer granularity of UE performance feedback

In RAN3#126, RAN3 has discussed the granularity of UE performance feedback, and the following agreement is achieved:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

However, RAN3 also has no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

In Rel-18, the UE performance including “Average UE Throughput DL/UL”, “Average Packet Delay” and “Average Packet Loss DL”. First, for Average UE Throughput, the Summary of offline discussion (R3-247785) captured how the UE throughput per UE/slice is calculate as:

  - Or if it is an average over time among all packets via all DRBs within the same slice/UE, e.g., R3-247412

- AvgTputDLUE−SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps

  - Or if it is an average of DRB level metric within the same slice/UE, e.g., R3-247412

- AvgTputDLUE−RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps

From our understanding, this example may be inappropriate. When calculating the average UE throughput, the DRBs for one UE does not measured in the same time window. But from our understanding, if we want to calculate the average UE throughput, all the DRBs should be measured in the same time window. If the time window is the same across all the DRBs, the above two ways to calculate the average throughput is the same, i.e. AvgTputDLUE−SA5= AvgTputDLUE−RAN3.

Observation 1: When calculating average UE throughput per UE/slice, all the DRBs should be measured in the same time window.

Proposal 3: RAN3 agrees that when calculating average UE throughput per UE/slice, the time window should be the same across all the DRBs.

However, we also have some concerns on the formula from 28.558. From our perspective, the average UE throughput should be derived as the average value of per DRB level measurements of the UE, which is align with the agreement in RAN3#124 meeting “UE level performance is derived as the average value of per DRB level measurements of the UE”. But this understanding is not align with the formula in SA5. So, it’s proposed that RAN3 discusses whether to inform SA5 RAN3’s understanding for the average UE throughput.

Proposal 4: RAN3 discusses whether to inform SA5 RAN3’s understanding for the calculating of average UE throughput, as RAN3 has some different understanding about how to calculate the average UE throughput.

Then, as the UE throughput highly depends on the traffic, and the traffic changes rapidly, another concern for UE throughput is that the traffic may change after handover, and the UE throughput collected after handover, which is used to be compared with the UE throughput before handover, may not valuable. From our perspective, the traffic is unlikely to change significantly in a short period of time, and what we need to collect is the average UE throughput in a short period of time after handover. Therefore, we believe that UE throughput is reasonable to be measured and feedback to the source node.

Observation 3: The UE throughput highly depends on the traffic. Although the traffic changes rapidly, we believe that it may not change significantly in a short period of time. Thus, the average UE throughput measured in a short period of time after handover is reasonable to be compared with the average UE throughput before handover as the traffic may not change significantly.

For Average Packet Delay and Average Packet Loss DL, the 28.558 also support the per S-NSSAI level as:

- 6.3.1.1.1 Average delay DL air-interface a) This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]). - 6.3.1.2.1 DL Packet Loss Rate on Uu a) This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

Observation 4: The per S-NSSAI level Average Packet Delay and Average Packet Loss DL already supported in 28.558. So, it’s easy for RAN3 to support the collection of UE performance per S-NSSAI.

Proposal 4: RAN3 agrees that the slice level UE performance feedback (i.e. Average UE Throughput DL/UL, Average Packet Delay and Average Packet Loss DL) after handover needs to be measured and feedback to the source node.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: Indicate which slice’s prediction information cannot be provided in Data Collection Response message. Same as the R18 procedure.

Proposal 2: Introduce a new IE under the Cell Measurement Initiation Result List IE in Data Collection Response message to indicate which slice’s prediction information cannot be provided.

Observation 1: When calculating average UE throughput per UE/slice, all the DRBs should be measured in the same time window.

Proposal 3: RAN3 agrees that when calculating average UE throughput per UE/slice, the time window should be the same across all the DRBs.

Proposal 4: RAN3 discusses whether to inform SA5 RAN3’s understanding for the calculating of average UE throughput, as RAN3 has some different understanding about how to calculate the average UE throughput.

Observation 3: The UE throughput highly depends on the traffic. Although the traffic changes rapidly, we believe that it may not change significantly in a short period of time. Thus, the average UE throughput measured in a short period of time after handover is reasonable to be compared with the average UE throughput before handover as the traffic may not change significantly.

Observation 4: The per S-NSSAI level Average Packet Delay and Average Packet Loss DL already supported in 28.558. So, it’s easy for RAN3 to support the collection of UE performance per S-NSSAI.

Proposal 4: RAN3 agrees that the slice level UE performance feedback (i.e. Average UE Throughput DL/UL, Average Packet Delay and Average Packet Loss DL) after handover needs to be measured and feedback to the source node.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC