3GPP TSG-RAN WG3 Meeting #127														R3-250673

Athens, GR, Feb 17th - Feb 21th, 2025

Agenda item:			11.3

Source:					CMCC

Title:						Discussion on AI/ML-based CCO

Document Type:		Discussion

Introduction

During RAN3#126, the following agreements have been captured for AI/ML enabled Coverage and Capacity Optimization.

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

The direction of the information over F1 can be further discussed.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

FFS on whether to send time information related to coverage modification cause over Xn.

Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

In this paper, we provide our consideration of AI/ML based Coverage and Capacity Optimization.

Discussion

F1 interface enhancement

For the split architecture, RAN3#126 has discussed the information exchanged between F1 interface, but the direction of the information over F1 needs further discuss.

The direction of the information over F1 can be further discussed.

Based on the contributions from previous meeting, the direction of the information over F1 can be considered separately on source node and target node. For the source node, it’s obvious that the predicted CCO issue is transferred from CU to DU, and the future CCO state is transferred from DU to CU.

For the target node, the legacy mechanism is the target CU sends the CCO issue received from the source node to the target DU, but we think it’s inappropriately for AI/ML-based CCO. Considering that the target DU can obtain the CCO state of source DU by itself in legacy mechanism, e.g. by means of detection/measurement, the target DU can modify its CCO state correspondingly to match that of the source node. However, for AI/ML-based CCO, target DU currently cannot obtain the future CCO state of the source DU. If the target CU only sends the source node’s CCO issue to the target DU, the future CCO state inferred by the target DU may not match the future CCO state of source DU. Therefore, it’s proposed that RAN3 discusses how to ensure the target DU accurately infer the desired future CCO state, which match that of source DU.

Observation 1: In Legacy CCO mechanism, the target DU can derive the CCO state, which match the modified CCO state of source DU, as the target DU can obtain the CCO state of source DU by itself, e.g. by means of detection/measurement.

Observation 2: For AI/ML-based CCO, the target DU cannot obtain the future CCO state of the source DU currently, and the future CCO state inferred by the target DU may not match the future CCO state of source DU, so the legacy mechanism is not applicable to the AI/ML-based CCO.

Proposal 1: RAN3 discusses how to ensure the target DU accurately infer/derive the desired future CCO state, which match that of source DU.

In order to enable target DU inferring/deriving its future CCO state appropriately to match the future CCO state of source DU, two options are provided:

Opt 1: Target CU sends its own predicted CCO issue to target DU.

As the target CU has the ability to derive it’s own predicted CCO issue based on the future CCO state and Coverage Modification Cause received from source CU, sending the predicted CCO issue of the target CU to the target DU can help the target DU infers/derives corresponding future CCO state.

Opt 2: Target CU sends the future CCO state of source DU to the target DU.

The target DU can easily infer/derive corresponding CCO state based on the future CCO state of source DU, which also matches that of the source DU.

Observation 3: Two options to enable the target DU inferring/deriving its future CCO state that match the future CCO state of source DU:

Opt 1: Target CU sends its own predicted CCO issue to target DU.

Opt 2: Target CU sends the future CCO state of source DU to the target DU.

From our understanding, Opt 2 is straightforward, but Opt 1 can align the information exchanged over F1 between source node and target node. It’s proposed that RAN3 discusses which option is selected.

Proposal 2: RAN3 discusses which option is selected for AI/ML-based CCO:

Opt 1: Target CU sends its own predicted CCO issue to target DU.

Opt 2: Target CU sends the future CCO state of source DU to the target DU.

Then, for the Time for Future Coverage Modification and Predicted CCO issue, we think 60s, which is the same as Requested Prediction Time IE, is not enough. For requested prediction time, as the requested prediction information changes over time, a long time prediction may be inaccurate, so a short prediction time is needed. However, for CCO, as the CCO state is a relatively static information, deriving/inferring CCO state/issue over a very short period of time in the future may not very useful. So, it’s proposed to have a more longer time for future CCO state/predicted CCO issue, e.g. 3600s.

Proposal 3: Time interval for predicted CCO issue and future CCO state proposes to have a longer time, e.g. 3600s.

Xn interface enhancement

For the Xn enhancement, RAN3#126 had discussed the exchanged information over Xn, and whether to send time information of coverage modification cause is FFS.

FFS on whether to send time information related to coverage modification cause over Xn.

From our understanding, whether the time information of the coverage modification cause needs to be transferred over Xn interface depends on whether this time information is needed in the target node. Since the target node has the time information of the future CCO state, it can be used by the target node to determine when to make the corresponding adjustments. Therefore, the time information of the predicted CCO issue seems to be less essential.

Proposal 4: No need to send time information related to coverage modification cause over Xn.

Then, for the Future Cell Coverage State IE, the following FFS needs further discuss:

Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

The value 0 of Cell Coverage State IE in the legacy CCO mechanism indicates that the cell is inactive. As the Future Cell Coverage State IE take legacy Cell Coverage State IE as baseline, the value 0 can be reused with the same meaning, i.e. value 0 in Future Cell Coverage State IE indicates that the cell will be inactive.

Proposal 5: Value 0 of Future Cell Coverage State IE is needed to indicate that the cell will be inactive.

Feedback information

In the offline discussion of RAN3#125bis, RAN3 has discussed whether UE performance or measurements are needed to be collected as feedback. And the following FFS has been captured.

Information to validate the prediction is needed.

For the prediction information, except the predicted CCO issue, we think the future CCO state also needs to be considered as it can be predicted via AI/ML.

Observation 4: As the future CCO state can be generated via AI/ML, it also needs to be validate after the coverage modification.

When validate the future CCO state information, the specific UE(s)’ performance needs to be collected to see whether the UE experience severe performance degradation after coverage modification. This can be used to check the reasonability of the future CCO state. For example, as the UE(s) may locate in the gNB1 before coverage configuration changes, and locate in gNB2 after coverage configuration changes, if the UE experience severe performance degradation after coverage modification, it seems that the generated future CCO state is not very reasonable. If the future CCO state is inferred based on the AI/ML in the source node, the AI/ML model needs to be updated. Also, if the target node, i.e. the serving gNB, aware the performance degradation, it can also perform actions to reduce the impact, e.g. provide more resources for the UE.

In R18, the legacy UE performance feedback including average UE throughput UL/DL, average packet delay and average packet loss. For the coverage edge UE(s), i.e. the UE(s) serving gNB is changed after CCO, the UE performance collected before and after CCO can reuse the legacy UE performance feedback information.

Proposal 6: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. And for the feedback information, except the UE performance, we also propose to collect the NG-RAN node performance. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node these information and the local node can update the AI/ML model with these information.

Also, we think the NG-RAN node performance can be used to validate whether the predicted CCO issue is resolved after the CCO, and whether new issue is predicted after CCO.

Proposal 7: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: In Legacy CCO mechanism, the target DU can derive the CCO state, which match the modified CCO state of source DU, as the target DU can obtain the CCO state of source DU by itself, e.g. by means of detection/measurement.

Observation 2: For AI/ML-based CCO, the target DU cannot obtain the future CCO state of the source DU currently, and the future CCO state inferred by the target DU may not match the future CCO state of source DU, so the legacy mechanism is not applicable to the AI/ML-based CCO.

Proposal 1: RAN3 discusses how to ensure the target DU accurately infer/derive the desired future CCO state, which match that of source DU.

Observation 3: Two options to enable the target DU inferring/deriving its future CCO state that match the future CCO state of source DU:

Opt 1: Target CU sends its own predicted CCO issue to target DU.

Opt 2: Target CU sends the future CCO state of source DU to the target DU.

Proposal 2: RAN3 discusses which option is selected for AI/ML-based CCO:

Opt 1: Target CU sends its own predicted CCO issue to target DU.

Opt 2: Target CU sends the future CCO state of source DU to the target DU.

Proposal 3: Time interval for predicted CCO issue and future CCO state proposes to have a longer time, e.g. 3600s.

Proposal 4: No need to send time information related to coverage modification cause over Xn.

Proposal 5: Value 0 of Future Cell Coverage State IE is needed to indicate that the cell will be inactive.

Observation 4: As the future CCO state can be generated via AI/ML, it also needs to be validate after the coverage modification.

Proposal 6: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Proposal 7: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.473

    - 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..3600, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

    - 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NG CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values Indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicates the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..3600, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |